| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Persistent scheduling for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Yongho Kim, Jin Sam Kwak LG Electronics | E-mail: {chegal, sixs, ksryu, ronnykim}@lge.com |
| Re: | Category: AWD comments / Area: Chapter 15.2.6 (Persistent scheduling) “ Comments on AWD 15.2.6 Persistent scheduling” |  |
| Abstract | The contribution proposes the text of Persistent scheduling section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Persistent scheduling for the IEEE 802.16m Amendment

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Yongho Ronny Kim, Jin Sam Kwak

LG Electronics.

- Introduction

This contribution proposes the text of persistent scheduling section to be included in IEEE 802.16m AWD.

- Text proposal for the 802.16m amendment

-----------------------------------------------------------Start of the Text---------------------------------------------------------

## 15.2.6 Persistent scheduling in advanced air interface

The persistent allocation is one of scheduling methods to reduce the resource assignment overhead for connections with periodic traffic pattern and with relatively fixed payload size. The logical resource assigned by the persistent scheduling repeats at a periodic interval.

## 15.2.6.1 Allocation mechanism

To allocate resources persistently, the ABS shall transmit the DL PA A-MAP IE (Section 15.3.6.4.2.5) for DL allocations and the UL PA A-MAP IE (Section 15.3.6.4.2.6) for UL allocations. The persistently allocated resource and the MCS shall be maintained by the ABS and AMS until the persistent assignment is de-allocated, changed, or an error event occurs. AMS shall use the Flow ID (FID) included in the DL PA A-MAP IE or the UL PA A-MAP IE to distinguish a specific persistent resource among multiple persistent resources.

## 15.2.6.2 De-allocation mechanism

To de-allocate the persistently allocated DL or UL resource, the ABS shall transmit the DL PA A-MAP IE or UL PA A-MAP IE where the allocation flag field is set to 0. If an AMS receives the DL PA A-MAP IE or UL PA A-MAP IE where the allocation flag is set to 0, the AMS shall not use the persistently allocated resource which is indicated by FID included in DL PA A-MAP IE or UL PA A-MAP IE.

## 15.2.6.3 HARQ retransmission

## 15.2.6.4 Error handling procedure

The MS shall transmit at the MAP ACK signal via assigned MAP ACK channel to acknowledge the correct reception of DL PA A-MAP IE or UL PA A-MAP IE, which includes the RSTID of the MS.

-----------------------------------------------------------End of the Text---------------------------------------------------------