| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Group resource allocation A-MAP IE section for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Yongho Kim, Jin Sam Kwak LG Electronics | E-mail: {chegal, sixs, ksryu, ronnykim}@lge.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL_CTRL) “ Comments on AWD 15.3.6 DL_CTRL” |  |
| Abstract | The contribution proposes the text of Group Resource allocation A-MAP IE section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Group Resource allocation A-MAP IE section for the IEEE 802.16m Amendment

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Yongho Ronny Kim, Jin Sam Kwak

LG Electronics.

- Introduction

This contribution proposes the text of Group Resource allocation A-MAP IE section to be included in IEEE 802.16m AWD.

- Text proposal for the 802.16m amendment

-----------------------------------------------------------Start of the Text---------------------------------------------------------

# Group Resource Allocation A-MAP IE

Group control information is used to allocate resources and/or configure resources to one or multiple mobile stations within a user group.

Group scheduling requires two operations

- Assignment of a user to a group. In order to add a user to a group in the DL or UL, the ABS shall transmit a [Group Configuration MAC management message] [Group Configuration A-MAP IE]

- Allocation of resources to users within a group. In order to assign resources to one or more users in a group, the ABS shall transmit the DL/UL Group Resource Allocation A-MAP IE. The DL/UL Group Resource Allocation A-MAP IE is included in user-specific resource assignment in an A-MAP region. The GRA A-MAP IE contains bitmaps to indicate scheduled users and signal resource assignment, MCS, resource size.

Table 663 DL/UL Group Resource Allocation A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | [4] | TBD A-MAP IE types distinguish between UL/DL, SU/MU, OL/CL MIMO operation, persistent/non-persistent allocation, basic/extended IEs including GRA |
| Resource Offset | [6][8] | Indicates starting LRU for resource assignment to this group |
| ACID | TBD | TBD Required for explicit assignment of a single ACID group. Not required if implicit cycling of ACIDs starting from an initial ACID value is used as in 802.16e |
| HARQ ReTx Indicator | TBD | TBD Indicates whether this group resource assignment IE is for HARQ retransmissions or initial transmission |
| User Bitmap Size | [2][5] | TBD Size of the user bitmap; may not be needed if user bitmap size is included in configuration message/A-MAP IE |
| User Bitmap | Variable | Bitmap to indicate scheduled users in a group. The size of the bitmap is equal to the User Bitmap Size |
| Resource Allocation Bitmap descriptor | TBD | TBD MCS group type, packet/resource size type information or attributes required to decode resource assignment bitmap |
| Resource Assignment Bitmap | Variable | Bitmap to indicate MCS/resource size for each scheduled user |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit masked CRC |

# Group configuration A-MAP IE: The group configuration A-MAP IE is used for initiating and maintaining a group for resource assignment.

-----------------------------------------------------------End of the Text---------------------------------------------------------