| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Persistent allocation A-MAP for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Yongho Kim, Jin Sam Kwak LG Electronics | E-mail: {chegal, sixs, ksryu, ronnykim}@lge.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL_CTRL) “ Comments on AWD 15.3.6 DL-CTRL” |  |
| Abstract | The contribution proposes the text of DL_CTRL section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Persistent allocation A-MAP IE for the IEEE 802.16m Amendment

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Yongho Ronny Kim, Jin Sam Kwak

LG Electronics.

- Introduction

This contribution is to optimize the DL PA A-MAP IE, which is included in IEEE 802.16m AWD, as follows.

- What is the benefit of the region for the persistent allocation in 16m? Remove the fields related to the persistent allocation region.

- DL PA A-MAP IE will be used only for persistent DL traffic. Remove the persistent flag and the related fields in DL PA A-MAP IE.

- MAP NACK channel for PA will increase the UL resource wastage. And what is the MAP NACK channel used for in 16m PA? If the persistent allocation does not employ the PA region and resource shifting mechanism, the DL PA A-MAP IE doesn’t need to contain the MAP NACK channel. Remove the MAP NACK channel in DL PA A-MAP IE.

- If a persistent resource immediately follows the previous persistent resource, AMS can know the start of the persistent resource without resource offset field. The DL PA A-MAP IE will include the resource offset indicator to indicate whether the resource offset field is included in this allocation or not.

- ABS will include Flow ID (FID) in DL (or UL) PA A-AMP IE to assign multiple persistent resources to one AMS.

- Text proposal for the 802.16m amendment

-----------------------------------------------------------Start of the Text---------------------------------------------------------

## 15.3.6.4.2.5 DL PA A-MAP IE

The DL persistent allocation A-MAP IE is shown in Table 664.

Table 664—DL persistent allocation A-MAP IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| DL Persistent allocation A-MAP IE () { | -- | -- |
| AMAP type | 4 | DL Persistent allocation A-MAP IE |
| Number of allocations | 5 | Number of allocation specified |
| RSTCID Type | 2 | Type of Reduced STID (RSTID) |
| ACK Region Index | 1 | The index of the ACK region associated with all sub-bursts defined in this DL Persistent allocation A-MAP IE |
| While (data remaining) { | -- | -- |
| Region ID use indicator | 1 | 0: Region ID not used 1: Region ID used |
| if (Region ID use indicator ==0) { |  |  |
| Region information | TBD | TBD Information that specifies the region relevant to this persistent allocation scheduling instance |
| } else { | -- | -- |
| Region ID | 8 | Index to the DL region defined in DL region definition S-SFH |
| } | -- | -- |
| For (j=0;j<Number of allocations; j++) { |  | For loop where each loop element specifies information for one allocation. |
| Allocation flag | 1 | 0: de-allocation 1: allocation |
| RSTID_IE() | variable | Specifies the station ID in RSTID format |
| FID | 4 | Indicates the service Flow ID which is mapped to this allocation. |
| MAP ACK Channel Index | TBD | TBD Depends on MAP ACK channel definition in UL control Index to a MAP ACK channel within the Fast Feedback region. The value 111111 is reserved. When MAP ACK Channel Index = 111111, it indicates NO MAP ACK channel is assigned to this allocation. |
| If (allocation flag = 1) { |  |  |
| Duration Indicator | [1] | If Duration Indicator is 1, it indicates that Duration is explicitly assigned for this subburst. Otherwise, this subburst will use the same Duration as the previous subburst. If j is 1 then this indicator shall be 1. |
| Resource Offset indicator | 1 | If Resource Offset Indicator is 1, it indicates that Resource offset is explicitly assigned for this persistent allocation. Otherwise, this allocation follows the previous allocation immediately. |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| If (Duration Indicator = 1) { | - | - |
| Duration | TBD | Duration in number of LRUs. |
| } |  |  |
| If (Resource Offset indicator =1) { |  |  |
| Resource Offset | TBD | Indicates the start of this persistent allocation in the allocation region. |
| } | - | - |
| If (Persistent Flag = 1) { |  |  |
| Allocation Period and N_ACID Indicator | 1 | If Allocation Period and N_ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (N_ACID) is explicitly assigned for this allocation. Otherwise, this allocation will use the same allocation period as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation Period and N_ACID Indicator = 1) { | - | - |
| Allocation Period (AP) | 5 | Period of the persistent allocation is this field value plus 1 (unit is sub-frame/frame TBD) |
| Number of ACID (N_ACID) | 3 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } |  |  |
| MAP NACK Channel Index | 6 | TBD Index to a shared MAP NACK channel within the Fast Feedback region. The value 111111 is reserved. When MAP NACK Channel Index = 111111, it indicates NO MAP NACK channel is assigned to this allocation. |
| MAP ACK Channel Index | TBD | TBD Index to a MAP ACK channel within the Fast Feedback region |
| Allocation MCS indicator | 1 | If Allocation MCS Indicator is 1, it indicates that MCS is explicitly assigned for this allocation. Otherwise, this allocation will use the same MCS as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Allocation MCS indicator =1) { |  |  |
| MCS | [6] | TBD Depends on supported modulation and coding schemes Specifies the modulation, coding and repetition coding used for this allocation |
| } |  |  |
| Allocation Boosting indicator | [1] | If Allocation Boosting Indicator is 1, it indicates that Boosting is explicitly assigned for this allocation. Otherwise, this allocation will use the same Boosting as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation Boosting indicator =1) { |  |  |
| Boosting | [3] | Specifies the boosting used for this allocation |
| } |  |  |
| Allocation ACID indicator | 1 | If Allocation ACID Indicator is 1, it indicates that ACID is explicitly assigned for this allocation. Otherwise, this allocation will use the same ACID as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation ACID indicator =1) { |  |  |
| ACID | [5] | Specifies the HARQ channel identifier used for this allocation |
| } |  |  |
| AI_SN | 1 | HARQ identifier sequence number |
| SPID/CoRE Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| } //if (allocation flag) |  |  |
| } // for; |  |  |
| } |  |  |
| Allocation Boosting indicator | [1] | If Allocation Boosting Indicator is 1, it indicates that Boosting is explicitly assigned for this allocation. Otherwise, this allocation will use the same Boosting as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation Boosting indicator ==1) { |  |  |
| Boosting |  |  |
| } |  |  |
| Allocation ACID indicator |  |  |
| If (Allocation ACID indicator ==1) { |  |  |
| ACID | [4][5] | HARQ channel identifier |
| } |  |  |
| AI_SN | 1 | HARQ identifier sequence number |
| SPID/CoRE Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| }//while |  |  |
| Padding | variable | Padding to bytes boundary; padding value shall be set to zero. |
| CRC | [16] | 16 bits CRC |
| } |  |  |

Persistent Flag

The persistent flag shall be set to 1 if the assignment is persistent and shall be set to 0 if the assignment is non-persistent.

DRU Offset

The DRU offset shall be set to the first DRU in the time-frequency resource assignment with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the Allocation region.

Duration Indicator

Duration Indicator flag determines whether or not Duration is specified for an allocation. If this flag is 1, it indicates that Duration is explicitly assigned for an allocation. Otherwise, the allocation has the same Duration as the previous allocation. This flag shall be 1 for the first allocation in a Allocation region.

Duration

Duration specifies the size (# DRUs) of an allocation/reallocation in an allocation region.

Resource Offset Indicator

Resource Offset flag determines whether or not Resource Offset is specified for an allocation. If this flag is 1, it indicates that Resource Offset is explicitly assigned for an allocation. Otherwise, the allocation follows the previous allocation.

Resource Offset

The Resource offset indicates the start of this persistent allocation in the allocation region.

Allocation Period and N_ACID Indicator

If Allocation Period and Index Indicator is 1, it indicates that allocation period, ACK and NACK channel index (allocation period and Number of ACID (N_ACID) is explicitly assigned for an allocation. Otherwise, the allocation will use the same allocation period and N_ACID as the previous allocation. This flag shall be 1 for the first allocation in a Allocation region.

Allocation Period

The allocation period (ap) shall be set to one less than the period of the persistent allocation, in units of sub-frames/frames.  For example, as illustrated below, if ap=0b00011, then the period of the persistent allocation is four frames, and the time-frequency resource assignment is valid in frames N, N+4, N+8, etc.

Figure 1: Illustration of Periodic Allocation

N_ACID

The values of ACID field (N0) and N_ACID field (N) are used together to specify an implicit cycling of HARQ channel identifiers as follows.  N0 is used as the HARQ channel identifier corresponding to the first occurrence of the persistent allocation. For each next allocation this value is incremented modulo (N + 1)

As illustrated below, if N_ACID = 0b011 (meaning the period is equal to 4), and if ACID = 2, the HARQ channel identifier follows the pattern 2, 3, 4, 5, 2, 3, 4, 5, etc.

Figure 2: Illustration of Implicitly ACID Cycling

ACID

The ACID field shall be set to the initial value of HARQ channel identifier as described above.

AI_SN

The AI_SN field value shall be set to the initial ARQ identifier sequence number for each HARQ channel.  The AI_SN toggles between 0 and 1 for each    particular HARQ channel.  For example, if the period equals 4 frames, N_ACID = 4, ACID = 2, and AI_SN = 0, the ACID follows the pattern 2, 3, 4, 5, 2, 3, 4, 5, etc, and the AI_SN follows the pattern 0, 0, 0, 0, 1, 1, 1, 1, etc.

ACK_channel

The ACK_channel field shall be set to the number of the ACK channel within the HARQ ACK Region.  The mobile station shall use the indicated ACK channel for transmitting acknowledgment information for each packet received using the time-frequency resource referred to by this persistent allocation.

MAP NACK Channel Index

The MAP NACK channel index is persistently allocated within the Fast Feedback region.  The mobile station shall used the indicated MAP NACK channel to report MAP decoding error in frames where it as a persistent resource allocation assigned with this instance of the persistent IE. The value 111111 is reserved.  When MAP NACK Channel Index = 111111, it indicates NO MAP NACK channel is assigned to this allocation.

MAP ACK Channel Index

The MAP ACK channel is allocated non-persistently within the Fast Feedback region. The mobile station shall use the indicated MAP ACK channel to report successful receipt of the persistent allocation IE. The value 111111 is reserved. When MAP ACK Channel Index = 111111, it indicates NO MAP ACK channel is assigned to this allocation

## 15.3.6.4.2.6 UL PA A-MAP IE

The UL persistent allocation A-MAP IE is shown in Table 664.

Table 665—UL persistent allocation A-MAP IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| UL Persistent allocation A-MAP IE () { | -- | -- |
| AMAP type | 4 | UL Persistent allocation A-MAP IE |
| Number of allocations | 5 | Number of allocation specified |
| RSTCID Type | [2] | Type of Reduced STID (RSTID) |
| ACK Region Index | 1 | The index of the ACK region associated with all sub-bursts defined in this UL Persistent allocation A-MAP IE |
| While (data remaining) { | -- | -- |
| Region ID use indicator | 1 | 0: Region ID not used 1: Region ID used |
| if (Region ID use indicator ==0) { |  |  |
| Region information | TBD | TBD Information that specifies the region relevant to this persistent allocation scheduling instance |
| } else { | -- | -- |
| Region ID | 8 | Index to the UL region defined in UL region definition S-SFH |
| } | -- | -- |
| For (j=0;j<Number of allocations; j++) { |  | For loop where each loop element specifies information for one allocation. |
| Allocation flag | 1 | 0: de-allocation 1: allocation |
| RSTID_IE() | variable | Specifies the station ID in RSTID format |
| FID | 4 | Indicates the service Flow ID which is mapped to this allocation. |
| If (allocation flag = 1) { |  |  |
| MAP ACK Channel Index | TBD | TBD Depends on MAP ACK channel definition in UL control Index to a MAP ACK channel within the Fast Feedback region. The value 111111 is reserved. When MAP ACK Channel Index = 111111, it indicates NO MAP ACK channel is assigned to this allocation. |
| Duration Indicator | [1] | If Duration Indicator is 1, it indicates that Duration is explicitly assigned for this subburst. Otherwise, this subburst will use the same Duration as the previous subburst. If j is 1 then this indicator shall be 1. |
| Resource Offset indicator | 1 | If Resource Offset Indicator is 1, it indicates that Resource offset is explicitly assigned for this persistent allocation. Otherwise, this allocation will follows the previous allocation immediately. |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| If (Duration Indicator = 1) { | - | - |
| Duration | TBD | Duration in number of LRUs. |
| } |  |  |
| If (Resource Offset indicator =1) { |  |  |
| Resource Offset | TBD | Indicates the start of this persistent allocation in the allocation region. |
| } | - | - |
| If (Persistent Flag = 1) { |  |  |
| Allocation Period and N_ACID Indicator | 1 | If Allocation Period and N_ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (N_ACID) is explicitly assigned for this allocation. Otherwise, this allocation will use the same allocation period as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation Period and N_ACID Indicator = 1) { | - | - |
| Allocation Period (AP) | 5 | Period of the persistent allocation is this field value plus 1 (unit is sub-frame/frame TBD) |
| Number of ACID (N_ACID) | 3 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } |  |  |
| MAP NACK Channel Index | 6 | TBD Index to a shared MAP NACK channel within the Fast Feedback region. The value 111111 is reserved. When MAP NACK Channel Index = 111111, it indicates NO MAP NACK channel is assigned to this allocation. |
| MAP ACK Channel Index | 6 | TBD Index to a MAP ACK channel within the Fast Feedback region |
| Allocation MCS indicator | 1 | If Allocation MCS Indicator is 1, it indicates that MCS is explicitly assigned for this allocation. Otherwise, this allocation will use the same MCS as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Allocation MCS indicator =1) { |  |  |
| MCS | [6] | TBD Depends on supported modulation and coding schemes Specifies the modulation, coding and repetition coding used for this allocation |
| } |  |  |
| Allocation Boosting indicator | [1] | If Allocation Boosting Indicator is 1, it indicates that Boosting is explicitly assigned for this allocation. Otherwise, this allocation will use the same Boosting as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation Boosting indicator =1) { |  |  |
| Boosting | [3] | Specifies the boosting used for this allocation |
| } |  |  |
| Allocation ACID indicator | 1 | If Allocation ACID Indicator is 1, it indicates that ACID is explicitly assigned for this allocation. Otherwise, this allocation will use the same ACID as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation ACID indicator =1) { |  |  |
| ACID | [5] | Specifies the HARQ channel identifier used for this allocation |
| } |  |  |
| AI_SN | 1 | HARQ identifier sequence number |
| SPID/CoRE Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| } //if (allocation flag) |  |  |
| } // for; |  |  |
| } |  |  |
| Allocation Boosting indicator | [1] | If Allocation Boosting Indicator is 1, it indicates that Boosting is explicitly assigned for this allocation. Otherwise, this allocation will use the same Boosting as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation Boosting indicator ==1) { |  |  |
| Boosting |  |  |
| } |  |  |
| Allocation ACID indicator |  |  |
| If (Allocation ACID indicator ==1) { |  |  |
| ACID | [4][5] | HARQ channel identifier |
| } |  |  |
| AI_SN | 1 | HARQ identifier sequence number |
| SPID/CoRE Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| }//while |  |  |
| Padding | variable | Padding to bytes boundary; padding value shall be set to zero. |
| CRC | [16] | 16 bits CRC |
| } |  |  |

-----------------------------------------------------------End of the Text---------------------------------------------------------