| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on differential feedback scheme for closed-loop beamforming |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Qinghua Li, Yuan Zhu, Eddie Lin, Shanshan Zheng, Jiacheng Wang, Xiaofeng Liu, Feng Zhou, Guangjie Li, Alexei Davydov, Huaning Niu, Yang-seok Choi Intel Corporation | alexei.davydov@intel.com guangjie.li@intel.com qinghua.li@intel.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) “Comments on AWD 15.3.7 DL-MIMO” |  |
| Abstract | The contribution proposes text on differential precoding mode for the 802.16m |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Text proposal on differential feedback scheme for closed-loop beamforming

Qinghua Li, Yuan Zhu, Eddie Lin, Shanshan Zheng, Jiacheng Wang, Xiaofeng Liu, Feng Zhou, Guangjie Li, Alexei Davydov, Huaning Niu, Yang-seok Choi

Intel Corporation

Add proposed text to line 63, page 91, section 15.3.7.2.6.6.4.

-------------------------------------------------- Start Text ------------------------------------------------

The differential feedbacks exploit the correlation between precoding matrixes adjacent in time or frequencies. The feedback shall start initially and restart periodically by sending a one-shot feedback that fully depicts the precoder by itself. The codebook for the one-shot feedback is defined for the base mode.

Denote the feedback index, the correspondingly fed back matrix, and the corresponding precoder by , , and , respectively. The sequential index is reset to  at . The index for the initial or the restart feedback is  and . The indexes of the subsequent differential feedback are  and the corresponding precoders are , where  is a unitary  matrix computed from the previous precoder ;  is the number of transmit antennas. The dimension of the fed back matrix  is  for , where  is the number of spatial streams.

The rotation matrix  of  has the form , where  consists of columns each of which has a unit norm and is orthogonal to the other columns of . Define the Householder matrix  of unit vector  as

where  and ;  is the phase of the first entry of ; . For ,  is an  vector and .	For  and ,  is . Denote  as . Two columns are appended to  as , where  and  are vectors with all zeros except that the -th and -th entries are ones, respectively. The index  and  are selected. Let the -th and -th entries of  be the smallest and the second smallest, respectively, where  converts ’s entries to their absolute values; and  are the real and imaginary parts of , respectively. Gram-Schmidt orthogonalization is applied on  as , where  is the conjugate of ’s entry of on the -th row and -th column. Normalization follows the orthogonalization as

. The matrix  is extended by one column as . The Gram-Schmidt process on  is . The followed normalization is . Finally, . The Gram-Schmidt orthogonalization is the same as the one applied in the transformed codebook. An illustration of the computation of  is shown in Figure xxx. Let  be a vector or a matrix with two columns. Denote  the rotation matrix of .

The feedback matrix  is selected from a differential codebook. Denote the codebook by , where  is the number of codewords in the codebook. The codebooks,,, and are listed in Table xxx, xxx, xxx, xxx. Denote  the i-th codeword of . The rotation matrixes s of the s comprises a set of  by  matrixes that is denoted by .

The differential codebook  is computed from . The i-th codeword of  denoted by  is computed as

,

where  consists of the last three columns of the i-th matrix in . The differential codebook  is computed from . The i-th codeword of  is the i-th matrix in . Two sets of differential codebooks are defined. One has a large step size for fast tracking capability and the other has a small step size for high tracking accuracy. For , the codebook with large step size shall be used. A 1-bit indicator may be fed back for the step size used for .

Table 1.  codebook.

|  | Index | Codeword | Index | Codeword |
| --- | --- | --- | --- | --- |
| Codebook of large step size | 1 | [1 0]T | 3 |  |
|  | 2 |  | 4 |  |

Table 2.  codebook.

|  | Index | Codeword | Index | Codeword |
| --- | --- | --- | --- | --- |
| Codebook of large step size | 1 |  | 3 |  |
|  | 2 |  | 4 |  |

Table 3.  codebook.

|  | Index | Codeword | Index | Codeword |
| --- | --- | --- | --- | --- |
| Codebook of large step size | 1 | [1 0 0 0]T | 9 | [cos(20o) 0.2553 + 0.1430i 0.0282 + 0.0897i 0.1469 + 0.0308i]T |
|  | 2 | [cos(20o) 0.2062 - 0.0657i 0.0485 - 0.2038i -0.0885 + 0.1358i]T | 10 | [cos(20o) 0.0507 - 0.3289i 0.0276 + 0.0448i 0.0508 - 0.0297i]T |
|  | 3 | [cos(20o) -0.0531 - 0.0765i 0.0806 - 0.1811i -0.1432 - 0.2203i]T | 11 | [cos(20o) -0.0352 + 0.2445i 0.0560 + 0.1197i -0.1178 - 0.1569i]T |
|  | 4 | [cos(20o) -0.0762 - 0.1024i -0.2492 - 0.1865i 0.0616 + 0.0028i]T | 12 | [cos(20o) -0.0505 - 0.0233i -0.1061 + 0.3140i 0.0505 + 0.0382i]T |
|  | 5 | [cos(20o) -0.0475 - 0.0535i 0.0266 - 0.0109i 0.1997 + 0.2668i]T | 13 | [cos(20o) -0.3407 - 0.0014i 0.0280 + 0.0108i 0.0021 + 0.0020i]T |
|  | 6 | [cos(20o) -0.0478 - 0.0010i -0.0229 + 0.0325i 0.2359 - 0.2397i]T | 14 | [cos(20o) -0.0180 - 0.0100i 0.3300 + 0.0502i 0.0685 - 0.0205i]T |
|  | 7 | [cos(20o) 0.0030 + 0.1854i -0.1733 -0.1136 + 0.1992i]T | 15 | [cos(20o) -0.0401 - 0.0885i 0.0946 + 0.1084i -0.2792 + 0.0942i]T |
|  | 8 | [cos(20o) 0.1926 - 0.0378i -0.1914 + 0.0534i -0.1467 - 0.1320i]T | 16 | [cos(20o) -0.0436 + 0.2160i 0.0596 - 0.2318i 0.1057 + 0.0002i]T |

Table 4.  codebook.

|  | Index | Codeword | Index | Codeword |
| --- | --- | --- | --- | --- |
| Codebook of large step size | 1 | [ 1 0 0 0 0 1 0 0]T | 9 | [ 0.9770 0.1518 + 0.0929i 0.0606 - 0.0773i 0.0063 - 0.0647i -0.0507 - 0.1011i -0.0981 + 0.8703i -0.1618 - 0.0957i -0.3914 + 0.1776i]T |
|  | 2 | [ 0.9571 -0.0238 + 0.0314i -0.0454 - 0.2541i -0.0790 + 0.0977i -0.0965 + 0.0299i 0.9114 + 0.0872i -0.0431 - 0.3386i -0.1023 + 0.1567i]T | 10 | [ -0.6295 -0.5472 - 0.3123i -0.0136 - 0.1891i 0.2222 - 0.3486i 0.5496 - 0.3201i -0.7539 - 0.0022i 0.0440 + 0.0657i -0.0189 + 0.1434i]T |
|  | 3 | [ -0.0262 0.7460 - 0.6224i 0.2085 + 0.1061i 0.0104 - 0.0226i 0.6933 + 0.5709i 0.1217 + 0.0055i -0.1479 - 0.3702i -0.1061 - 0.0917i]T | 11 | [ 0.3622 -0.8103 - 0.3554i 0.0797 - 0.2550i -0.1050 + 0.0596i -0.8270 + 0.3289i -0.2410 - 0.0429i -0.1349 - 0.3222i -0.0937 + 0.1311i]T |
|  | 4 | [ 0.9990 0.0386 - 0.0212i 0.0035 - 0.0023i 0.0002 + 0.0019i -0.0343 - 0.0200i 0.8730 + 0.0488i 0.3473 - 0.1714i 0.2857 - 0.0483i]T | 12 | [ -0.4402 -0.8115 - 0.0841i 0.0434 + 0.1299i -0.1636 + 0.3083i -0.7666 + 0.1113i 0.5535 + 0.0180i 0.0170 + 0.1186i -0.1660 + 0.2268i]T |
|  | 5 | [ 0.9556 0.1479 - 0.0806i -0.0215 + 0.1307i -0.0706 - 0.1894i -0.0844 + 0.0610i 0.8284 - 0.3568i 0.1996 - 0.1472i -0.1478 + 0.3037i]T | 13 | [ 1 0 0 0 0 -0.8741 + 0.0445i 0.3194 - 0.1760i 0.3172 – 0.0173i]T |
|  | 6 | [ -0.8726 0.1100 - 0.0735i -0.4250 - 0.1821i -0.0795 + 0.0325i 0.1648 + 0.1221i 0.9722 - 0.0007i -0.0410 - 0.1039i 0.0018 + 0.0180i]T | 14 | [ -0.8851 -0.3025 + 0.3449i 0.0049 + 0.0437i -0.0340 - 0.0557i 0.2630 + 0.2692i -0.7941 - 0.0049i 0.2671 - 0.0632i -0.2947 + 0.2561i]T |
|  | 7 | [ -0.6845 -0.0048 - 0.7234i 0.0310 - 0.0167i 0.0831 + 0.0006i 0.0085 + 0.6243i 0.6200 + 0.0054i -0.3294 - 0.2343i 0.2292 - 0.0994i]T | 15 | [ 0.8990 -0.1582 - 0.1183i 0.1246 - 0.0775i -0.3616 - 0.0214i 0.0035 + 0.2203i -0.8650 + 0.3492i -0.0464 + 0.0693i 0.2338 + 0.1398i]T |
|  | 8 | [ 0.5617 0.8043 + 0.1719i -0.0617 - 0.0099i 0.0607 - 0.0241i 0.7006 - 0.1414i -0.5130 - 0.0152i -0.1561 + 0.2422i 0.3191 + 0.2023i]T | 16 | [ 0.5212 0.3746 + 0.7570i 0.0670 + 0.1016i -0.0085 - 0.0003i 0.3025 - 0.7018i -0.4381 + 0.0708i -0.2495 - 0.2784i 0.2622 - 0.1028i]T |

----------------------------------------------- End of Text -------------------------------------------------