| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Harmonized proposed AWD text of Frame Structure for 5 MHz, 10 MHz and 20 MHz bandwidth with 1/4 Tb CP Length |  |
| Date Submitted | 2009-05-01 |  |
| Source(s) | Jia-Hao Wu, Yu-Tao Hsieh, Pang-An Ting ITRI Yanfeng Guan, Huiying Fang ZTE | Voice : 886-3-5914854 E-mail: jefferywu@itri.org.tw E-mail: guan.yanfeng@zte.com.cn |
| Re: | Re: Category: AWD comments /Area: Chapter 15.3.3 (Frame Structure) “Comments on AWD 15.3.3 Frame Structure” |  |
| Abstract | The contribution proposes the text of frame structure section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Harmonized Proposed AWD text of Frame Structure for 5, 10 and 20 MHz bandwidth with 1/4 Tb CP length

Jia-Hao Wu, Yu-Tao Hsieh, Pang-An Ting

ITR

Yanfeng Guan, Huiying FangI

ZTE

# Introduction

According to the current version of SDD [1] and IEEE 802.16m amendment working document (AWD) [2], the OFDMA numerology with cyclic prefix (CP) length equal to 1/4 Tb has been defined. The OFDMA symbols with this longer CP structure are expected to be used for long delay spread channel. For example, delay spread  of MBSFN channel model for 5 km cell inter-site distance is about 24.5 microseconds [3]. However, the frame structures for CP equal to 1/4 Tb have not been defined yet. In this harmonized contribution based on the IEEE 802.16m-09_0884 [5] and IEEE 802.16m-09_0996 [6], amendment text including several illustrations is proposed to provide the frame structures for the long CP scenario.

# Proposed Frame Structure for 5, 10 and 20MHz with CP = 1/4 Tb

### 2.1 Design Criteria

Following we will discuss some design criteria when considering frame structures for CP = 1/4 Tb. This kind of long CP frame structure is mainly used to mitigate inter-symbol interference (ISI) in long delay spread channel. For example, it may be adopted for dedicated MBSFN carrier which contains only downlink broadcasting subframes. Besides, when transmitting uplink control channel in large cells, long CP frame structures can be used to accommodate diverse round-trip delay(RTD).

To follow the regulations of current SDD, we take following criteria into considerations:

- For 5, 10 and 20 MHz bandwidth, there shall be 8 subframes in a radio frame as cases of CP equal to 1/8 Tb and 1/16 Tb in order to reuse the same timing design for HARQ operation .

- A radio frame should be include 43 symbols for FDD and 42 symbols for TDD.

- A radio frame is composed of type-1, type-2 and type-3 subframes which are defined in both SDD and AWD for 5, 10 and 20 MHz bandwidth, but including less subframe type in a radio frame will be better.

- The first subframe of a radio frame shall not be type-3 which only contains 5 OFDMA symbols since the SCH would be allocated for synchronization purpose.

- For FDD subframes, the first, fifth and last subframe type should be the same type as cases of CP equal to 1/8 Tb and 1/16 Tb. This would ease hardware implementation for FDD frame structure.

2.2 Frame structure for 5, 10 and 20 MHz

In the current SDD, three types of subframes have been defined, i.e., type-1 with 6 symbols, type-2 with 7 symbols, and type-3 with 5 symbols. Moreover, there shall be 8 subframes in a radio frame with 5, 10 and 20 MHz bandwidth. In order not to violate the frame structure regulation in current SDD and to meet the designated numerology in AWD, we propose following frame structures for TDD and FDD transmission modes.

2.2.1 TDD frame structure

There are two possible frame structures for CP=1/4 Tb with TDD transmission modes. One contains two type-1 subframes and six type-3 subframes in a radio frame. The other contains one type-2 subframe and seven type-3 subframes in a radio frame. Figure 1 illustrates the TDD frame structure A for CP=1/4 Tb. The downlink and uplink ratio (DL:UL) shown is 5:3 with TTG = 114.286 us and RTG = 85.7 us at the switching points.

Figure 2 illustrates TDD frame structure B for CP=1/4 Tb. The figure shows D:U = 5:3. Although there are two possible frame structures that can meet the criteria stated before, we are more fond of the frame structure A. It is because the last subframe is type-1 rather than type-3 and may be more feasible when uplink transmission is allocated.

Figure. 1 TDD frame structure A for 5, 10 and 20 MHz TDD with CP = 1/4 Tb

Figure. 2 TDD frame structure B for 5, 10 and 20 MHz TDD with CP = 1/4 Tb

2.2.2 FDD frame structure

In the FDD transmission mode, there are also two possible frame structures when taking aforementioned design criteria into considerations. One contains three type-1 subframes and five type-3 subframes. The other contains one type-1 subframe, one type-2 subframe and six type-3 subframes. Figure 3 and 4 show the two kinds of FDD frame structures. In order to make frame structure as simple as possible, the frame structure C is preferable since it is only composed of two subframe types.

Figure. 3 Frame structure C for 5, 10 and 20 MHz FDD with CP = 1/4 Tb

Figure 4. Frame structure D for 5, 10 and 20 MHz FDD with CP = 1/4 Tb

# References

[1] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document.”

[2] IEEE 802.16m-08/0010r1a, “IEEE 802.16m Amendment Working Document.”

[3] IEEE 802.16m-07/004r5, “IEEE 802.16m Evaluation Methodology Document.”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment.”

[5] IEEE 802.16m-08_0884, “Proposed AWD text of Frame Structure for 5 MHz, 10 MHz and 20 MHz bandwidth with 1/4 Tb CP Length.”

[6] IEEE 802.16m-08_0996, “Frame Structure for 1/4 Tb in IEEE 802.16m Amendment Working Document

# Text proposal for the 802.16m amendment

============================== Start of Proposed Text ================================

[Insert the following text and illustration after Figure 402 on page 26 and modify the involved clause indices:]

15.3.3.4 Frame structure for CP = 1/4 Tb

The frame structure for CP = 1/4 Tb shall consist of type-1 and type-3 subframes.

For channel bandwidths of 5, 10, and 20 MHz, a TDD frame shall have six type-3 subframes and two type-1 subframes, and an FDD subframe shall have five type-3 subframes and three type-1 subframes.

In the TDD frame, the first and last subframes within each frame shall be type-1 subframes.

In the FDD frame, the first, fifth, and last subframes within each frame shall be type-1 subframes.

Figure XXX illustrates an example of TDD and FDD frame structure for 5, 10 and 20 MHz channel bandwidth with CP = 1/4Tb. Assuming OFDMA symbol duration of 114.286 us, the type-1 and type-3 subframe durations are 0.6857 ms and 0.5714 ms, respectively. For TDD frame, the figure shows DL:UL = 5:3 with TTG = 114.286 us and RTG = 85.7 us.

Figure. XXX—TDD and FDD frame structure for 5, 10 and 20 MHz with CP = 1/4Tb

============================== End of Proposed Text =================================