| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Hierarchical structure of the DL 4TX base codebook |  |
| Date Submitted | 2009-05-01 |  |
| Source(s) | David Mazzarese, Bruno Clerckx, Heewon Kang, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) “Comments on AWD 15.3.7 DL-MIMO” |  |
| Abstract | The contribution proposes text for the MIMO section of the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Hierarchical structure of the DL 4TX base codebook

David Mazzarese et Al.

Samsung Electronics

- Introduction

This contribution proposes amendment text for MIMO, to be included in the IEEE 802.16m Amendment Working Document, subclause 15.3.7. Comment 286 in 80216m-09_0023 proposes to delete the text that describes the hierarchical structure of the DL 4Tx base codebook. Comment 295 in 80216m-09_0023 clarifies the hierarchical structure, and proposes to keep this description with a correction to the statement on the properties related to chordal distance.

- Hierarchical structure of the DL 4Tx base codebook

The statement on line 2 page 84 "much larger than" is misleading. The hierarchical structure described in Figure 433 and Table 678 ensures a relationship "equal to or larger than".

Table 1 shows the chordal distance relationship between codewords of index 17 to 64 with other codewords in the base codebook. We show the first 6 closest codewords for each PMI with index 17 to 64. The right-most 6 columns of Table 1 show the chordal distances. This Table confirms that the statement on line 2 page 84 should be corrected to:

“Each codeword cluster has four codewords, of which one codeword is from Layer 1 and the three other codewords are from Layer 2. For any given Layer 2 codeword, its chordal distance to all other Layer 1 codewords of different clusters is always much equal to or larger than that distance to the Layer 1 codeword of its same cluster.”

Table 1 – Chordal relationship of rank-1 codewords of the DL 4Tx base codebook

| Layer 2 PMI | PMIs with closest chordal distance to the Layer 2 PMI |  |  |  |  |  | Chordal distances |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | #1 | #2 | #3 | #4 | #5 | #6 | #1 | #2 | #3 | #4 | #5 | #6 |
| 16 | 30 | 62 | 32 | 14 | 0 | 46 | 0.3633 | 0.3801 | 0.3963 | 0.4230 | 0.4230 | 0.4285 |
| 17 | 49 | 33 | 1 | 29 | 63 | 26 | 0.4830 | 0.4830 | 0.4987 | 0.6063 | 0.6295 | 0.7090 |
| 18 | 34 | 50 | 2 | 61 | 36 | 13 | 0.4830 | 0.4914 | 0.4987 | 0.5386 | 0.5396 | 0.5969 |
| 19 | 35 | 51 | 3 | 62 | 14 | 16 | 0.4887 | 0.4930 | 0.4975 | 0.5316 | 0.5991 | 0.6011 |
| 20 | 29 | 61 | 13 | 4 | 45 | 36 | 0.3633 | 0.3744 | 0.4230 | 0.4230 | 0.4356 | 0.4921 |
| 21 | 31 | 63 | 37 | 15 | 5 | 47 | 0.3633 | 0.3750 | 0.3963 | 0.4230 | 0.4230 | 0.4285 |
| 22 | 38 | 28 | 60 | 12 | 6 | 44 | 0.3583 | 0.3633 | 0.3744 | 0.4230 | 0.4230 | 0.4356 |
| 23 | 60 | 55 | 39 | 7 | 12 | 24 | 0.3792 | 0.4910 | 0.4910 | 0.4989 | 0.5542 | 0.5619 |
| 24 | 40 | 56 | 8 | 50 | 23 | 34 | 0.4830 | 0.4830 | 0.4988 | 0.5600 | 0.5619 | 0.6029 |
| 25 | 41 | 57 | 9 | 51 | 35 | 62 | 0.4829 | 0.4830 | 0.4988 | 0.5621 | 0.6029 | 0.6566 |
| 26 | 58 | 42 | 10 | 62 | 63 | 14 | 0.4906 | 0.4911 | 0.4989 | 0.6573 | 0.6614 | 0.6743 |
| 27 | 59 | 43 | 11 | 60 | 23 | 63 | 0.4826 | 0.4830 | 0.4988 | 0.6612 | 0.6644 | 0.6652 |
| 28 | 22 | 60 | 44 | 12 | 6 | 38 | 0.3633 | 0.4830 | 0.4900 | 0.4988 | 0.5418 | 0.6292 |
| 29 | 20 | 49 | 61 | 45 | 13 | 33 | 0.3633 | 0.4607 | 0.4830 | 0.4900 | 0.4988 | 0.5309 |
| 30 | 55 | 16 | 32 | 46 | 62 | 14 | 0.3603 | 0.3633 | 0.4010 | 0.4829 | 0.4910 | 0.4988 |
| 31 | 21 | 37 | 63 | 47 | 15 | 5 | 0.3633 | 0.4026 | 0.4826 | 0.4829 | 0.4988 | 0.5418 |
| 32 | 16 | 30 | 0 | 59 | 55 | 39 | 0.3963 | 0.4010 | 0.5016 | 0.5362 | 0.5439 | 0.5442 |
| 33 | 17 | 49 | 1 | 4 | 29 | 42 | 0.4830 | 0.4830 | 0.4988 | 0.5064 | 0.5309 | 0.5954 |
| 34 | 18 | 50 | 2 | 61 | 36 | 13 | 0.4830 | 0.4867 | 0.4988 | 0.5119 | 0.5470 | 0.5969 |
| 35 | 19 | 51 | 3 | 62 | 14 | 25 | 0.4887 | 0.4910 | 0.4988 | 0.5060 | 0.5969 | 0.6029 |
| 36 | 61 | 20 | 4 | 2 | 18 | 34 | 0.4834 | 0.4921 | 0.4989 | 0.5066 | 0.5396 | 0.5470 |
| 37 | 21 | 31 | 5 | 56 | 40 | 15 | 0.3963 | 0.4026 | 0.4989 | 0.5345 | 0.5559 | 0.5965 |
| 38 | 22 | 6 | 7 | 12 | 60 | 28 | 0.3583 | 0.4989 | 0.5066 | 0.5375 | 0.5453 | 0.6292 |
| 39 | 55 | 23 | 7 | 0 | 32 | 48 | 0.4830 | 0.4910 | 0.4988 | 0.5064 | 0.5442 | 0.5601 |
| 40 | 56 | 24 | 8 | 37 | 34 | 18 | 0.4830 | 0.4830 | 0.4988 | 0.5559 | 0.6133 | 0.6164 |
| 41 | 25 | 57 | 9 | 35 | 19 | 53 | 0.4829 | 0.4829 | 0.4987 | 0.6134 | 0.6189 | 0.6749 |
| 42 | 58 | 26 | 10 | 33 | 54 | 29 | 0.4874 | 0.4911 | 0.4988 | 0.5954 | 0.6748 | 0.6815 |
| 43 | 27 | 59 | 11 | 32 | 23 | 39 | 0.4830 | 0.4874 | 0.4988 | 0.5542 | 0.6071 | 0.6133 |
| 44 | 22 | 28 | 60 | 12 | 23 | 24 | 0.4356 | 0.4900 | 0.4925 | 0.5016 | 0.6720 | 0.6816 |
| 45 | 20 | 29 | 61 | 13 | 33 | 25 | 0.4356 | 0.4900 | 0.4925 | 0.5016 | 0.6563 | 0.6816 |
| 46 | 16 | 30 | 62 | 14 | 32 | 55 | 0.4285 | 0.4829 | 0.4911 | 0.4988 | 0.6106 | 0.6676 |
| 47 | 21 | 31 | 63 | 15 | 37 | 5 | 0.4285 | 0.4829 | 0.4874 | 0.4988 | 0.6117 | 0.6810 |
| 48 | 13 | 0 | 39 | 7 | 55 | 40 | 0.4230 | 0.4230 | 0.5601 | 0.6402 | 0.6510 | 0.6748 |
| 49 | 29 | 17 | 33 | 1 | 4 | 20 | 0.4607 | 0.4830 | 0.4830 | 0.4987 | 0.5065 | 0.5607 |
| 50 | 34 | 18 | 2 | 24 | 36 | 61 | 0.4867 | 0.4914 | 0.4989 | 0.5600 | 0.6302 | 0.6419 |
| 51 | 35 | 19 | 3 | 25 | 62 | 54 | 0.4910 | 0.4930 | 0.4989 | 0.5621 | 0.6400 | 0.6972 |
| 52 | 12 | 4 | 49 | 1 | 36 | 43 | 0.4230 | 0.4230 | 0.6358 | 0.6402 | 0.6583 | 0.6748 |
| 53 | 14 | 5 | 2 | 41 | 50 | 30 | 0.4230 | 0.4230 | 0.6402 | 0.6749 | 0.6985 | 0.7415 |
| 54 | 15 | 6 | 3 | 42 | 51 | 31 | 0.4230 | 0.4230 | 0.6402 | 0.6748 | 0.6972 | 0.7415 |
| 55 | 30 | 39 | 23 | 7 | 0 | 32 | 0.3603 | 0.4830 | 0.4910 | 0.4988 | 0.5064 | 0.5439 |
| 56 | 40 | 24 | 8 | 37 | 55 | 30 | 0.4830 | 0.4830 | 0.4987 | 0.5345 | 0.5639 | 0.6612 |
| 57 | 41 | 25 | 9 | 31 | 28 | 51 | 0.4829 | 0.4830 | 0.4987 | 0.6612 | 0.6638 | 0.7090 |
| 58 | 42 | 26 | 10 | 28 | 29 | 49 | 0.4874 | 0.4906 | 0.4988 | 0.6584 | 0.6653 | 0.7080 |
| 59 | 27 | 43 | 11 | 32 | 23 | 39 | 0.4826 | 0.4874 | 0.4988 | 0.5362 | 0.5993 | 0.6049 |
| 60 | 22 | 23 | 28 | 44 | 12 | 38 | 0.3744 | 0.3792 | 0.4830 | 0.4925 | 0.4988 | 0.5453 |
| 61 | 20 | 29 | 36 | 45 | 13 | 34 | 0.3744 | 0.4830 | 0.4834 | 0.4925 | 0.4988 | 0.5119 |
| 62 | 16 | 30 | 46 | 14 | 35 | 19 | 0.3801 | 0.4910 | 0.4911 | 0.4989 | 0.5060 | 0.5316 |
| 63 | 21 | 31 | 47 | 15 | 5 | 1 | 0.3750 | 0.4826 | 0.4874 | 0.4988 | 0.5582 | 0.5596 |

- Text proposal for inclusion in the 802.16m amendment

Change "much larger than" to "equal to or larger than" on line 2 page 84.