| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed A-MAP Relevance and HARQ Timing for the IEEE 802.16m Amendment (Proposed Text for AWD) |  |
| Date Submitted | 2009-05-03 |  |
| Source(s) | Mihyun Lee, Jaeweon Cho, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. Jing Zhu Intel Corporation Yih-Shen Chen and I-Kang Fu MediaTek | mihyun.mac.lee@samsung.com jing.z.zhu@intel.com |
| Re: | Category: AWD-DG comments / Area: HARQ Protocol DG “Comments on the Proposed Text of HARQ Protocol DG” |  |
| Abstract | This contribution provides the proposed text of A-MAP relevance and HARQ timing for the IEEE 802.16m Amendment. |  |
| Purpose | To be discussed and adopted by HARQ Protocol DG and TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed A-MAP Relevance and HARQ Timing 
for the IEEE 802.16m Amendment (Proposed Text for AWD)

Mihyun Lee, Jaeweon Cho, Hokyu Choi, Heeweon Kang

Samsung Electronics Co., Ltd.

Jing Zhu

Intel Corporation

Yih-Shen Chen and I-Kang Fu

MediaTek

- Introduction

This contribution provides the proposed text of A-MAP relevance and HARQ timing for the IEEE 802.16m Amendment. The proposed text is aligned and can be readily combined with HARQ Protocol DG Draft document [1]. The design principles, features and benefits of the proposed A-MAP relevance and HARQ timing, and also illustrations of HARQ timings for various configurations are shown in another input contribution [2].

- References

[1] IEEE C802.16m-09/0859, “ Proposed Text of HARQ Protocol for the 802.16m Amendment”

[2] IEEE C802.16m-09/1131, “Proposed A-MAP Relevance and HARQ Timing for the IEEE 802.16m 
Amendment (Design Principles and Key Features)”

- Text proposal

[Insert the following new subcluase into Section 15.2]

-------------------------------  Text Start  ---------------------------------------------------

### 15.2.x.2.2   A-MAP relevance and HARQ timing

Transmissions	 of Assignment A-MAP IE, the HARQ subpacket, and the corresponding feedback shall be in accordance to a pre-defined timing. In UL, retransmission of the HARQ subpacket shall also follow a pre-defined timing.

Each transmission time is represented by frame index and subframe index. The frame index shall range from 0 to 3. In FDD, the index of DL or UL subframe shall range from 0 to F-1, where F is the number of subframes per frame. In TDD, the index of DL subframe shall range from 0 to D-1, where D is the number of DL subframes per frame, and the index of UL subframe shall range from 0 to U-1, where U is the number of UL subframes per frame.

### 15.2.x.2.2.1   FDD

### 15.2.x.2.2.1.1   Downlink

In DL HARQ transmission, DL Basic Assignment A-MAP IE, the HARQ subpacket, and the corresponding feedback shall follow the timing defined in Table 1.

Table 1 – FDD DL HARQ timing

| Content | Subframe index | Frame index |
| --- | --- | --- |
| Basic Assignment A-MAP IE Tx in DL | l | i |
| HARQ Subpacket Tx in DL | m=l or l+NA−MAP−1 | i |
| HARQ feedback in UL | n = ceil(m+F/2) mod F | j=i+floorceil(m+F2)F+zmod4 |

DL HARQ subpacket transmission corresponding to a DL Basic Assignment A-MAP IE in l-th DL subframe of the i-th frame shall begin in the m-th DL subframe of the i-th frame. A HARQ feedback for the DL HARQ subpacket shall be transmitted in the n-th UL subframe of the j-th frame. The subframe index m, n and frame index j shall be determined by using l and i, as shown in Table 1.

Note that the subframe index l shall range from 0 to NA-MAP(ceil(F/NA-MAP) –1) with an increment of NA-MAP. For the case that the A-MAP transmission period is two subframes, i.e. NA-MAP = 2, m shall be selected between l and l+1. The selection information of m shall be provided in DL Basic Assignment A-MAP IE.

- DL HARQ feedback offset z shall be set to 1 only if a time gap from completion of the HARQ subpacket transmission to its feedback time derived with z = 0 is shorter than the data burst processing time Tproc. Otherwise, z shall be set to 0. This rule shall be also applied to the long TTI transmission:

- z=0,      if (ceil(F2)−NTTI≥Tproc)

- where NTTI is the number of subframes which a HARQ subpacket spans; i.e. 1 for the default TTI and 4 for the long TTI in FDD. The index m in Table 1 indicates the 1st subframe which a long TTI subpacket spans.

Figure 1 – Example of FDD DL HARQ timing for 5, 10 and 20 MHz channel bandwidths

- Figure 1 shows an example of the timing relationship between a DL Basic Assignment A-MAP IE with NA-MAP = 1, a DL HARQ subpacket with the default TTI, corresponding HARQ feedback, and retransmission in FDD frame structure, for 5, 10 and 20 MHz channel bandwidths. In this example, Tproc is 3.

### 15.2.x.2.2.1.2   Uplink

In UL HARQ transmission, UL Basic Assignment A-MAP IE, the HARQ subpacket, the corresponding feedback, and retransmission of the HARQ subpacket shall follow the timing defined in Table 2.

Table 2 – FDD UL HARQ timing

| Content | Subframe index | Frame index |
| --- | --- | --- |
| Basic Assignment A-MAP Tx IE in DL | l | i |
| HARQ Subpacket Tx in UL | m=n or n+NA−MAP−1 where n =ceil(l+F/2) mod F. | j=i+floorceil(l+F2)F+v mod 4 |
| HARQ feedback in DL | l | k=j+floorm+F2F+w mod 4 |
| HARQ Subpacket ReTx in UL | m | p=k+floorceil(l+F2)F+v mod 4 |

UL HARQ subpacket transmission corresponding to a UL Basic Assignment A-MAP IE in l-th DL subframe of the i-th frame shall begin in the m-th UL subframe of the j-th frame. A HARQ feedback for the UL HARQ subpacket shall be transmitted in the l-th DL subframe of the k-th frame. When the UL HARQ feedback indicates a negative-acknowledgement, retransmission of the UL HARQ subpacket shall begin in the m-th UL subframe of the p-th frame. The subframe index m, n and frame index j, k, p shall be determined by using l and i, as shown in Table 2.

Note that the subframe index l shall range from 0 to NA-MAP(ceil(F/NA-MAP) –1) with an increment of NA-MAP. For NA-MAP =2, m shall be selected between n and n+1. The selection information of n shall be provided in UL Basic Assignment A-MAP IE.

- UL HARQ transmission offset v shall be set to 1 only if a time gap from completion of the UL Basic Assignment A-MAP IE transmission to the HARQ subpacket transmission time derived with v = 0 is shorter than the data burst processing time Tproc. Otherwise, v shall be set to 0:

- v=0,      if (ceil(F2)−1≥Tproc)

- UL HARQ feedback offset w shall be set to 1 only if a time gap from completion of the HARQ subpacket transmission to its feedback time derived with w = 0 is shorter than the data burst processing time Tproc. Otherwise, w shall be set to 0. This rule shall be also applied to the long TTI transmission:

- w=0,      if (floor(F2)−NTTI≥Tproc)

- where NTTI is the number of subframes which a HARQ subpacket spans; i.e. 1 for the default TTI and 4 for the long TTI in FDD. The index m in Table 2 indicates the 1st subframe which a long TTI subpacket spans.

Figure 2 – Example of FDD UL HARQ timing for 5, 10 and 20 MHz channel bandwidths.

- Figure 2 shows an example of the timing relationship between a UL Basic Assignment A-MAP IE with NA-MAP = 1, a UL HARQ subpacket with the default TTI, corresponding HARQ feedback and retransmission in FDD frame structure, for 5, 10 and 20 MHz channel bandwidths. In this example, Tproc is 3.

### 15.2.x.2.2.2   TDD

### 15.2.x.2.2.2.1   Downlink

In DL HARQ transmission, DL Basic Assignment A-MAP IE, the HARQ subpacket, and the corresponding feedback shall follow the timing defined in Table 3.

Table 3 – TDD DL HARQ timing

| Content | Subframe index | Frame index |
| --- | --- | --- |
| Basic Assignment A-MAP IE Tx in DL | l | i |
| HARQ Subpacket Tx in DL | m=l or l+NA−MAP−1 | i |
| HARQ feedback in UL | For D > U, n=0, for 0≤m<Km−K, for K≤m<U+K | j = (i+z) mod 4 |
|  | For D ≤ U, n = m – K |  |

DL HARQ subpacket transmission corresponding to a DL Basic Assignment A-MAP IE in l-th DL subframe of the i-th frame shall begin in the m-th DL subframe of the i-th frame. A HARQ feedback for the DL HARQ subpacket shall be transmitted in the n-th UL subframe of the j-th frame. The subframe index m, n and frame index j shall be determined by using l and i, as shown in Table 3. In the table, if the sum of D and U is an odd number and D is less than U/NA-MAP, K = ceil((D-U)/2) for D  U, and K = –ceil((U-D)/2) for D < U. Otherwise, K = floor((D-U)/2) for D  U, and K = –floor((U-D)/2) for D < U.

Note that the subframe index l shall range from 0 to NA-MAP(ceil(D/NA-MAP) –1) with an increment of NA-MAP. For NA-MAP =2, m shall be selected between l and l+1. The selection information of m shall be provided in DL Assignment A-MAP IE.

- DL HARQ feedback offset z shall be set to 1, only if a time gap from completion of the HARQ subpacket transmission to its feedback time derived with z = 0 is shorter than the data burst processing time Tproc. Otherwise, z shall be set to 0. This rule shall be also applied to the long TTI transmission:

- z=0,      if (D−m−NTTI+n≥Tproc)

- where NTTI is the number of subframes which a HARQ subpacket spans; i.e. 1 for the default TTI and D for the long TTI in TDD DL. The index m in Table 3 indicates the 1st subframe which a long TTI subpacket spans.

Figure 3 – Example of TDD DL HARQ timing for 5, 10 and 20 MHz channel bandwidths.

- Figure 3 shows an example of the timing relationship between a DL Basic Assignment A-MAP IE with NA-MAP = 1, a DL HARQ subpacket with the default TTI, corresponding HARQ feedback and retransmission in TDD frame structure, for 5, 10 and 20 MHz channel bandwidths. In this example, Tproc is 3.

### 15.2.x.2.2.2.2   Uplink

In UL HARQ transmission, UL Basic Assignment A-MAP IE, the HARQ subpacket, the corresponding feedback, and retransmission of the HARQ subpacket shall follow the timing defined in Table 4.

Table 4 – TDD UL HARQ timing

| Content | Subframe index | Frame index |
| --- | --- | --- |
| Basic Assignment A-MAP IE Tx in DL | l | i |
| HARQ Subpacket Tx in UL | For ceil(D/ NA-MAP) ≥ U, | j = (i+v) mod 4 |
|  | m=0, for 0≤l<Kl−K, for K≤l<U+K |  |
|  | For 1 < ceil(D/ NA-MAP) < U, |  |
|  | m=0, ⋯, or l−K+NA−MAP−1, for l=0 l−K or l−K+NA−MAP−1, for 0<l<lmax where lmax = NA-MAP(ceil(D/NA-MAP) –1). |  |
|  | For ceil(D/ NA-MAP) = 1 |  |
|  | m = 0, 1, … , or U-1 for l = 0 |  |
| HARQ feedback in DL | l | k = (j+1+w) mod 4 |
| HARQ Subpacket ReTx in UL | m | p = (k+v) mod 4 |

UL HARQ subpacket transmission corresponding to a UL Basic Assignment A-MAP IE in l-th DL subframe of the i-th frame shall begin in the m-th UL subframe of the j-th frame. A HARQ feedback time for the HARQ subpacket shall be transmitted in the l-th DL subframe of the k-th frame. When the UL HARQ feedback indicates a negative acknowledgement, retransmission of the UL HARQ subpacket shall begin in the m-th UL subframe of the p-th frame. The subframe index m, n and frame index j, k, p shall be calculated as shown in Table 4.

In the table, if the sum of D and U is an odd number and D is less than U/NA-MAP, K = ceil((D-U)/2) for D  U, and K = –ceil((U-D)/2) for D < U. Otherwise, K = floor((D-U)/2) for D  U, and K = –floor((U-D)/2) for D < U. Note that the subframe index l shall range from 0 to NA-MAP(ceil(D/NA-MAP) –1) with an increment of NA-MAP.

For ceil(D/ NA-MAP) <U, m for a certain range of l shall be selected one of multiple values. The selection information of m shall be provided in UL Basic Assignment A-MAP IE.

- UL HARQ transmission offset v shall be set to 1 only if a time gap from completion of the UL Assignment A-MAP IE transmission to the HARQ subpacket transmission time derived with v = 0 is shorter than the data burst processing time Tproc. Otherwise, v shall be set to 0:

- v=0,      if (D−l−1+m≥Tproc)

- UL HARQ feedback offset w shall be set to 1 only if a time gap from completion of the HARQ subpacket transmission to its feedback time derived with w = 0 is shorter than the data burst processing time Tproc. Otherwise, w shall be set to 0. This rule shall be also applied to the long TTI transmission:

- w=0,      if (U−m−NTTI+l≥Tproc)

- where NTTI is the number of subframes which a HARQ subpacket spans; i.e. 1 for the default TTI and U for the long TTI in TDD UL. The index m in Table 4 indicates the 1st subframe which a long TTI subpacket spans.

Figure 4 – Example of TDD UL HARQ timing for 5, 10 and 20 MHz channel bandwidths.

- Figure 4 shows an example of the timing relationship between a UL Basic Assignment A-MAP IE with NA-MAP = 1, a UL HARQ subpacket with the default TTI, corresponding HARQ feedback and retransmission in TDD frame structure, for 5, 10 and 20 MHz channel bandwidths. In this example, Tproc is 3.

### 15.2.x.2.2.2.3   HARQ Timing in frame structure supporting the WirelessMAN-OFDMA frames

- The A-MAP relevance and HARQ timing defined in 15.2.x.2.2.2 shall be applied to the frame structure supporting the WirelessMAN-OFDMA TDD frames in 15.3.3.4.1.

- Subframes in the frame supporting the WirelessMAN-OFDMA TDD frames shall be indexed as follows: the DL subframe index shall range from 0 to D-1, where D is the number of DL subframes dedicated to the Advanced Air Interface operation in frame. The UL subframe index shall range from 0 to U-1, where U is the number of UL subframes dedicated to the Advanced Air Interface operation in frame.

- Figure 5 shows an example of subframe indexing for 5, 10 and 20 MHz channel bandwidths. In this example, the ratio of whole DL subframes to whole UL subframes, D’:U’ is 5:3. FRAME_OFFSET is 2, and UL subframes of the WirelessMAN-OFDMA and the Advanced Air Interface are frequency-division multiplexed. Then, the ratio of DL to UL subframes for the Advanced Air Interface, D:U is 3:3. The subframe index, l, m, and n are the renumbered index of l’, m’, and n’, respectively.

- Figure 5 – Example of subframe indexing in frame structure supporting 
the WirelessMAN-OFDMA frame.

- The same equations and rule in Table 3 and 4 shall be applied for deciding HARQ timing with l, m, n, D, and U, except that l’, m’, n’, D’, and U’ shall be used to set z, v, and w, as follows:

- z=0,      if (D'−m'−NTTI+n'≥Tproc)

- v=0,      if (D'−l'−1+m'≥Tproc)

- w=0,      if (U'−m'−NTTI+l'≥Tproc)

Figure 6 – Example of TDD DL HARQ timing in frame structure supporting 
the WirelessMAN-OFDMA frame.

Figure 7 – Example of TDD UL HARQ timing in frame structure supporting 
the WirelessMAN-OFDMA frame.

- Figure 6 and 7 show examples of the DL and UL timing relationships between a Assignment A-MAP IE with NA-MAP = 1, a HARQ subpacket with the default TTI, corresponding HARQ feedback and retransmission, for 5, 10 and 20 MHz channel bandwidths. The ratio of whole DL subframes to whole UL subframes, D’:U’ is 5:3. In this example, FRAME_OFFSET is 2, UL subframes of the WirelessMAN-OFDMA and the Advanced Air Interface are frequency-division multiplexed, the ratio of DL to UL subframes for the Advanced Air Interface, D:U is 3:3, and Tproc is 3.

-------------------------------  Text End  ---------------------------------------------------