| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comment on Recommended AWD Text Proposal #1 of idle mode PC DG contribution |  |
| Date Submitted | 2009-05-02 |  |
| Source(s) | Hyunjeong Kang, Jungje Son, Rakesh Taori Samsung Electronics Mary Chion, Lei Zhang, Li Wang ZTE Ming-Hung Tao, Ying-Chuan Hsiao ITRI Yih Shen Chen MediaTek | hyunjeong.kang@samsung.com zhang.lei13@zte.com.cn mhtao@itri.org.tw yihshen.chen@mediatek.com |
| Re: | Category: AWD-DG comment / Area: Power conservation |  |
| Abstract | The contribution proposes the text of idle mode operation to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comment on Recommended AWD Text Proposal #1 of Idle Mode PC DG Draft

- Text proposal

-------------------------------  Text Start  ---------------------------------------------------

- Advanced Air Interface

- 15.2.x.x Idle mode

[Change the 1st paragraph in subclause 15.2.x.x as below:]

An ABS may be a member of one or more paging groups that may have different cycle and offset. When AMS operating in legacy mode select the mixed ABS as a preferred ABS, AMS mayshould stay in the Lzone and perform the legacy Idle Mode operation. If an idle mode AMS in the Lzone of a mixed mode ABS decides to move to the Mzone of the ABS, in which case the AMS shall perform location update. When an AMS is paged Paging message is received in the Lzone of a mixed mode ABS, the AMS can performs the network re-entry in the either LZone of the ABS and may switch to the MZone of the ABS or MZone.

-------------------------------  Text End  ---------------------------------------------------