| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comment on Recommended AWD Text Proposal #2 of idle mode PC DG contribution |  |
| Date Submitted | 2009-05-02 |  |
| Source(s) | Hyunjeong Kang, Baowei Ji, Anshuman Nigam, Jungje Son, Rakesh Taori Samsung Electronics | E-mail: hyunjeong.kang@samsung.com |
| Re: | Category: AWD-DG comment / Area: Power conservation |  |
| Abstract | The contribution proposes the text of idle mode operation to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comment on Recommended AWD Text Proposal #2 of Idle Mode PC DG Draft

- Text proposal

-------------------------------  Text Start  ---------------------------------------------------

- 15.2.x.x.2.3 Operation during paging listening interval

[Replace a text “The location of paging message is TBD” in line 18 of page 10 as follows:]

The frame Npaging frame that includes the paging message is pre-determined for each AMS.

The AMS can be explicitly assigned a frame Npaging frame to monitor paging message at idle mode initiation using a DREG-CMD message. If the frame Npaging frame, is not assigned at idle mode initiation, the AMS can implicitly determine the frame Npaging frame, as follows:

Npaging_frame = AMS’s temporary identifier mod 4

-------------------------------  Text End  ---------------------------------------------------