| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for the DL and UL Persistent Allocation A-MAP IEs in the IEEE 802.16m AWD |  |
| Date Submitted | 2009-05-05 |  |
| Source(s) | Roshni Srinivasan, Hujun Yin, Shantidev Mohanty, Kamran Etemad Intel Corporation Seho Kim, Jason Junsung Lim, Heewon Kang, Hokyu Choi Samsung Electronics Co., Ltd. Yih-Shen Chen, Paul Cheng Mediatek Ming-Hung Tao ITRI | roshni.m.srinivasan@intel.com seho42.kim@samsung.com yihshen.chen@mediatek.com mhtao@itri.org.tw |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6 DL-CTRL” |  |
| Abstract | The contribution proposes updates to the text related to persistent allocation in the 802.16m AWD |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for the DL and UL Persistent Allocation A-MAP IEs in the IEEE 802.16m AWD

Roshni Srinivasan, Hujun Yin, Shantidev Mohanty, Kamran Etemad,

Intel Corporation

Seho Kim, Jason Junsung Lim, Heewon Kang, Hokyu Choi,

Samsung Electronics Co., Ltd.

Yih-Shen Chen, Paul Cheng,

Mediatek

Ming-Hung Tao,

ITRI

Introduction

The current text in section 15.3.6.4.2.5, including Table 664 describes the DL Persistent A-MAP IE. Section 15.3.6.4.2.6 is a placeholder for the UL Persistent A-MAP IE.

New subclauses and tables describing the Individual and Composite Persistent A-MAP IEs and for the DL and UL are proposed. Since all retransmissions are scheduled non-persistently, the AI_SN and SPID/CoRe fields are not included in the Individual and Composite Persistent A-MAP IEs. Instead, the DL/UL Individual and Composite Persistent A-MAP IEs include the Allocation Period and N_ACID to handle persistent operation with ACID cycling. All other fields in the DL and UL Individual and Composite Persistent A-MAP IEs are consistent with the features described in the AWD [1].

The proposed IEs are consistent with the MAC procedure described in the latest revision of [2] and [3].

In order to limit the feedback overhead on the UL for a large number of VoIP users, it is proposed that only diversity allocations that utilize DRUs or mini-band based CRUs are supported for VoIP. Only non-adaptive precoding is supported. The following table describes the supported MIMO modes and corresponding feedback requirements.

MIMO modes supported for VoIP

| MIMO Mode | Description | Feedback content | Resource Type |
| --- | --- | --- | --- |
| Mode 0 | OL SU MIMO STBC | 1. Rank 2. Wideband CQI | DRU or Mini-band based CRU |
| Mode 1 | OL SU MIMO SM | 1. Rank 2. Wideband CQI | DRU or Mini-band based CRU |
| Mode 2 | CL SU MIMO | 1. Rank 2. Wideband CQI 3. Correlation matrix | Mini-band based CRU |
| Mode 4 | CL MU MIMO | 1. Wideband CQI 2. Correlation matrix | Mini-band based CRU |

Configurations of 2, 4 and 8 transmit antennas are supported in the DL Individual Persistent A-MAP IE, and 2 and 4 transmit antennas are supported in the UL Individual Persistent A-MAP IE.

The current AWD text in sections 15.3.6.4.2.5 and 15.3.6.4.2.6 have been replaced by the proposed text as shown below. Proposed text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through. It must be noted that definitions that followed the DL PA A-MAP IE in the current AWD were also deleted since they are already contained in the 802.16e standard and do not need to be repeated.

References

[1] P802.16m Amendment Working document, 802.16m-09/0010r1a, March 2009.

[2] C802.16m-09/1136, “Proposed Text for the MAC Procedure for DL and UL Persistent Allocation in the IEEE 802.16m Amendment”, May 2009.

[3] C802.16m-09/926, “Proposed Text for the MAC Procedure for Persistent Allocation in the IEEE 802.16m Amendment”, April 2009.

Text proposal for inclusion in 802.16m amendment

------------------------------------------- Start of Proposed Amendment Text -----------------------------------------------

### 15.3.6.4.2.5 DL PA A-MAP IE

The DL persistent A-MAP IE is specified in Table 664.

Table 664: DL Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Persistent A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | DL Persistent A-MAP IE |
| if MCRC is masked with Station ID { |  |  |
| DL Individual Persistent A-MAP_IE() |  | Refer to Table 665 |
| } else if MCRC is masked with Composite ID { |  |  |
| DL Composite Persistent A-MAP_IE() |  | Refer to Table 666 |
| } |  |  |
| MCRC | [16] | 16 bit CRC masked by Station ID for individual PA and masked with composite ID (well-known ID specified in the system, TBD) for multi-user PA. |
| } |  |  |

### 15.3.6.4.2.5.1 DL Individual PA A-MAP IE

The DL individual persistent A-MAP IE is specified in Table 665.

Table 665: DL Individual Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Persistent A-MAP_IE() { | - | - |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| If (Allocation Period==0b00){ |  |  |
| Resource Allocation | Variable | Variable number of bits - depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) - Allocation size |
| HFA | [4] | TBD HARQ Feedback Allocation |
| } else if (Allocation Period != 0b00){ |  |  |
| MCS | 4 | Depends on supported modes, 16 modes assumed as baseline |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: n/a |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| if(Nt == 2){ |  |  |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| }else if(Nt == 4){ |  |  |
| Mt | 2 | Number of streams in transmission for Nt = 4 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| }else if(Nt == 8){ |  |  |
| Mt | 3 | Number of streams in transmission for Nt = 8 (Mt <= Nt ) 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| } |  |  |
| } else if(MEF == 0b10){ |  | Parameters for horizontal encoding |
| if(Nt == 2){ |  |  |
| PSI | 1 | Allocated pilot stream index for Nt = 2 0b0: #1 stream 0b1: #2 stream |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| } else{ |  |  |
| PSI | 2 | Allocated pilot stream index for Nt = 4 or 8 0b00: #1 stream 0b01: #2 stream 0b10: #3 stream 0b11: #4 stream |
| Mt | 2 | Number of streams in transmission for Nt = 4 or 8 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| } |  |  |
| } |  |  |
| Resource Allocation | Variable | Variable number of bits - depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) - Allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| if (Nsubframe, A-MAP == 2){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| } |  |  |
| HFA | [4] | TBD HARQ Feedback Allocation |
| ACID | 4 | HARQ channel identifier. The ACID field shall be set to the initial value of HARQ channel identifier for implicit cycling of HARQ channel identifiers. |
| N_ACID | 2 | Number of ACIDs for implicit cycling of HARQ channel identifier 0b00: 2 0b01: 3 0b10: 4 0b11: 5 |
| } |  |  |
| Reserved | TBD | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| } | - | - |

### 15.3.6.4.2.5.2 DL Composite PA A-MAP IE

The DL composite persistent A-MAP IE is specified in Table 666.

Table 666: DL Composite Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Composite Persistent A-MAP_IE() { | - | - |
| Number of allocations | 5 | Number of allocation specified |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| For (j=0;j<Number of allocations; j++) { |  | For loop where each loop element specifies information for one allocation. |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| RCID | variable | Specifies the station ID in RCID format, type defined by RCID Type |
| Allocation MCS indicator | 1 | If Allocation MCS Indicator is 1, it indicates that MCS is explicitly assigned for this allocation. Otherwise, this allocation will use the same MCS as the previous subburst. If j is 0 then this indicator shall be 1. |
| if (Allocation MCS indicator == 1) { |  |  |
| MCS | 4 | Depends on supported modes, 16 modes assumed as baseline |
| } |  |  |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: n/a |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| if(Nt == 2){ |  |  |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| }else if(Nt == 4){ |  |  |
| Mt | 2 | Number of streams in transmission for Nt = 4 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| }else if(Nt == 8){ |  |  |
| Mt | 3 | Number of streams in transmission for Nt = 8 (Mt <= Nt ) 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| } |  |  |
| } else if(MEF == 0b10){ |  | Parameters for horizontal encoding |
| if(Nt == 2){ |  |  |
| PSI | 1 | Allocated pilot stream index for Nt = 2 0b0: #1 stream 0b1: #2 stream |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| } else{ |  |  |
| PSI | 2 | Allocated pilot stream index for Nt = 4 or 8 0b00: #1 stream 0b01: #2 stream 0b10: #3 stream 0b11: #4 stream |
| Mt | 2 | Number of streams in transmission for Nt = 4 or 8 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| } |  |  |
| } |  |  |
| Resource Allocation Indicator | 1 | If Resource Allocation Indicator is 1, it indicates that Duration is explicitly assigned for this subburst (allocation or deallocation). Otherwise, this subburst (allocation or deallocation) will use the same Duration as the previous subburst. If j is 1 then this indicator shall be 1. |
| if (Resource Allocation Indicator ==1) { |  |  |
| Resource Allocation | Variable | Variable number of bits - depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) - Allocation size |
| if (Nsubframe, A-MAP == 2){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| } |  |  |
| } |  |  |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | [4] | TBD HARQ Feedback Allocation |
| if (Persistent Flag == 1) { |  |  |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| if (Allocation Period==0b00){ |  |  |
| Resource Allocation | Variable | Variable number of bits – depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) Allocation size |
| HFA | [4] | TBD HARQ Feedback Allocation |
| } else if (Allocation Period != 0b00){ |  |  |
| Allocation Period and ACID Indicator | 1 | If Allocation Period and ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (ACID) is explicitly assigned for this allocation. Otherwise, this allocation will use the same allocation period as the previous allocation. If j is 0 then this indicator shall be 1. |
| if (Allocation Period and ACID Indicator == 1) { | - | - |
| Allocation Periodicity | 2 | Periodicity of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| Allocation Period (AP) | 5 | Period of the persistent allocation is this field value plus 1 (unit is sub-frame/frame TBD) |
| ACID | 4 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

### 15.3.6.4.2.6 UL PA A-MAP IE

The UL persistent A-MAP IE is specified in Table 667.

Table 667: UL Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Persistent A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Persistent A-MAP IE |
| if MCRC is masked with Station ID { |  |  |
| UL Individual Persistent A-MAP_IE() |  | Refer to Table 668 |
| } else if MCRC is masked with Composite ID { |  |  |
| UL Composite Persistent A-MAP_IE() |  | Refer to Table 669 |
| } |  |  |
| MCRC | [16] | 16 bit CRC masked by Station ID for individual PA and masked with composite ID (well-known ID specified in the system, TBD) for multi-user PA. |
| } |  |  |

### 15.3.6.4.2.6.1 UL Individual PA A-MAP IE

The UL individual persistent A-MAP IE is specified in Table 668.

Table 668: UL Individual Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Persistent A-MAP_IE() { | - | - |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of persistent resource. 0b00: 0 frame 0b01: 2 frames 0b10: 4 frames 0b11: 6 frames |
| If (Allocation Period==0b00){ |  |  |
| Resource Allocation | Variable | Variable number of bits - depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) - Allocation size |
| HFA | [4] | TBD HARQ Feedback Allocation |
| } else if (Allocation Period != 0b00){ |  |  |
| MCS | 4 | MCS used for burst |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding |
| CSM | 1 | - 0b0: CSM enabled 0b1: No CSM enabled |
| If(MEF == 0b1){ |  | Paraneters for vertical encoding |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ) 0b00: 1 stream 0b11: 2 streams 0b00: 3 stream 0b11: 4 streams |
| if(CSM == 0b1){ |  |  |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if(TNS == 0b01){ |  |  |
| SI | - 1 | - First pilot index for CSM with TNS = 2 |
| } else if( TNS == 0b10 or TNS==0b11){ |  |  |
| SI | - 2 | - First pilot index for CSM with TNS = 3,4 |
| } |  |  |
| } |  |  |
| PF | 1 | - Precoding Flag - 0b0: non adaptive precoding 0b1: adaptive codebook precoding using the precoder of rank Mt of MS’s choice |
| } |  |  |
| Resource Allocation | Variable | Variable number of bits - depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) - Allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD |
| if (Nsubframe, A-MAP == 2){ |  |  |
| if (DL:UL != 3:5){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 8:0, 6:2, 4:4 or 5:3 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| }else if (DL:UL == 3:5){ |  |  |
| Allocation Relevance | 2 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 3:5 i.e., the first A-MAP region is relevant to the first two UL subframes and the second A-MAP region is relevant to the last 3 UL subframes 0b00: Allocation in the first UL subframe relevant to an A-MAP region 0b01: Allocation in the second UL subframe relevant to an A-MAP region 0b10: Allocation in the third UL subframe relevant to an A-MAP region 0b11: reserved |
| } |  |  |
| } |  |  |
| else if (Nsubframe, A-MAP == 1 and DL:UL == 3:5){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every subframes (Nsubframe, A-MAP =1) and DL:UL subframe ratio is 3:5 i.e., the first A-MAP region is relevant to the first UL subframe and the next two A-MAP regions are relevant to two UL subframes each. 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| } |  |  |
| HFA | [4] | TBD HARQ Feedback Allocation |
| ACID | 3 | HARQ channel identifier |
| N_ACID | 2 | Number of ACID for implicit cycling of HARQ channel identifier 0b00: 1 0b01: 2 0b10: 3 0b11: 4 |
| } |  |  |
| Reserved | TBD | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| } | - | - |

### 15.3.6.4.2.6.2 UL Composite PA A-MAP IE

The UL composite persistent A-MAP IE is specified in Table 669.

Table 669: UL Composite Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Composite Persistent A-MAP_IE() { | - | - |
| Number of allocations | 5 | Number of allocation specified |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| For (j=0;j<Number of allocations; j++) { |  | For loop where each loop element specifies information for one allocation. |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| RCID | variable | Specifies the station ID in RCID format, type defined by RCID Type |
| Allocation MCS indicator | 1 | If Allocation MCS Indicator is 1, it indicates that MCS is explicitly assigned for this allocation. Otherwise, this allocation will use the same MCS as the previous subburst. If j is 0 then this indicator shall be 1. |
| if (Allocation MCS indicator == 1) { |  |  |
| MCS | 4 | Depends on supported modes, 16 modes assumed as baseline |
| } |  |  |
| MEF | 1 | MIMO encoder format - 0b0: SFBC 0b1: Vertical encoding |
| CSM | 1 | - 0b0: CSM enabled 0b1: No CSM enabled |
| if(MEF == 0b1){ |  | Paraneters for vertical encoding |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ) 0b00: 1 stream 0b11: 2 streams 0b00: 3 stream 0b11: 4 streams |
| if(CSM == 0b1){ |  |  |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if(TNS == 0b01){ |  |  |
| SI | 1 | First pilot index for CSM with TNS = 2 |
| } else if( TNS == 0b10 or TNS==0b11){ |  |  |
| SI | 2 | First pilot index for CSM with TNS = 3,4 |
| } |  |  |
| } |  |  |
| PF | 1 | - Precoding Flag - 0b0: non adaptive precoding 0b1: adaptive codebook precoding using the precoder of rank Mt of MS’s choice |
| Resource Allocation Indicator | 1 | If Resource Allocation Indicator is 1, it indicates that Duration is explicitly assigned for this subburst (allocation or deallocation). Otherwise, this subburst (allocation or deallocation) will use the same Duration as the previous subburst. If j is 1 then this indicator shall be 1. |
| if (Resource Allocation Indicator ==1) { |  |  |
| Resource Allocation | Variable | Variable number of bits - depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) - Allocation size |
| if (Nsubframe, A-MAP == 2){ |  |  |
| if (DL:UL != 3:5){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 8:0, 6:2, 4:4 or 5:3 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| }else if (DL:UL == 3:5){ |  |  |
| Allocation Relevance | 2 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 3:5 i.e., the first A-MAP region is relevant to the first two UL subframes and the second A-MAP region is relevant to the last 3 UL subframes 0b00: Allocation in the first UL subframe relevant to an A-MAP region 0b01: Allocation in the second UL subframe relevant to an A-MAP region 0b10: Allocation in the third UL subframe relevant to an A-MAP region 0b11: reserved |
| } |  |  |
| }else if (Nsubframe, A-MAP == 1 and DL:UL == 3:5){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every subframes (Nsubframe, A-MAP =1) and DL:UL subframe ratio is 3:5 i.e., the first A-MAP region is relevant to the first UL subframe and the next two A-MAP regions are relevant to two UL subframes each. 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| } |  |  |
| } |  |  |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | [4] | TBD HARQ Feedback Allocation |
| if (Persistent Flag == 1) { |  |  |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| if (Allocation Period==0b00){ |  |  |
| Resource Allocation | Variable | Variable number of bits – depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) Allocation size |
| HFA | [4] | TBD HARQ Feedback Allocation |
| } else if (Allocation Period != 0b00){ |  |  |
| Allocation Period and ACID Indicator | 1 | If Allocation Period and ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (ACID) is explicitly assigned for this allocation. Otherwise, this allocation will use the same allocation period as the previous allocation. If j is 0 then this indicator shall be 1. |
| if (Allocation Period and ACID Indicator == 1) { | - | - |
| Allocation Periodicity | 2 | Periodicity of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| Allocation Period (AP) | 5 | Period of the persistent allocation is this field value plus 1 (unit is sub-frame/frame TBD) |
| ACID | 4 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

15.3.6.4.2.5 DL PA A-MAP IE

The DL persistent A-MAP IE is shown in Table 664.

Table 664: DL Persistent A-MAP IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| DL Persistent A-MAP IE () { | -- | -- |
| AMAP type | 4 | DL Persistent A-MAP IE |
| Number of allocations | 5 | Number of allocation specified |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| ACK Region Index | 1 | The index of the ACK region associated with all sub-bursts defined in this DL Persistent A-MAP IE |
| While (data remaining) { | -- | -- |
| Region ID use indicator | 1 | 0: Region ID not used 1: Region ID used |
| if (Region ID use indicator ==0) { |  |  |
| Region information | TBD | TBD Information that specifies the region relevant to this persistent scheduling instance |
| } else { | -- | -- |
| Region ID | 8 | Index to the DL region defined in DL region definition S-SFH |
| } | -- | -- |
| For (j=0;j<Number of allocations; j++) { |  | For loop where each loop element specifies information for one allocation. |
| Duration Indicator | [1] | If Duration Indicator is 1, it indicates that Duration is explicitly assigned for this subburst (allocation or deallocation). Otherwise, this subburst (allocation or deallocation) will use the same Duration as the previous subburst. If j is 1 then this indicator shall be 1. |
| MAP ACK Channel Index | TBD | TBD Depends on MAP ACK channel definition in UL control Index to a MAP ACK channel within the Fast Feedback region. The value 111111 is reserved. When MAP ACK Channel Index = 111111, it indicates NO MAP ACK channel is assigned to this de-allocation. |
| RCID_IE() | variable | Specifies the station ID in RCID format |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| If (Duration Indicator == 1) { | - | - |
| Duration | TBD | Duration in number of LRUs. |
| } |  |  |
| Resource Offset | TBD | Indicates the start of this persistent allocation in the allocation region. |
| If (Persistent Flag == 1) { |  |  |
| Allocation Period and N_ACID Indicator | 1 | If Allocation Period and N_ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (N_ACID) is explicitly assigned for this allocation. Otherwise, this allocation will use the same allocation period as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation Period and N_ACID Indicator == 1) { | - | - |
| Allocation Period (AP) | 5 | Period of the persistent allocation is this field value plus 1 (unit is sub-frame/frame TBD) |
| Number of ACID (N_ACID) | 3 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } |  |  |
| MAP NACK Channel Index | 6 | TBD Index to a shared MAP NACK channel within the Fast Feedback region. The value 111111 is reserved. When MAP NACK Channel Index = 111111, it indicates NO MAP NACK channel is assigned to this allocation. |
| MAP ACK Channel Index | 6 | TBD Index to a MAP ACK channel within the Fast Feedback region |
| Allocation MCS indicator | 1 | If Allocation MCS Indicator is 1, it indicates that MCS is explicitly assigned for this allocation. Otherwise, this allocation will use the same MCS as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Allocation MCS indicator ==1) { |  |  |
| MCS | [6] | TBD Depends on supported modulation and coding schemes Specifies the modulation, coding and repetition coding used for this allocation |
| } |  |  |
| Allocation Boosting indicator | [1] | If Allocation Boosting Indicator is 1, it indicates that Boosting is explicitly assigned for this allocation. Otherwise, this allocation will use the same Boosting as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation Boosting indicator ==1) { |  |  |
| Boosting | [3] | Specifies the boosting used for this allocation |
| } |  |  |
| Allocation ACID indicator | 1 | If Allocation ACID Indicator is 1, it indicates that ACID is explicitly assigned for this allocation. Otherwise, this allocation will use the same ACID as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation ACID indicator ==1) { |  |  |
| ACID | [5] | Specifies the HARQ channel identifier used for this allocation |
| } |  |  |
| AI_SN | 1 | HARQ identifier sequence number |
| SPID/CoRE Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| } |  |  |
| } |  |  |
| Allocation Boosting indicator | [1] | If Allocation Boosting Indicator is 1, it indicates that Boosting is explicitly assigned for this allocation. Otherwise, this allocation will use the same Boosting as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation Boosting indicator ==1) { |  |  |
| Boosting |  |  |
| } |  |  |
| Allocation ACID indicator |  |  |
| If (Allocation ACID indicator ==1) { |  |  |
| ACID | [4][5] | HARQ channel identifier |
| } |  |  |
| AI_SN | 1 | HARQ identifier sequence number |
| SPID/CoRE Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| } |  |  |
| Padding | variable | Padding to bytes boundary; padding value shall be set to zero. |
| MCRC | [16] | 16 bit masked CRC |
| } |  |  |

Persistent Flag

The persistent flag shall be set to 1 if the assignment is persistent and shall be set to 0 if the assignment is non-persistent.

DRU Offset

The DRU offset shall be set to the first DRU in the time-frequency resource assignment with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the Allocation region.

Duration Indicator

Duration Indicator flag determines whether or not Duration is specified for an allocation. If this flag is 1, it indicates that Duration is explicitly assigned for an allocation. Otherwise, the allocation has the same Duration as the previous allocation. This flag shall be 1 for the first allocation in a Allocation region.

Duration

Duration specifies the size (# DRUs) of an allocation/de-allocation/reallocation in a Allocation region.

Allocation Period and N_ACID Indicator

If Allocation Period and Index Indicator is 1, it indicates that allocation period, ACK and NACK channel index (allocation period and Number of ACID (N_ACID) is explicitly assigned for an allocation. Otherwise, the allocation will use the same allocation period and N_ACID as the previous allocation. This flag shall be 1 for the first allocation in a Allocation region.

Allocation Period (ap)

The allocation period (ap) shall be set to one less than the period of the persistent allocation, in units of sub-frames/frames.  For example, as illustrated below, if ap=0b00011, then the period of the persistent allocation is four frames, and the time-frequency resource assignment is valid in frames N, N+4, N+8, etc.

N_ACID

The values of ACID field (N0) and N_ACID field (N) are used together to specify an implicit cycling of HARQ channel identifiers as follows.  N0 is used as the HARQ channel identifier corresponding to the first occurrence of the persistent allocation. For each next allocation this value is incremented modulo (N + 1)

As illustrated below, if N_ACID = 0b011 (meaning the period Num_HARQ_Chan = 4), and if ACID = 2, the HARQ channel identifier follows the pattern 2, 3, 4, 5, 2, 3, 4, 5, etc.

ACID

The ACID field shall be set to the initial value of HARQ channel identifier as described above.

AI_SN

The AI_SN field value shall be set to the initial ARQ identifier sequence number for each HARQ channel.  The AI_SN toggles between 0 and 1 for each    particular HARQ channel.  For example, if the period equals 4 frames, N_ACID = 4, ACID = 2, and AI_SN = 0, the ACID follows the pattern 2, 3, 4, 5, 2, 3, 4, 5, etc, and the AI_SN follows the pattern 0, 0, 0, 0, 1, 1, 1, 1, etc.

ACK_channel

The ACK_channel field shall be set to the number of the ACK channel within the HARQ ACK Region.  The mobile station shall use the indicated ACK channel for transmitting acknowledgment information for each packet received using the time-frequency resource referred to by this persistent allocation.

MAP NACK Channel Index

The MAP NACK channel index is persistently allocated within the Fast Feedback region.  The mobile station shall used the indicated MAP NACK channel to report MAP decoding error in frames where it as a persistent resource allocation assigned with this instance of the persistent IE. The value 111111 is reserved.  When MAP NACK Channel Index = 111111, it indicates NO MAP NACK channel is assigned to this allocation.

MAP ACK Channel Index

The MAP ACK channel is allocated non-persistently within the Fast Feedback region.  The mobile station shall use the indicated MAP ACK channel to report successful receipt of the persistent allocation IE. The value 111111 is reserved.  When MAP ACK Channel Index = 111111, it indicates NO MAP ACK channel is assigned to this allocation

15.3.6.4.2.6 DL PA A-MAP IE

--------------------------------------------- End of Proposed Amendment Text ---------------------------------------------