| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for the MAC Procedure for DL and UL Persistent Allocation in the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-05-05 |  |
| Source(s) | Roshni Srinivasan, Hujun Yin, Shantidev Mohanty, Kamran Etemad Intel Corporation Seho Kim, Jason Junsung Lim, Heewon Kang, Hokyu Choi Samsung Electronics Co., Ltd. Yih-Shen Chen, Paul Cheng Mediatek Ming-Hung Tao ITRI | roshni.m.srinivasan@intel.com seho42.kim@samsung.com yihshen.chen@mediatek.com mhtao@itri.org.tw |
| Re: | Category: AWD comments / Area: Chapter 15.2.6 (Persistent Scheduling) “Comments on AWD 15.2.6 Persistent Scheduling” |  |
| Abstract | The contribution proposes text for the MAC procedure for persistent allocation to be included in the 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for the MAC Procedure for DL and UL Persistent Allocation in the IEEE 802.16m Amendment

Roshni Srinivasan, Hujun Yin, Shantidev Mohanty, Kamran Etemad

Intel Corporation

Seho Kim, Jason Junsung Lim, Heewon Kang, Hokyu Choi,

Samsung Electronics Co., Ltd.

Yih-Shen Chen, Paul Cheng,

Mediatek

Ming-Hung Tao,

ITRI

Introduction

This contribution proposes text for the MAC procedure for persistent allocation in the 802.16m AWD based on the template described in section 15.2.6 [1].

Resources may be persistently allocated or individual connections or simultaneously for multiple connections. The proposed text specifies the procedure for such allocation. The DL and UL Persistent A-MAP IEs required to support persistent allocation consistent with the MAC procedure in this contribution are described in [2] and [3].

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

References

[1] P802.16m Amendment Working document, 802.16m-09/0010r1a, March 2009.

[2] C802.16m-09/1135, Proposed Text for the DL and UL Persistent Allocation A-MAP IEs in the IEEE 802.16m AWD, May 2009.

[3] C802.16m-09/925, Proposed Updates to the Persistent Allocation A-MAP IE in the IEEE 802.16m AWD, April 2009.

------------------------------------------- Start of Proposed Amendment Text -----------------------------------------------

15.2.6 Persistent Scheduling in the Advanced Air Interface

Persistent allocation is a technique used to reduce assignment overhead for connections with periodic traffic pattern and with relatively fixed payload size. To allocate resources persistently to a single connection, the ABS shall transmit the DL Individual Persistent Allocation A-MAP IE for DL allocations and the UL Individual Persistent Allocation A-MAP IE for UL allocations. To allocate resources persistently to multiple connections, the ABS shall transmit the DL Composite Persistent Allocation A-MAP IE for DL allocations and the UL Composite Persistent Allocation A-MAP IE for UL allocations. The persistently allocated resource size, position and the MCS shall be maintained by the ABS and AMS until the persistent assignment is de-allocated, changed, or an error event occurs. Persistent scheduling does not include special arrangements for HARQ retransmission of data initially transmitted using persistently allocated resources. Resources for retransmissions can be allocated one at a time as needed using a DL Basic Assignment A-MAP IE or a DL Basic Assignment A-MAP IE.

### 15.2.6.1 Allocation Mechanism

### 15.2.6.1.1 Allocation Mechanism for an Individual Connection

For individual persistent allocation in the DL/UL, the ABS shall transmit the DL/UL Individual Persistent A-MAP IE. Allocation of the persistently assigned resource begins in the DL/UL subframe that is referenced by the DL/UL Individual Persistent A-MAP IE and repeats after an allocation period that is specified in the DL/UL Individual Persistent A-MAP IE. The attributes of the persistently allocated resource including size, location, MIMO encoder format and MCS are maintained as per the DL/UL Individual Persistent A-MAP IE. The values of ACID field and N_ACID field in the DL/UL Individual Persistent A-MAP IE are used together to specify an implicit cycling of HARQ channel identifiers. The allocation period and number of ACIDs required for persistent operation are configured in the DL/UL Individual Persistent A-MAP IE.

In order to facilitate link adaptation and avoid resource holes, the attributes of a persistently allocated resource can be changed. To change an individual persistent assignment, the ABS shall transmit the DL Individual Persistent A-MAP IE for DL reallocation and the UL Individual Persistent A-MAP IE for UL reallocation respectively. If an AMS has an existing individual persistent allocation in a particular subframe and receives a new individual persistent allocation in the same subframe, the new individual persistent allocation replaces the original allocation (i.e., the original persistent allocation is de-allocated).

### 15.2.6.1.1 Allocation Mechanism for Multiple Connections

For multiple persistent allocations in the DL/UL, the ABS shall transmit the DL/UL Composite Persistent A-MAP IE. Allocation of the persistently assigned resource for each connection begins in the DL/UL subframe that is referenced by the DL/UL Composite Persistent A-MAP IE and repeats after an allocation periods that are specified in the DL/UL Composite Persistent A-MAP IE. The attributes of the persistently allocated resource for each connection including size, location, MIMO encoder format and MCS are maintained as per the DL/UL Composite Persistent A-MAP IE. The values of ACID field and N_ACID field in the DL/UL Composite Persistent A-MAP IE are used together to specify an implicit cycling of HARQ channel identifiers. The allocation period and number of ACIDs required for persistent operation are configured in the DL/UL Composite Persistent A-MAP IE.

In order to facilitate link adaptation and avoid resource holes, the attributes of a persistently allocated resource can be changed. To change persistent assignments for multiple connections, the ABS shall transmit a DL Composite Persistent A-MAP IE for DL reallocation and the UL Composite Persistent A-MAP IE for UL reallocation respectively. If an AMS has an existing persistent allocation in a particular subframe and receives a new persistent allocation in the same subframe, the new persistent allocation replaces the original allocation (i.e., the original persistent allocation is de-allocated).

### 15.2.6.2 Deallocation Mechanism

### 15.2.6.2.1 Deallocation Mechanism for an Individual Connection

For deallocation of individual persistent allocations in the DL/UL, the ABS shall transmit the DL/UL Individual Persistent A-MAP IE. When the Allocation Period is set to 0b00 in the DL/UL Individual Persistent A-MAP IE, the assigned persistent resource in DL/UL Individual Persistent A-MAP IE is deallocated in referenced DL/UL subframe and the ABS terminates the persistent allocation.

### 15.2.6.2.2 Deallocation Mechanism for Multiple Connections

For deallocation of multiple persistent allocations in the DL/UL, the ABS shall transmit the DL/UL Composite Persistent A-MAP IE. When the Allocation Period is set to 0b00 for each connection that is being deallocated in the DL/UL Composite Persistent A-MAP IE, the assigned persistent resource in DL/UL Composite Persistent A-MAP IE is deallocated in referenced DL/UL subframe and the ABS terminates the persistent allocation.

### 15.2.6.3 HARQ Retransmissions

Asynchronous HARQ retransmission is used for downlink individual and composite persistent allocations. The DL Basic Assignment A-MAP IE is transmitted to signal control information for HARQ retransmission. Synchronous HARQ retransmission is used for uplink individual and composite persistent allocations. The UL Basic Assignment A-MAP IE is transmitted to signal control information for HARQ retransmission.

### 15.2.6.4 Error Handling Procedure

### 15.2.6.4.1 Error Handling Procedure for an Individual Connection

For transmissions with HARQ enabled, an ACK is transmitted to acknowledge the successful decoding of a data burst, or a NACK is transmitted to notify failure in decoding a burst transmitted on the DL/UL. If an ACK or a NACK for the initial data burst identified by the DL/UL Individual Persistent A-MAP IE is detected in the assigned HARQ Feedback channel, the ABS shall assume that the DL/UL Individual Persistent A-MAP IE is correctly received.

In the absence (NULL detection) of an ACK or a NACK in the HARQ feedback channel assigned in the DL/UL Individual Persistent A-MAP IE, the ABS shall assume that the AMS has not received the DL/UL Individual Persistent A-MAP IE and the same DL/UL Persistent A-MAP IE can be transmitted again.

In the case of deallocation of individual persistent allocations in the DL/UL, the ABS shall transmit a HARQ Feedback Allocation in the DL/UL Individual Persistent A-MAP IE. This allocation is used to identify the HARQ channel in which the ACK for the DL/UL Individual Persistent A-MAP IE signaling the deallocation is transmitted. In the absence (NULL detection) of an ACK, the  shall assume that the AMS has not received  the DL/UL Individual Persistent A-MAP IE, and the same DL/UL Persistent A-MAP IE that signaled the deallocation can be transmitted again.

### 15.2.6.4.2 Error Handling Procedure for Multiple Connections

For transmissions with HARQ enabled, an ACK is transmitted to acknowledge the successful decoding of a data burst, or a NACK is transmitted to notify failure in decoding a burst transmitted on the DL/UL. If an ACK or a NACK for all the initial data bursts identified by the DL/UL Compoosite Persistent A-MAP IE is detected in the assigned HARQ Feedback channel, the ABS shall assume that the DL/UL Composite Persistent A-MAP IE is correctly received by all the connections.

In the absence (NULL detection) of an ACK or a NACK in the HARQ feedback channel assigned to one or more connections in the DL/UL Composite Persistent A-MAP IE, the ABS shall assume that the corresponding AMSs have not received the DL/UL Composite Persistent A-MAP IE and the DL/UL Composite A-MAP IE can be transmitted again for these connections. If the persistent allocation needs to be transmitted to only one connection the DL/UL Individual A-MAP IE can be transmitted again for this connection.

In the case of deallocation of multiple persistent allocations in the DL/UL, the ABS shall transmit HARQ Feedback Allocations in the DL/UL Composite Persistent A-MAP IE. These allocations are used to identify the HARQ channels in which the ACK for the DL/UL Composite Persistent A-MAP IE signaling the deallocations are transmitted. In the absence (NULL detection) of an ACK from one or more connections, the ABS shall assume that the corresponding AMSs have not received the DL/UL Composite Persistent A-MAP IE, and the same DL/UL Composite A-MAP IE that signaled the deallocation can be transmitted again. If the deallocation needs to be transmitted to only one connection the DL/UL Individual A-MAP IE can be transmitted again for this connection.

--------------------------------------------- End of Proposed Amendment Text ----------------------------------------------