| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text of HARQ Timing for IEEE 802.16m Amendment |  |
| Date Submitted | 2009-05-03 |  |
| Source(s) | Doo-hyun Sung, Youngsoo Yuk and Jin Sam Kwak LG Electronics | +82-31-450-1910 dh.sung@lge.com |
| Re: | Category: AWD-DG comments / Area: HARQ Protocol DG “Comments on the Proposed Text of HARQ Protocol DG” |  |
| Abstract | This contribution provides text proposal on HARQ timing |  |
| Purpose | To be discussed and adopted by TGm AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text of HARQ Timing for IEEE 802.16m Amendment

Doo-hyun Sung, Youngsoo Yuk and Jin Sam Kwak

LG Electronics

- Introduction

This contribution provides the text proposal of HARQ timing.

- Proposed Text

-------------------------------  Text Start  ---------------------------------------------------

15.2.x.2.2   HARQ timing

Notations:

- Nsf: Total number of subframes in a frame

- Nproc: HARQ processing latency

- NA−DL: Number of DL subframes assigned to the Advanced Air Interface HARQ operation, excluding relay, E-MBS, etc (in case of UL HARQ operation, a DL subframe which has no A-MAP region is not counted as NA−DL)

- NA−UL: Number of UL subframes assigned to the Advanced Air Interface HARQ operation, excluding relay, E-MBS, etc

- NULretx: Number of required UL subframes for UL synchronous HARQ operation

- nDL: Physical DL subframe index		n'DL: Logical DL subframe index (0≤n'DL<NA−DL)

- nUL: Physical UL subframe index		n'UL: Logical UL subframe index (0≤n'UL<NA−UL)

- nULretx: Physical UL subframe index where UL HARQ retransmission should be done

- m: Frame index where DL HARQ burst or UL Allocation A-MAP IE is transmitted

- mHARQFeedback: Frame index where UL HARQ feedback is transmitted

- mULtx & mULretx: Frame index where UL HARQ Initial & Retransmission occurs, respectively

- nA−DLStart & nA−DLLast: The index of Start & Last DL subframe among the NA−DL of DL subframes, respectively

- nA−ULStart & nA−ULLast: The index of Start & Last UL subframe among the NA−UL UL subframes, respectively

15.2.x.2.2.1   Downlink

15.2.x.2.2.1.1   FDD

Corresponding to DL HARQ burst transmission in a DL subframe nDL, the frame and the UL subframe for UL ACKCH is determined as follows:

nUL=(nDL+Nproc+1)modNsf

mHARQFeedback=m+nDL+Nproc+1Nsf

15.2.x.2.2.1.2   TDD

[NA−DL≥NA−UL]

Figure 15.2.x.2.2.1.2-1 Example of the relationship between physical frame and logical frame (NA−DL≥NA−UL)

- Assuming that DL HARQ burst is transmitted in the physical DL subframenDL, the corresponding logical DL subframe index n'DL is as follows.

n'DL=(nDL+NA−DL−1−min(nA−ULLast−Nproc−1,nA−DLLast))modNA−DL

- Each physical UL subframe nUL in which UL ACKCH is allocated is mapped to logical UL subframe n'UL in ascending order.

- Regarding logical DL subframe n'DL, the corresponding logical UL subframe n'UL for UL ACKCH is as follows.

n'UL=n'DLNA−DLNA−UL

- According to the physical-to-logical mapping for UL subframe (nULn'UL), physical UL subframe nUL is derived.

- The corresponding frame index mHARQFeedback is determined as follows:
mHARQFeedback=m+min(nDL,nA−ULLast−Nproc−1)nA−ULLast−Nproc−1

[NA−DL<NA−UL]

Figure 15.2.x.2.2.1.2-1 Example of the relationship between physical frame and logical frame (NA−DL<NA−UL)

- Corresponding to DL HARQ burst transmission in a DL subframe nDL, the UL ACKCH subframe nUL is determined as follows:

nUL=max[min(nUL(nDL),nA−ULLast+1)mod(nA−ULLast+1), nA−ULLast−NA−UL+1],
where nUL(nDL)=max(nDL+Nproc+1,nA−ULLast−NA−UL+1)

- The corresponding frame index mHARQFeedback is determined as follows.
mHARQFeedback=m+min(nA−DLLast+Nproc+1,nA−ULLast+1)nA−ULLast+1

15.2.x.2.2.2   Uplink

[UL synchronous retransmission timing]

Each UL HARQ retransmission is done in the nretxth UL subframe from the one where the initial UL HARQ burst was transmitted.

nULretx=nA−ULStart+(nUL−nA−ULStart+(NULretxmodNAUL))modNA−UL

mULretx=m+nUL−nA−ULStart+NULretxNAUL

Table 15.2.x.2.2.2-1 Required 16m UL subframes for UL synchronous HARQ operation (NULretx)

| System BW | System Configuration | UL MUXing | Frame Configuration | DL/UL Ratio | A-MAP interval (n=1) |  | A-MAP interval (n=2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | HARQ latency 2 subf | HARQ latency 3 subf | HARQ latency 2 subf | HARQ latency 3 subf |
| 5, 10, 20MHz | 16m only | N.A | DDDDDDUU | 6:2 | 2 | 2 | 2 | 3 |
|  | 16m only | N.A | DDDDDUUU | 5:3 | 3 | 3 | 3 | 4 |
|  | 16m only | N.A | DDDDUUUU | 4:4 | 4 | 4 | 4 | 5 |
|  | 16m only | N.A | DDDUUUUU | 3:5 | 5 | 7 | 5 | 7 |
|  | 16e/m co-ex | TDM | DDDDDUUU | 3:2:2:1 | 1 | 1 | 1 | 1 |
|  | 16e/m co-ex | TDM | DDDDDUUU | 3:2:1:2 | 2 | 3 | 2 | 3 |
|  | 16e/m co-ex | FDM | DDDDDUUU | 3:2:3:3 | 4 | 5 | 4 | 5 |
|  | 16e/m co-ex | TDM | DDDDDUUU | 2:3:2:1 | 1 | 1 | 1 | 2 |
|  | 16e/m co-ex | TDM | DDDDDUUU | 2:3:1:2 | 2 | 2 | 2 | 3 |
|  | 16e/m co-ex | FDM | DDDDDUUU | 2:3:3:3 | 3 | 4 | 3 | 4 |
|  | 16e/m co-ex | TDM | DDDDDUUU | 1:4:2:1 | 1 | 1 | 1 | 1 |
|  | 16e/m co-ex | TDM | DDDDDUUU | 1:4:1:2 | 2 | 2 | 2 | 3 |
|  | 16e/m co-ex | FDM | DDDDDUUU | 1:4:3:3 | 3 | 3 | 3 | 4 |
| 8.75MHz | 16m only | N.A | DDDDDUU | 5:2 | 2 | 3 | 2 | 3 |
|  | 16m only | N.A | DDDDUUU | 4:3 | 3 | 4 | 3 | 5 |
|  | 16m only | N.A | DDDUUUU | 3:4 | 4 | 6 | 4 | 6 |
| 7MHz | 16m only | N.A | DDDDUU | 4:2 | 2 | 4 | 3 | 4 |
|  | 16m only | N.A | DDDUUU | 3:3 | 3 | 5 | 4 | 6 |
|  | 16m only | N.A | DDUUUU | 2:4 | 6 | 8 | 7 | 9 |

15.2.x.2.2.2.1   FDD

Corresponding to UL allocation A-MAP in a DL subframe nDL, the frame and the UL subframe for UL HARQ burst transmission are determined as follows:

nUL=(nDL+Nproc+1)modNsf

mULtx=m+nDL+Nproc+1Nsf

15.2.x.2.2.2.2   TDD

[A-MAP relevance, NA−UL>NA−DL]

Figure 15.2.x.2.2.2.2-1 Example of the relationship between physical frame and logical frame (NA−UL>NA−DL)

- Each physical DL subframe nDL in which A-MAP region is allocated is mapped to logical DL subframe n'DL in ascending order.

- Assuming that UL HARQ burst is transmitted in the physical UL subframenUL, the corresponding logical UL subframe index n'UL is as follows.

n'UL=(nUL+NA−UL−min(max(nA−ULStart,nA−DLStart+Nproc+1),nA−ULLast))modNA−UL,

- Regarding logical UL subframe n'UL, the corresponding logical DL subframe n'DL for UL A-MAP allocation is as follows.

n'DL=n'ULNA−ULNA−DL

- According to the physical-to-logical mapping for UL subframe (nULn'UL), physical UL subframe nUL is derived.

- The corresponding frame index mULtx is determined as follows.
mULtx=m+min(nA−DLStart+Nproc+1,nA−ULLast)nA−ULLast

[A-MAP relevance, NA−UL≤NA−DL]

Each UL Allocation A-MAP IE in a DL subframe nDL indicates a UL resource allocation for UL HARQ burst transmission at the closest UL subframe nUL which is larger than nDL+ Nproc.

-------------------------------  Text End  -----------------------------------------------------