| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reply comments on recommended AWD Text Proposal #1 of idle mode AWD-DG comment on Power conservation |  |
| Date Submitted | 2009-05-03 |  |
| Source(s) | Giwon Park, Kiseon Ryu, Ronny Yongho Kim and Jin Sam Kwak LG Electronics | E-mail: gwpark@lge.com |
| Re: |  |  |
| Abstract | The contribution proposes the text of idle mode operation to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reply comments on recommended AWD Text Proposal #1 of idle mode AWD-DG comment on Power conservation

Giwon Park, Kiseon Ryu, Ronny Yongho Kim and Jin Sam Kwak

LG Electronics

# Introduction

Section 15.2.x.x.1.2 of Idle mode DG final document [1] does not explain the operation in case of De-registration_Request Code parameter = 0x03. This contribution is proposed to clearly define the procedure of idle mode initiation by ABS. The additional text on section 15.2.x.x.1.2 of idle mode DG final document [1] is indicated in blue with underline.

# Text proposal

-------------------------------  Text Start  ---------------------------------------------------

- 15. Advanced Air Interface

  - Medium access control

- 15.2.x.x.1.2 ABS initiated

Using ABS initiated idle mode entry, a serving ABS may signal for an AMS to begin idle mode by sending a DREG-CMD message with action code 0x05 in unsolicited manner. This unsolicited DREG-CMD may include REQ-Duration TLV. When an AMS receives an unsolicited DREG-CMD without REQ_Duration TLV, the AMS shall immediately start the idle mode initiation procedures. In this case of ABS-initiated idle mode, the serving ABS shall start T46 timer as well as Management_Resource_Holding_Timer at the same time. If the ABS does not receive the DREG-REQ message with the De-registration_Request_Code parameter = 0x02 from the AMS in response to the unsolicited DREG-CMD message with action code 0x05 within T46 timer expiry, the ABS shall retransmit the DREG-CMD message with action code 0x05 in unsolicited manner as long as DREG command retry count has not been exhausted. AMS shall enter idle mode after it sends DREG-REQ message with the De-registration_Request Code parameter = 0x02 in response to the unsolicited DREG-CMD message with action code 0x05. If the AMS has a pending UL data to transmit, it shall send DREG-REQ message with De-registration_Request Code parameter = 0x03 in response to the unsolicited DREG-CMD message with action code 0x05 by the ABS. These procedures are illustrated in Figure xx2.

-------------------------------  Text End  ---------------------------------------------------

# Reference

[1] “C80216m-09_0798” Apr. 2009.