| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reply comments on recommended AWD Text Proposal #3 of idle mode AWD-DG comment on Power conservation |  |
| Date Submitted | 2009-05-03 |  |
| Source(s) | Giwon Park, Ronny Yongho Kim, Kiseon Ryu and Jin Sam Kwak LG Electronics Yerang Hur Posdata | E-mail: gwpark@lge.com yehur@posdata-usa.com |
| Re: |  |  |
| Abstract | The contribution proposes the text of idle mode operation to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reply comments on recommended AWD Text Proposal #3 of idle mode AWD-DG comment on Power conservation

Giwon Park, Ronny Yongho Kim, Kiseon Ryu and Jin Sam Kwak

LG Electronics

Yerang Hur

Posdata

# Introduction

Primary purpose to use a timer for single paging group is to prevent frequent location updates at paging group boarder area. The concern with using PG_LU_TIMER in single paging group case is that AMS may miss paging massage during PG_LU_TIMER is running because it left a primary paging group. In order to overcome this problem it is possible to provide restriction that the value of PG_LU_TIMER shall be less than paging cycle * paging retry count. This contribution is proposed to define the PG_LU_TIMER for single paging group. The additional texts on section 15.2.x.x.4.1.1 of idle mode DG final document [1] are indicated in blue with underline.

# Text proposal

-------------------------------  Text Start  ---------------------------------------------------

- Advanced Air Interface

  - Medium access control

- 15.2.x.x.4.1.1 Paging group based update

ABSs and Idle Mode AMSs may belong to multiple paging groups. In case an AMS belongs to multiple paging groups, it starts Paging Group Location Update Timer (PG_LU_TIMER) when it leaves primary paging group. An AMS performs the paging group location update after PG_LU_TIMER and may inform its mobility (slow, medium, fast) to ABS. Based on the AMS mobility information, the ABS may assign new paging group(s) of different size(s) to AMS.

If the AMS returns to the primary paging group before the expiration of PG_LU_TIMER, it releases the timer and does not perform location update. An ABS may assign PG_LU_TIMER to an AMS during idle mode initiation. When only a primary paging group is assigned to the AMS, PG_LU_TIMER shall be set to a value less than paging cycle * paging retry count.

-------------------------------  Text End  ---------------------------------------------------

# Reference

[1] “C80216m-09_0798” Apr. 2009.