| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for IEEE 802.16m A-Preamble Design |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Pei-Kai Liao, Yu-Hao Chang MediaTek Inc | pk.liao@mediatek.com |
| Re: | Category: AWD reply comments / Area: Chapter 15.3.6 (DL-CTRL) “Reply comments on AWD 15.3.6.1 Advance Preamble” |  |
| Abstract | The contribution proposes the text of SFH contents to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for IEEE 802.16m A-Preamble Design

Pei-Kai Liao, Yu-Hao Chang

MediaTek Inc.

Introduction

Due to femtocell deployment, A-Preamble may suffer serious interference between femtocell and macrocell.

For PA-Preamble, frequency reuse-1 property results in full collision of two different code sequences from femtocell and macrocell BSs over the whole frequency band if BS type information is carried in PA-Preamble. This not only largely degrades detection performance of PA-Preamble but also deteriorates the performance of cell ID search seriously since PA-Preamble ID detection is the first step of cell ID search. To take advantage of frequency reuse-1 property of PA-Preamble, a better way to enhance its detection performance is to let every BS signal the same code sequence. With cell ID partition, AMS can also obtain BS type information after cell ID search in SA-Preamble. It is suggested to adopt the following text proposal 1 into AWD.

For SA-Preamble, AMS may not be able to hear SA-Preamble from macro-BS if its nearby femto-BSs transmit their SA-Preamble sequences over the same segment as that of the overlay macrocell. Since SA-Preamble is frequency reuse-3, it is possible to make use of this feature for minimized interference between femtocell and macrocell. The best way is to let femto-BS self-configure the segment for SA-Preamble transmission for minimized interference based on the segment information of the overlay macrocell. It is suggested to adopt the following text proposal 2 into AWD.

- Text Proposal 1

[Add the following into the TGm Amendment Working Document]

---------------------------------------------------------Start of the Text---------------------------------------------------------

# 15.3.x.1.1 Primary advanced preamble (PA-Preamble)

………………………

Table xxx─ PA-Preamble series

| Index | Carrier | ABS type | BW | Series to modulate |
| --- | --- | --- | --- | --- |
| 0 | Fully configured | ABS included in NBR_ADV | 5 MHz | 6DB4F3B16BCE59166C9CEF7C3C8CA5EDFC16A9D1DC01F2AE6AA08F |
| 1 |  |  | 10 MHz | 1799628F3B9F8F3B22C1BA19EAF94FEC4D37DEE97E027750D298AC |
| 2 |  |  | 20 MHz | 92161C7C19BB2FC0ADE5CEF3543AC1B6CE6BE1C8DCABDDD319EAF7 |
| 3 |  |  | reserved | 6DE116E665C395ADC70A89716908620868A60340BF35ED547F8281 |
| 4 |  |  | reserved | BCFDF60DFAD6B027E4C39DB20D783C9F467155179CBA31115E2D04 |
| 5 |  | ABS not included in NBR_ADV | 5 MHz reserved | 7EF1379553F9641EE6ECDBF5F144287E329606C616292A3C77F928 |
| 6 |  |  | 10 MHz reserved | 8A9CA262B8B3D37E3158A3B17BFA4C9FCFF4D396D2A93DE65A0E7C |
| 7 |  |  | 20 MHz reserved | DA8CE648727E4282780384AB53CEEBD1CBF79E0C5DA7BA85DD3749 |
| 8 |  |  | reserved | 3A65D1E6042E8B8AADC701E210B5B4B650B6AB31F7A918893FB04A |
| 9 |  |  | reserved | D46CF86FE51B56B2CAA84F26F6F204428C1BD23F3D888737A0851C |
| 10 | Partially configured | N/A | N/A | 640267A0C0DF11E475066F1610954B5AE55E189EA7E72EFD57240F |

Macro BS and Relay BS are included in NBR_ADV, whereas CSG Femto is not included in NBR_ADV.

-----------------------------------------------------------End of the Text--------------------------------------------------------

- Text Proposal 2

[Add the following into the TGm Amendment Working Document]

---------------------------------------------------------Start of the Text---------------------------------------------------------

# 15.3.x.1.2 Secondary advanced preamble (SA-Preamble)

...............

For the support of femtocell deployment, a femtocell BS may transmit its SA-Preamble sequence over the segment or subcarrier set different from that used by the overlay macrocell BS shall self-configure the segment or subcarrier set for SA-Preamble transmission based on the segment information of the overlay macrocell BS for minimized interference to macrocell if the femtocell BS is synchronized to macrocell BSs. The segment information of the overlay macrocell BS may be obtained by communications with macrocell BS through backbone network or active scanning of SA-Preamble transmitted by macrocell BS.

...............

-----------------------------------------------------------End of the Text--------------------------------------------------------