| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reply comments on recommended AWD Text Proposal of Intra-16m Handover AWD-DG comment on Mobility Management |  |
| Date Submitted | 2009-05-03 |  |
| Source(s) | Inuk Jung, Kiseon Ryu, Ronny Yongho Kim LG Electronics | E-mail: cooper@lge.com |
| Re: |  |  |
| Abstract | The contribution proposes the text of Intra-16m HO operation to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reply comments on recommended AWD Text Proposal of Intra-16m Handover AWD-DG comment on Mobility Management

Inuk Jung, Kiseon Ryu, Ronny Yongho Kim and Jin Sam Kwak

LG Electronics

# Introduction

This contribution is proposed to further clarify the Intra-16m HO procedures. The additional texts on section 15.2.5 are indicated in blue with underline and deleted texts are indicated in red with strikeout.

# Text proposal

-------------------------------  Text Start  ---------------------------------------------------

- 15. Advanced Air Interface

  - Medium access control

- 15.2.5 MAC HO procedures

[Suggested Remedy 1: Add the new text in section 15.2.4.3 as the following:]

[Editor’s Note: Since the ABS or AMS cannot not exactly tell when or why to omit CDMA based ranging during HO, the AMS shall perform a CDMA code based HO ranging based on the information provided in the AAI_HO-CMD message. Hence, if a dedicated ranging code is provided, the AMS shall perform CDMA code ranging based HO. If a UL grant for AAI_RNG-REQ is provided, the AMS shall send a AAI_RNG-REQ right away. A flag for each of such conditions are needed]

- 15.2.4.3 AAI_HO-CMD

8. DEDICATED_CDMA_RNG_FLAG

9. Dedicated UL Grant flag indicating assignment of unsolicited UL grant for AAI_RNG-REQ at T-ABS Optional dedicated CDMA ranging code/opportunity if CDMA_RNG_FLAG=1

[Suggested Remedy 2: Modify the text as following in section 15.2.5.2.5.2:]

[Editor’s Note: This text reflects the consideration of Remedy 1 with clarified text that describes the action of AMS based on the given ranging information given in the AAI_HO-CMD message.]

### 15.2.5.2.5.2 Network Re-entry Procedure

The network re-entry procedure with the target ABS may be optimized by target ABS possession of AMS information obtained from serving ABS over the backbone network.

At the Action Time specified in the AAI_HO-CMD message, the AMS performs network re-entry at the target ABS.

When the assignment of dedicated ranging code and the uplink resource for the AAI_RNG-REQ transmission is indicated by the HO Command message, the AMS may send both the dedicated ranging code and the AAI_RNG-REQ message to the target ABS respectively. If the target ABS receives the dedicated ranging code and the AAI_RNG-REQ message is decoded successfully, the target ABS may send the AAI_RNG-RSP message in response only to the AAI_RNG-REQ message. In this case, the AAI_RNG-RSP message may include the information of the ranging adjustment. If the AMS receives AAI_RNG-RSP message including only the information of the ranging adjustment from the target ABS, the AMS shall send AAI_RNG-REQ message to the target ABS after completion of CDMA-based ranging.

…

[Suggested Remedy 3: Modify the text as following in section 15.2.5.2.5.2:]

[Editor’s Note: To efficiently acquire new STID in case of uncoordinated HO, the AMS shall include its previously used STID in the AAI_RNG-REQ message during the network re-entry to a target ABS. This is to provide location privacy by not needing to send the MAC address over the air.]

### 15.2.5.2.5.2 Network Re-entry Procedure

…

If the AMS performs network reentry based on a CDMA ranging procedure using a dedicated CDMA HO ranging code, or a UL grant for sending AAI_RNG-REQ message without CDMA HO ranging, tThe AAI_RNG-REQ message shall include STID, CMAC_KEY_COUNT and a valid CMAC tuple, but not include AMS MAC address or previous serving ABSID. When ABS receives the AAI_RNG-REQ message, the ABS shall respond to the AAI_RNG-REQ message by transmitting AAI_RNG-RSP message with valid CMAC tuple. The AAI_RNG-RSP message shall be addressed to the AMS’s STID.

Otherwise, if the AMS performs network reentry based on a CDMA ranging procedure using a random CDMA HO ranging code, the AAI_RNG-REQ message shall include the former serving BSID, previously used STID. When ABS receives the AAI_RNG-REQ message, the ABS shall respond to the AAI_RNG-REQ message by transmitting AAI_RNG-RSP message with valid CMAC tuple. The AAI_RNG-RSP message shall include a Temporary STID for the AMS.

-------------------------------  Text End  ---------------------------------------------------

# Reference

[1] “C80216m-09_0913” Apr. 2009.