| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Modification on Section 15.3.5.3.4 for Amendment Working Document |  |
| Date Submitted | 2009-05-05 |  |
| Source(s) | Yu-Tao Hsieh, Pang-An Ting ITRI Yi Hsuan Intel | Voice: E-mail: ythsieh@itri.org.tw |
| Re: | IEEE 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document.” |  |
| Abstract | This contribution proposes the modified text for section 15.3.5.3.4 Formation of MLRU |  |
| Purpose | Discussion and adoption by TG m. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Modification on Section 15.3.5.3.4 for Amendment Working Document

Yu-Tao Hsieh, Pang-An Ting

ITRI

Yi Hsuan

Intel

Introduction

In the current AWD [1], section 15.3.5.3.4, there are some issues to be address. Firstly, the parameter LAMAP is not defined in the document. Secondly, it is better to use the notational symbol for the derivation of the equation. Regarding the equation revision, we think that the original formula revision derived in the comment #166 may be not necessary due to some misunderstanding of the mapping for the A-MAP information. The detailed proposed text is as follows.

Proposed text

Replace the AWD, section 15.3.5.3.4, pp. 42, line 17-27 as follows.

---------------------------------------------------------Start of the Text--------------------------------------------------------

1)  Renumber all tone pairs in the distributed LRUs in the A-MAP region in a time first manner. Assuming that Since each LRU has LSP 8 tone-pairs per symbol, the renumbered A-MAP tone-pairs are denoted by RMP[u], where u ranges from 0 to LAMAP · Nsym · LSP8 – 1. LAMAP is the number of LRU allocated to the A-MAP.

2)  A distributed tone-pair, SCLRUs,l,FPi [m], is mapped to RMP[u], where u = s · Nsym ·LSP 8 + m · Nsym + l1. SCLRUs,l,FPi[m] is the tone-pair index of the mth tone-pair in the llth OFDMA symbol in the sth distributed LRU of frequency partition ii as defined in Section <<15.3.5.3.23>>.

---------------------------------------------------------End of the Text---------------------------------------------------------

References

[1] IEEE 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document.”