| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Harmonised AWD Text on MAC PDU Formats |  |
| Date Submitted | 2009-05-05 |  |
| Source(s) | Anil Agiwal, Youngbin Chang, Sungjin Lee, Rakesh Taori, Jungje Son Samsung Electronics Hongyun Qu, Mary Chion ZTE Corporation Eunjong Lee, Doo-hyun Sung, Jeongki Kim and Kiseon Ryu LG Electronics Jaesun Cha, Kwangjae Lim, Sungcheol Chang, Chulsik Yoon ETRI Dongyao Wang, Kaibin Zhang, Gang Shen, Shan Jin Alcatel-Lucent Shanghai Bell Shiann-Tsong Sheu, Chih-Cheng Yang National Central University Whai-En Chen, Shih-Yuen Cheng National Ilan University Kanchei (Ken) Loa, Chun-Yen Hsu, Youn-Tai Lee, Tsung-Yu Tsai, Chiu-Wen Chen Institute for Information Industry | anilag@samsung.com qu.hongyun@zte.com.cn mchion@zteusa.com mehappy3@lge.com,dh.sung@lge.com, chegal@lge.com, ksryu@lge.com jscha@etri.re.kr Dongyao.wang@alcatel-sbell.com.cn, Kaibin.Zhang@alcatel-sbell.com.cn jasonyyy@gmail.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | “802.16m AWD”: IEEE 802.16m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. Target topic: MAC PDU formats |  |
| Abstract | This contribution proposes text for construction and transmission of MAC PDU section to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Harmonized Amendment Text on MAC PDU formats

Anil Agiwal, Youngbin Chang, Sungjin Lee, Rakesh Taori, Jungje Son

Samsung Electronics

Hongyun Qu, Mary Chion

ZTE Corporation

Eunjong Lee, Doo-hyun Sung, Jeongki Kim and Kiseon Ryu

LG Electronics

Jaesun Cha, Kwangjae Lim, Sungcheol Chang, Chulsik Yoon

ETRI

Dongyao Wang, Kaibin Zhang, Gang Shen, Shan Jin

Alcatel-Lucent Shanghai Bell

Shiann-Tsong Sheu, Chih-Cheng Yang                                                              
National Central University

Whai-En Chen, Shih-Yuen Cheng

National Ilan University

Kanchei (Ken) Loa, Chun-Yen Hsu, Youn-Tai Lee, Tsung-Yu Tsai, Chiu-Wen Chen

Institute for Information Industry

# Introduction

This contribution proposes the AWD text on MAC PDU formats based on SDD [2].

# Outline of AWD text on MAC PDU formats

15.2. x MAC PDU formats

##### 15.2.x.1 MAC header formats

15.2.x.1.1 Generic MAC header

15.2.x.1.2 Compact MAC header

##### 15.2.x.2 Extended header formats

15.2.x.1.1 Fragmentation and packing extended header

15.2.x.2.2 Fragmentation extended header

15.2.x.2.3 Multiplexing extended header

# Text Proposal for inclusion in the 802.16m amendment working document

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Insert the texts in subclause 15.2.x as follows:]

15.2.x MAC PDU formats

MAC PDUs shall be of the form illustrated in Figure x.1. Each PDU shall begin with a MAC header. The header may be followed one or more extended headers. The MAC PDU may also contain payload.

Multiple MAC SDUs and/or SDU fragments from different unicast connections belonging to the same AMS can be multiplexed into a single MAC PDU.

Figure x.1 MAC PDU formats

15.2.x.1 MAC header formats

There are two defined MAC header formats. The first is the generic MAC header that begins each DL and UL MAC PDUs containing either MAC management messages or CS data. The second is the compact MAC header that begins MAC PDUs of the connections using persistent allocation or group allocation.

15.2.x.1.1 Generic MAC header (GMH)

The GMH format is defined in Table x.1. Its fields are defined in Table x.2.

Table x.1 - GMH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Generic MAC Header () { | - |  |
| FlowID | 4 | Flow Identifier |
| EH | 1 | Extended header presence indicator |
| Length | 11 | Length of the payload |
| } | - |  |

Table x.2 - GMH fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| FlowID | 4 | This field indicates the service flow that is addressed. |
| EH | 1 | When set to ‘1’, this field indicates that an Extended Header is present following this GMH. |
| Length | 11 | This field indicates the length of the payload in bytes |

15.2.x.1.2 Compact MAC header (CMH)

The CMH format is defined in Table x.3. Its fields are defined in Table x.4.

Table x.3 – CMH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Compact MAC Header () { | - |  |
| EH | 1 | Extended header presence indicator |
| Length | 7 | Length of the payload |
| } | - |  |

Table x.4 - CMH fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| EH | 1 | When set to ‘1’, this field indicates that an Extended Header is present following this CMH. |
| Length | 7 | This field indicates the length of the payload in bytes |

##### 15.2.x.2 Extended header formats

The inclusion of Extended Header is indicated by EH bit in MAC Header. The Extended header format is defined in table x.5 and will be used unless specified otherwise.

Table x.5 – Extended Header format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Extended Header () { | - |  |
| LAST | 1 | Last Extended Header indication: 0 = one or more extended header follows the current extended header unless specified otherwise; 1 = this extended header is the last extended header unless specified otherwise |
| Type | TBD | Type of extended header |
| Body Contents | Variable | Type dependent contents |
| } | - |  |

The list of defined extended headers is given in Table x.6.

Table x.6 – Description of Extended header types

| Extended header type | Name | Description |
| --- | --- | --- |
| - | Fragmentation and packing extended header | See 15.2.x.2.1 |
| TBD | Fragmentation extended header | See 15.2.x.2.2 |
| TBD | Multiplexing extended header | See 15.2.x.2.3 |
| TBD | Reserved |  |

15.2.x.2.1 Fragmentation and packing extended header (FPEH)

The FPEH shall be used when MAC PDU contains payload from single transport connection. The FPEH exists after the last extended header (i.e. the extended header with ‘Last’ = ‘1’) if ‘EH’ in GMH set to ‘1’ or after the GMH if ‘EH’ in GMH set to ‘0’. The FPEH format is defined in table x.7 and its fields are defined in table x.8.

Table x.7 FPEH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| FPEH () { | - |  |
| SN | 10 | Payload sequence number |
| FC | 2 | Fragmentation control (see table |
| do { | - |  |
| End | 1 |  |
| if (End==0) { | - |  |
| Length | 11 | Length of SDU or SDU fragment |
| } | - |  |
| } while (!End) | - |  |
| Reserved | variable |  |
| } | - |  |

Table x.8 FPEH Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| SN | 10 | Payload Sequence Number |
| FC | 2 | Fragmentation Control bits (encoding shown in Table x.9) |
| End | 1 | Indication of more information 0 = Indicating another “Length” and “End” fields are followed 1 = Indicating no more “Length” and “End” fields are followed |
| Length | 11 | This field indicates the length of SDU or SDU fragment. If a payload consists of ‘N’ SDU/SDU fragments, N-1 ‘Length’ fields are present in FPEH. The length of the first SDU or SDU fragment in the payload is indicated by the ‘Length’ field in GMH. |
| Rsvd | Variable | Reserved bits are added at the end of FPEH for byte alignment |

Table x.9 Encoding of FC field

| FC | Meaning | Examples |
| --- | --- | --- |
| 00 | The first byte of data in the MPDU payload is the first byte of a MAC SDU. The last byte of data in the MPDU payload is the last byte of a MAC SDU. | One or Multiple Full SDUs packed in an MPDU |
| 01 | The first byte of data in the MPDU payload is the first byte of a MAC SDU. The last byte of data in the MPDU payload is not the last byte of a MAC SDU. | a) MPDU with only First fragment of an SDU b) MPDU with one or more unfragmented SDUs, followed by first fragment of subsequent SDU |
| 10 | The first byte of data in the MPDU payload is not the first byte of a MAC SDU. The last byte of data in the MPDU payload is the last byte of a MAC SDU. | a) MPDU with only Last fragment of an SDU b) MPDU with Last fragment of an SDU, followed by one or more unfragmented subsequent SDUs |
| 11 | The first byte of data in the MPDU payload is not the first byte of a MAC SDU. The last byte of data in the MPDU payload is not the last byte of a MAC SDU. | a) MPDU with only middle fragment of an SDU b) MPDU with Last fragment of an SDU, followed by zero or more unfragmented SDUs, followed by first fragment of a subsequent SDU |

15.2.x.2.2 Fragmentation extended header (FEH)

The FEH shall be used when MAC PDU contains payload from single management connection. The FEH format is defined in table x.10.

Table x.10 FEH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| FEH () { | - |  |
| LAST | 1 | Always set to ‘1’ |
| Type | TBD | FEH type |
| SN | 8 | Payload sequence number |
| FC | 2 | Fragmentation control (see Table x.9) |
| } | - |  |

15.2.x.2.3 Multiplexing extended header (MEH)

The format of MEH is defined in table x.11. It is used when SDUs/SDUs fragments from different connections are included in the same MPDU. Each MEH contains multiple Multiplexing Extended Header Blocks (MEHBs). The SDUs or SDU fragments belonging to the same connection are packed together and the information related to these SDUs or SDU fragments is included in one MEHB. The fields of MEH are defined in table x.12. The M bit in MEHB indicates if there is more MEHB followed.

Table x.11 Multiplexing Extended Header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MEH () { | - |  |
| LAST | 1 | 0 = Another extended header follows MEH 1 = another extended header does not follow MEH |
| Type | TBD | MEH type |
| do { | - |  |
| MEHB (M) | variable | Multiplexing extended header block |
| }while (!M) | - |  |
| Reserved | variable |  |
| } | - |  |

Table x.12 Multiplexing Extended Header Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| Last | 1 | Last Extended Header Indicator (always set to 1) |
| Type | TBD | Extended Header Type |
| MEHB | variable | Multiplexing extended header block (format shown in ) |

The format of MEHB is shown in table x.13, except the first MEHB. The format of the first MEHB is shown in figure x.14. The first MEHB doesn’t contain the Flow ID and the length for the first SDU/SDU fragment associated with the Flow ID. The Flow ID and the Length fields in the generic MAC header represent the flow ID and the length of the first SDU/SDU fragment associated with the first MEHB. The fields of MEHB are defined in table x.15.

Table x.13 MEHB format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MEHB () { | - |  |
| M | 1 | 0 = Another MEHB follows this MEHB 1 = This is the last MEHB in MEH |
| FlowID | 4 | Flow Identifier |
| FC | 2 | Fragmentation Control |
| SN | 10 |  |
| do { | - |  |
| Length | 11 | SDU or SDU fragment length |
| End | 1 |  |
| } while (!End) | - |  |
| } | - |  |

Table x.14 First MEHB format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| First MEHB () { | - |  |
| M | 1 | 0 = Another MEHB follows this MEHB 1 = This is the last MEHB in MEH |
| FC | 2 | Fragmentation Control |
| SN | 10 |  |
| do { | - |  |
| End | 1 |  |
| If ( End == 0) { | - |  |
| Length | 11 | SDU or SDU fragment length |
| } | - |  |
| } while (!End) | - |  |
| } | - |  |

Table x.15 MEHB Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| M | 1 | Indication of more MEHB 0 = no more MEHB follows the current MEHB 1 = one or more MEHB follows the current MEHB |
| Flow ID | 4 | Flow ID of the SDU/SDUs fragment identified in the MEHB |
| FC | 2 | Fragmentation Control bits (encoding shown in Table x.9) |
| SN | 10 | Sequence number for the payload identified in the MEHB |
| Length | 11 | Length of the correspondent SDU/SDU fragment |
| End | 1 | Indication of more information 0 = Indicating another “Length” and “End” fields are followed 1 = Indicating no more “Length” and “End” fields are followed |

# Reference

[1] IEEE 802.16m-09/0010r1a, “802.16m Amendment Working Document (AWD)”

[2] IEEE 802.16m-08/003r8, “802.16m System Description Document (SDD)”