| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Signaling Header SDD Proposal |  |
| Date Submitted | 2009-07-13 |  |
| Source(s) | Joey Chou, Shantidev Mohanty, Muthaiah Venkatachalam, Xiangying Yang, Jose Puthenkulam, Sassan Ahmadi, Jie Hui, Intel YihShen Chen, MediaTek Frank Ren, ITRI | E-mail: joey.chou@intel.com E-mail: yihshen.chen@mediatek.com E-mail: frank_ren@ITRI.ORG.TW |
| Re: | TGm SDD: |  |
| Abstract | This contribution proposes text for MAC signaling header. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Signaling Header SDD Proposal

Joey Chou, Muthaiah Venkatachalam, Sassan Ahmadi, Xiangying Yang, Jose Puthenkulam, Jie Hui, Shantidev Mohanty, Intel

YihShen Chen, MediaTek

Frank Ren, ITRI

- Introduction

Table 1 summarizes the MAC signaling header type I/II, and UL/DL extended header messages being defined in IEEE 802.16-2009.

|  | Signaling Messages | Sizes |  | Signaling Messages | Sizes |
| --- | --- | --- | --- | --- | --- |
| MAC Signaling Type I | BR incremental | 19 bits | MAC Signaling Type II | Combined DL average CINR | 5 bits |
|  | BR aggregate | 19 bits |  | MIMO mode condition feedback | 32 bits |
|  | PHY channel report | 16 bits |  | CINR feedback | 16 bits |
|  | BR with UL Tx power report | 19 bits |  | Close-loop MIMO feedback | 26 bits |
|  | BR and CINR report | 19 bits | DL EH | SDU SN | 8 bits |
|  | BR with UL sleep control | 19 bits |  | DL sleep control | 24 bits |
|  | SN Report | 11 bits |  | Feedback request | 24 bits |
|  | CQICH allocation request | 7 bits |  | SN request | 8 bits |
| MAC Signaling Type II | CQI & MIMO feedback | 9 bits |  | PDU SN (short) | 8 bits |
|  | DL average CINR | 5 bits |  | PDU SN (long) | 16 bits |
|  | MIMO Coeff feedback | 9 bits | UL EH | MIMO mode feedback | 8 bits |
|  | Preferred DL Channel | 8 bits |  | UL Tx Power report | 8 bits |
|  | UL Tx Power | 8 bits |  | Mini feedback | 16 bits |
|  | PHY Channel feedback | 18 bits |  | PDU SN (short) | 8 bits |
|  | AMC band indication bitmap | 32 bits |  | PDU SN (long) | 16 bits |
|  | Short-term precoding feedback | 32 bits |  | Persistent allocation error event | 8 bits |
|  | Multiple types of feedback | 32 bits |  | ertPS reassumption bitmap | 8 bits |
|  | Long-term precoding feedback | 14 bits |  |  |  |

Table 1: IEEE 802.16-2009 Signaling Messages

The signaling messages range from 5 bits to 32 bits in size, and are not piggybacked on the data payload. Therefore, the 11 bit length field in GMH becomes huge overhead if GMH is used to send signaling messages.

This contribution proposes type II MAC signaling headers to send signaling messages.

Type II signaling header

Figure 1 shows the Type II MAC signaling header to be used to support UD/DL signaling messages that are greater that 16 bits long and are not piggybacked on the data payload. It does not require dedicated flow ID. It contains signaling data size of 39 bits.

Sig = 1

SigTyp (1)

Flow ID = xxxx (4)

Signaling Data (8)

Signaling Data (7)

SigTyp

(1)

Signaling Data (8)

Signaling Data (8)

Signaling Data (8)

Figure 1: Type II Signaling Header

- Flow ID (4): not dedicated flow

- Sig (1): 1 – signaling header type II, 0 – two byte GMH

- SigTyp (4): support up to 16 signaling types

- Signaling Data: signaling body – 39 bits

Figure 2 shows examples of MPDU structure that compare two MPDUs that use the GMH, EH, and FPEH, as defined in the existing AWD, and the GMH, EH, and FPEH, as proposed in this contribution. It clearly indicates that the proposed GMH, EH, and FPEH retain all features of EH and FPEH without the need of EH bit in GMH. Every bit in GMH is very precious, since GMH is needed for every MPDU. Therefore, removing one unnecessary bit is a great improvement to the header efficiency.

Flow ID = xxxx (4)

LEN (8)

LEN (3)

Sig = 0

EH body (variable)

EH = 1

Length (8)

EH = 0

SN (7)

SN (3)

FC (2)

Reserved (3)

Length (3)

End = 0

Length (7)

Length (4)

EH Type (variable)

Flow ID = xxxx (4)

LEN (8)

LEN (3)

EH = 1

EH body (variable)

Last = 1

Length (8)

SN (8)

SN (2)

FC (2)

Reserved (3)

Length (3)

End = 0

Length (4)

Length (7)

EH Type (variable)

End = 0

End = 1

End = 1

Existing GMH, EH, FPEH in AWD

Proposed GMH, EH, FPEH

Figure 2: GMH, EH, FPEH examples

Figure 3 shows other examples of MPDU structure that compare two MPDUs that use the GMH and FPEH, as defined in the existing AWD, and the GMH and FPEH, as proposed in this contribution. It clearly indicates that the proposed GMH and FPEH have no impact to the FPEH design without the need of EH bit in GMH.

Flow ID = xxxx (4)

LEN (8)

LEN (3)

Sig = 0

Length (8)

EH = 0

SN (7)

SN (3)

FC (2)

Reserved (3)

Length (3)

End = 0

Length (7)

Length (4)

Flow ID = xxxx (4)

LEN (8)

LEN (3)

EH = 0

Length (8)

SN (8)

SN (2)

FC (2)

Reserved (3)

Length (3)

End = 0

Length (4)

Length (7)

End = 0

End = 1

End = 1

Existing GMH, FPEH in AWD

Proposed GMH, FPEH

Figure 3: GMH, FPEH examples

- Proposed Text

10.12.1.1 Generic MAC Header

Flow ID (4)

LEN (8)

LEN (3)

Sig (0)

Figure 1 Generic MAC header format

- FlowID (Flow Identifier): This field indicates the service flow that is addressed.. This field is 4bits long.

- EH (Extended Header Presence Indicator): When set to ‘1”, this field indicates that an Extended Header is present following this GMH.

- Sig (Signaling bit): ‘1’ – signaling header type II, ‘0’ – GMH; Sig bit should be ‘0’, since this is the GMH

- Length: Length of the payload. This field is 11bits long

### 10.12.2 Extended header

The EH format is shown in Figure 2 and will be used unless specified otherwise.

Body Content (variable length)

EH (1)

Type (Length TBD)

Figure 2 Extended Header Format

- EH: ‘1’ = extended header, ‘0’ = not extended header; EH bit should be ‘1’, since this is the extended header.

- Type: indicates the type of extended header. The length is TBD.

- Body Contents: Type-dependent contents.

#### 10.12.2.1 Fragmentation and packing extended header for transport connection

This fragmentation and packing extended header is shown in Figure 3. This header shall be used when MAC PDU contains single transport connection payload. The location of this header exists if ‘EH’ is set to ‘1’.

Length (8)

EH (1)

SN (7)

SN (3)

FC (2)

Length (3)

End (1)

Padding or Length (7)

.

.

.

Figure 3 Fragmentation and packing extended header format for transport connection

- EH (1bit): ‘0’ = FPEH, ‘1’ = extended header; EH bit should be ‘0’, since this is the FPEH

- SN (10 bits): Payload sequence number

- FC (2 bits): Fragmentation control bits definition is given in Error: Reference source not found.

-

- Length (11bits): This field represents the length of SDU/SDU fragment. If a payload consists of ‘N’ SDU/SDU fragments, N-1 length fields are present in the header

- End (1 bit): If this bit set to ‘0’, another ‘Length’ and ‘End’ field are followed. If this bit set to ‘1’, reserved bits may follow for byte alignment

- Padding: Padding bits for byte alignment.

10.12.2.4 Signaling Header

Signaling header shall be used to send signaling messages, and consists of type I and type II signaling headers.

10.12.2.4.1 Type II signaling header

Figure 27 shows the Type II signaling header to be used to support UD/DL signaling messages that are greater that 16 bits long and are not piggybacked on the data payload. It contains signaling data size of 39 bits.

Sig = 1

SigTyp (1)

Flow ID = xxxx (4)

Signaling Data (8)

Signaling Data (7)

SigTyp

(1)

Signaling Data (8)

Signaling Data (8)

Signaling Data (8)

Figure 27: Type II Signaling Header

- Flow ID (4): not dedicated flow

- Sig (1): ‘1’ – signaling header type II, ‘0’ –GMH; Sig bit should be ‘1’, since this is the signaling header

- SigTyp (4): support up to 16 signaling types

- Signaling Data: signaling body – 39 bits