| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Signaling Header AWD Proposal |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Joey Chou, Shantidev Mohanty, Muthaiah Venkatachalam, Xiangying Yang, Jose Puthenkulam, Sassan Ahmadi, Jie Hui, Intel YihShen Chen, MediaTek Frank Ren, ITRI | E-mail: joey.chou@intel.com E-mail: yihshen.chen@mediatek.com E-mail: frank_ren@ITRI.ORG.TW |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text for MAC signaling header. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Signaling Header AWD Proposal

Joey Chou, Muthaiah Venkatachalam, Sassan Ahmadi, Xiangying Yang, Jose Puthenkulam, Jie Hui, Shantidev Mohanty, Intel

YihShen Chen, MediaTek

Frank Ren, ITRI

- Introduction

Table 1 summarizes the MAC signaling header type I/II, and UL/DL extended header messages being defined in IEEE 802.16-2009.

|  | Signaling Messages | Sizes |  | Signaling Messages | Sizes |
| --- | --- | --- | --- | --- | --- |
| MAC Signaling Type I | BR incremental | 19 bits | MAC Signaling Type II | Combined DL average CINR | 5 bits |
|  | BR aggregate | 19 bits |  | MIMO mode condition feedback | 32 bits |
|  | PHY channel report | 16 bits |  | CINR feedback | 16 bits |
|  | BR with UL Tx power report | 19 bits |  | Close-loop MIMO feedback | 26 bits |
|  | BR and CINR report | 19 bits | DL EH | SDU SN | 8 bits |
|  | BR with UL sleep control | 19 bits |  | DL sleep control | 24 bits |
|  | SN Report | 11 bits |  | Feedback request | 24 bits |
|  | CQICH allocation request | 7 bits |  | SN request | 8 bits |
| MAC Signaling Type II | CQI & MIMO feedback | 9 bits |  | PDU SN (short) | 8 bits |
|  | DL average CINR | 5 bits |  | PDU SN (long) | 16 bits |
|  | MIMO Coeff feedback | 9 bits | UL EH | MIMO mode feedback | 8 bits |
|  | Preferred DL Channel | 8 bits |  | UL Tx Power report | 8 bits |
|  | UL Tx Power | 8 bits |  | Mini feedback | 16 bits |
|  | PHY Channel feedback | 18 bits |  | PDU SN (short) | 8 bits |
|  | AMC band indication bitmap | 32 bits |  | PDU SN (long) | 16 bits |
|  | Short-term precoding feedback | 32 bits |  | Persistent allocation error event | 8 bits |
|  | Multiple types of feedback | 32 bits |  | ertPS reassumption bitmap | 8 bits |
|  | Long-term precoding feedback | 14 bits |  |  |  |

Table 1: IEEE 802.16-2009 Signaling Messages

The signaling messages range from 5 bits to 32 bits in size, and are not piggybacked on the data payload. Therefore, the 11 bit length field in GMH becomes huge overhead if GMH is used to send signaling messages.

This contribution proposes type II MAC signaling headers to send signaling messages.

Type II signaling header

Figure 1 shows the Type II MAC signaling header to be used to support UD/DL signaling messages that are greater that 24 bits long and are not piggybacked on the data payload. It does not require dedicated flow ID. It contains signaling data size of 39 bits.

Sig = 1

SigTyp (1)

Flow ID = xxxx (4)

Signaling Data (8)

Signaling Data (7)

SigTyp

(1)

Signaling Data (8)

Signaling Data (8)

Signaling Data (8)

Figure 1: Type II Signaling Header

- Flow ID (4): not dedicated flow

- Sig (1): 1 – signaling header type II, 0 – two byte GMH

- SigTyp (4): support up to 16 signaling types

- Signaling Data: signaling body – 39 bits

Figure 2 shows examples of MPDU structure that compare two MPDUs that use the GMH, EH, and FPEH, as defined in the existing AWD, and the GMH, EH, and FPEH, as proposed in this contribution. It clearly indicates that the proposed GMH, EH, and FPEH retain all features of EH and FPEH without the need of EH bit in GMH. Every bit in GMH is very precious, since GMH is needed for every MPDU. Therefore, removing one unnecessary bit is a great improvement to the header efficiency.

Flow ID = xxxx (4)

LEN (8)

LEN (3)

Sig = 0

EH body (variable)

EH = 1

Length (8)

EH = 0

SN (7)

SN (3)

FC (2)

Reserved (3)

Length (3)

End = 0

Length (7)

Length (4)

EH Type (variable)

Flow ID = xxxx (4)

LEN (8)

LEN (3)

EH = 1

EH body (variable)

Last = 1

Length (8)

SN (8)

SN (2)

FC (2)

Reserved (3)

Length (3)

End = 0

Length (4)

Length (7)

EH Type (variable)

End = 0

End = 1

End = 1

Existing GMH, EH, FPEH in AWD

Proposed GMH, EH, FPEH

Figure 2: GMH, EH, FPEH examples

Figure 3 shows other examples of MPDU structure that compare two MPDUs that use the GMH and FPEH, as defined in the existing AWD, and the GMH and FPEH, as proposed in this contribution. It clearly indicates that the proposed GMH and FPEH have no impact to the FPEH design without the need of EH bit in GMH.

Flow ID = xxxx (4)

LEN (8)

LEN (3)

Sig = 0

Length (8)

EH = 0

SN (7)

SN (3)

FC (2)

Reserved (3)

Length (3)

End = 0

Length (7)

Length (4)

Flow ID = xxxx (4)

LEN (8)

LEN (3)

EH = 0

Length (8)

SN (8)

SN (2)

FC (2)

Reserved (3)

Length (3)

End = 0

Length (4)

Length (7)

End = 0

End = 1

End = 1

Existing GMH, FPEH in AWD

Proposed GMH, FPEH

Figure 3: GMH, FPEH examples

- Proposed Text

15.2.3.1.1 Generic MAC header (GMH)

The GMH format is defined in Table x.1. Its fields are defined in Table 1—.

Table 1 - GMH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Generic MAC Header () { | - |  |
| FlowID | 4 | Flow Identifier |
| Sig | 1 | Signaling header indicator |
| Length | 11 | Length of the payload |
| } | - |  |

Table 2 - GMH fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| FlowID | 4 | This field indicates the service flow that is addressed. |
| Sig | 1 | 1 = signaling header 0 = GMH Sig bit should be ‘0’, since this is GMH |
| Length | 11 | This field indicates the length of the payload in bytes |

##### 15.2.3.2 Extended header formats

The inclusion of Extended Header is indicated by EH bit. The Extended header format is defined in table 5 and will be used unless specified otherwise.

Table 5 – Extended Header format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Extended Header () { | - |  |
| EH | 1 | Extended Header indication: 0 = not extended header; 1 = extended header EH bit should be ‘1’, since this is the extended header |
| Type | TBD | Type of extended header |
| Body Contents | Variable | Type dependent contents |
| } | - |  |

15.2.3.2.1 Fragmentation and packing extended header (FPEH)

The FPEH shall be used when MAC PDU contains payload from single transport connection. The FPEH exists if ‘EH’  is set to ‘0’. The FPEH format is defined in table 7 and its fields are defined in table 8.

Table 7 FPEH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| FPEH () { | - |  |
| EH | 1 | Extended header indicator |
| SN | 10 | Payload sequence number |
| FC | 2 | Fragmentation control (see table |
| do { | - |  |
|  | - |  |
| Length | 11 | Length of SDU or SDU fragment |
| End | 1 | Indicatoin of the last fragment |
| } | - |  |
| } while (!End) | - |  |
| Reserved | variable |  |
| } | - |  |

Table 8 FPEH Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| EH | 1 | Extended header indicator should be ‘0’, since this is FPEH 0 = not extended header 1 = extended header |
| SN | 10 | Payload Sequence Number |
| FC | 2 | Fragmentation Control bits (encoding shown in Table x.9) |
| Length | 11 | This field indicates the length of SDU or SDU fragment. If a payload consists of ‘N’ SDU/SDU fragments, N-1 ‘Length’ fields are present in FPEH. The length of the first SDU or SDU fragment in the payload is indicated by the ‘Length’ field in GMH. |
| End | 1 | Indication of the last fragment 0 = Indicating another “Length” and “End” fields are followed 1 = Indicating no more “Length” and “End” fields are followed |
| Rsvd | Variable | Reserved bits are added at the end of FPEH for byte alignment |

Figure 27 and 28 show the examples of MPDU with and without the extended header

Flow ID = xxxx (4)

LEN (8)

LEN (3)

Sig = 0

EH body (variable)

EH = 1

Length (8)

EH = 0

SN (7)

SN (3)

FC (2)

Reserved (3)

Length (3)

End = 0

Length (7)

Length (4)

EH Type (variable)

End = 1

Figure 26: MPDU example with GMH, EH, and FPEH

Flow ID = xxxx (4)

LEN (8)

LEN (3)

Sig = 0

Length (8)

EH = 0

SN (7)

SN (3)

FC (2)

Reserved (3)

Length (3)

End = 0

Length (7)

Length (4)

End = 1

Figure 27: MPDU example with GMH and FPEH

15.2.3.2.4  Signaling Header

Signaling header shall be used to send signaling messages, and consists of type I and type II signaling headers.

15.2.3.2.4.2 Type II signaling header

Figure 27 shows the Type II signaling header to be used to support UD/DL signaling messages that are greater that 16 bits long and are not piggybacked on the data payload. It contains signaling data size of 39 bits.

Table 10 Type II SH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Type II SH () { | - |  |
| Flow ID | 4 | Flow identifier |
| Sig | 1 | Signaling header indicator |
| Sig Type | 4 | Signaling type |
| Signaling Data | 39 | Fragmentation control (see Table x.9) |
| } | - |  |

Table 10 Type II SH Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| Flow ID | 4 | This field indicates the service flow that has been addressed |
| Sig | 1 | Signaling header indicator; this bit should always be ‘1’, since this is the signaling header 0 = Normal GMH 1 = signaling header |
| Sig Type | 4 | Support up to 16 signaling types |
| Signaling data | 39 | Signaling data |