| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC header for management message |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Joey Chou Shantidev Mohanty, Muthaiah Venkatachalam, Sassan Ahmadi, Xiangying Yang, Jose Puthenkulam, Jie Hui, Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text for MAC header for MAC management messages. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC header for management message

Joey Chou, Muthaiah Venkatachalam, Sassan Ahmadi, Xiangying Yang,

Jose Puthenkulam, Jie Hui, Shantidev Mohanty

Intel

- Introduction

The existing FEH is assumed to be sent in the data flow ID that will not be able to meet the latency requirement of certain MAC management messages (see SDD text below). Therefore, MAC management messages should be sent in two flow IDs to meet the delay-tolerant and delay-intolerant requirements.

To meet the latency requirements for aspects of network entry, handover, state transition, 802.16m supports fast and reliable transmission of MAC management messages.

Here are some proposed changes:

- Add EH bit to be consistent with the extended format

- Last bit is not needed, since there is always one MAC management fragment in a MPDU. No packing is supported.

- Type field, if MAC management messages are sent in dedicated flows

- Add EC bit to support encryption

- Add padding for byte alignment

- Add field definitions

- Proposed Text

15.2.3.2.2 MAC Management extended header (MMEH)

The MMEH shall be used when MAC PDU contains payload from a single MAC management message or fragment. MAC management messages that are delay-intolerant and delay-tolerant shall be sent in two flow IDs respectively. The MMEH format is defined in table 10.

Table 10 MMEH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| FEH () { | - |  |
| EH | 1 | Exteneded header indicator |
| EC | 1 | Encryption Control indicator |
| SN | 8 | Payload sequence number |
| FC | 2 | Fragmentation control (see Table x.9) |
| Reserved | 4 | Reserved |
| } | - |  |

Table 10 MMEH Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| EH | 1 | Extended header indicator should be ‘1’, since this is the extended header 0 = not extended header 1 = extended header |
| EC | 1 | Encryption control indicator 0 = encryption is disabled 1 = encryption is enabled |
| SN | 8 | MAC management fragment sequence number |
| FC | 2 | Fragmentation Control bits |
| Reserved | 4 | Reserved |