| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Signaling Header based on dedicated flow ID |  |
| Date Submitted | 2009-07-04 |  |
| Source(s) | Joey Chou Shantidev Mohanty, Muthaiah Venkatachalam, Sassan Ahmadi, Xiangying Yang, Jose Puthenkulam, Jie Hui, Intel | E-mail: joey.chou@intel.com |
| Re: | TGm SDD: |  |
| Abstract | This contribution proposes text for MAC signaling header. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Signaling Header Based on Dedicated Flow ID

Joey Chou, Muthaiah Venkatachalam, Sassan Ahmadi, Xiangying Yang,

Jose Puthenkulam, Jie Hui, Shantidev Mohanty

Intel

- Introduction

Table 1 summarizes the MAC signaling header type I/II, and UL/DL extended header messages being defined in IEEE 802.16-2009.

|  | Signaling Messages | Sizes |  | Signaling Messages | Sizes |
| --- | --- | --- | --- | --- | --- |
| MAC Signaling Type I | BR incremental | 19 bits | MAC Signaling Type II | Combined DL average CINR | 5 bits |
|  | BR aggregate | 19 bits |  | MIMO mode condition feedback | 32 bits |
|  | PHY channel report | 16 bits |  | CINR feedback | 16 bits |
|  | BR with UL Tx power report | 19 bits |  | Close-loop MIMO feedback | 26 bits |
|  | BR and CINR report | 19 bits | DL EH | SDU SN | 8 bits |
|  | BR with UL sleep control | 19 bits |  | DL sleep control | 24 bits |
|  | SN Report | 11 bits |  | Feedback request | 24 bits |
|  | CQICH allocation request | 7 bits |  | SN request | 8 bits |
| MAC Signaling Type II | CQI & MIMO feedback | 9 bits |  | PDU SN (short) | 8 bits |
|  | DL average CINR | 5 bits |  | PDU SN (long) | 16 bits |
|  | MIMO Coeff feedback | 9 bits | UL EH | MIMO mode feedback | 8 bits |
|  | Preferred DL Channel | 8 bits |  | UL Tx Power report | 8 bits |
|  | UL Tx Power | 8 bits |  | Mini feedback | 16 bits |
|  | PHY Channel feedback | 18 bits |  | PDU SN (short) | 8 bits |
|  | AMC band indication bitmap | 32 bits |  | PDU SN (long) | 16 bits |
|  | Short-term precoding feedback | 32 bits |  | Persistent allocation error event | 8 bits |
|  | Multiple types of feedback | 32 bits |  | ertPS reassumption bitmap | 8 bits |
|  | Long-term precoding feedback | 14 bits |  |  |  |

Table 1: IEEE 802.16-2009 Signaling Messages

The signaling messages range from 5 bits to 32 bits in size, and are not piggybacked on the data payload. Therefore, the 11 bit length field in GMH becomes huge overhead if GMH is used to send signaling messages.

This contribution proposes type I MAC signaling headers to send signaling messages.

Type I signaling header

Figure 1 shows the Type I MAC signaling header to be used to support UD/DL signaling messages that are up to 16 bits long and are not piggybacked on the data payload. It can be identified by the dedicated flow ID, (e.g. 0011 as shown in Figure 1). It contains fixed size signaling data (i.e. 32 bits); therefore length field can be eliminated for better efficiency.

Flow ID = 0011 (4)

Signaling Data (8)

Signaling Data (8)

Signaling Type (4)

1

2

CRC (8) *

5

* A byte CRC is included if the signaling message is not protected by HARQ

Signaling Data (8)

Signaling Data (8)

3

4

Figure 1: Type I Signaling Header

- Flow ID (4): dedicated flow, e.g. 0011

- Signaling type (4): support up to 16 signaling types

- Signaling data (32): signaling body, 32 bits

- CRC (8): protect signaling data integrity if HARQ is not used

Table 1 compares the header efficiency between current design in SDD and the proposed signaling headers.

| Signaling data size | MPDU size based on current design in SDD | MPDU size based on proposed signaling header |
| --- | --- | --- |
| 2 bytes signaling data | GMH(2 bytes) + EH (1 byte) + EH body(2 bytes) = 5 bytes | Flow ID (4 bits) + sig type (4 bits) + signaling data (16 bites) = 3 bytes |

Table 2: Header Efficiency Comparison

- Proposed Text

10.12.2.4 Signaling Header

Signaling header shall be used to send signaling messages, and consists of type I and type II signaling headers.

10.12.2.4.1 Type I signaling header

Figure 26 shows the Type I signaling header to be used to support UD/DL signaling messages that are up to 16 bits long and are not piggybacked on the data payload. It can be identified by the dedicated flow ID, (e.g. 0011 as shown in Figure 26). It contains fixed size signaling data (i.e. 32 bits); therefore length field can be eliminated for better efficiency.

Flow ID = 0011 (4)

Signaling Data (8)

Signaling Data (8)

Signaling Type (4)

1

2

CRC (8) *

5

* A byte CRC is included if the signaling message is not protected by HARQ

Signaling Data (8)

Signaling Data (8)

3

4

Figure 26: Type I Signaling Header

- Flow ID (4): dedicated flow, e.g. 0011

- Signaling type (4): support up to 16 signaling types

- Signaling data (32): signaling body, 32 bits

- CRC (8): protect signaling data integrity if HARQ is not used