| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Change to AWD Chapter 15.2.5.5, 15.2.5.6, and 15.2.5.7 |  |
| Date Submitted | 2009-07-08 |  |
| Source(s) | Jing Zhu Intel Corporation I-Kang Fu MediaTek Inc. | jing.z.zhu@intel.com IK.Fu@mediatek.com |
| Re: | IEEE 802.16m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content” – Comments on AWD 15.2.4 MAC Management Message |  |
| Abstract | Proposed text for multi-radio coexistence in the draft P802.16m amendment |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Change to Chapter 15.2.5.5, 15.2.5.6, and 15.2.5.7

Jing Zhu

Intel Corporation

I-Kang Fu

MediaTek Inc.

- Introduction

The contribution proposes the text changes to the latest AWD text [1]

- References

[1] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

[2] IEEE C802.16m-09/0971r1, “Proposed Text for the IEEE 802.16m Amendment on Multi-Radio Coexistence: Management Messages and TLVs”

Change the texts in 15.2.5.5, 15.2.5.6, and 15.2.5.7 as follows:

-----------------------------------------------------------Start of the Text---------------------------------------------------------Page 20, line 16:

15.2.5.5 CLC-REQ (Co-Located Coexistence Request) message

Page 20, Table 16:

Table 16— CLC-REQ message parameters

| Name | Size (bit) |
| --- | --- |
| Management Message Type |  |
| Request Action | 8 |
| Request Action Parameters CLC-INFO parameters | variable |

Page 20, line 39~59:

Parameters shall be as follows:

Request Action

Bit #i of the field set to “0” indicates that AMS requests to terminate the existing CLC class with CLC ID = i, or the CLC class with CLC ID = i does not exist

Bit #i of the field set to “1” indicates that AMS requests to activate the CLC class with CLC ID = i if it does not exist. For existing CLC ID, MS may keep existing configuration, request to re-configure or replace existing CLC class.

The request action parameters may be included as follows:

CLC Information compound

These parameters may be present more than once if AMS wants to include multiple CLC information fields (Table 18 –).

CLC-INFO parameters

These parameters may be present more than once if AMS wants to include multiple CLC information fields (Table 18—).

Page 20, line 62:

15.2.5.6 CLC-RSP (Co-Located Coexistence Request Response) message

Page 21, Table 17:

Table 17—CLC-RSP message parameters

| Name | Size (bit) |
| --- | --- |
| Syntax | Notes |
| Confirmed Action | 8 |
| Parameter set for each Type II or III CLC class that ABS wants to configure its starting time differently from what is recommended by AMS |  |
| CLC Start Time parameters | 8n |
| CLC ID |  |
| Start Superframe Number |  |
| Start Frame Index | shall be set to 0 for Type III CLC class |

Page 21, Line 24~48:

Parameters shall be as follows:

Confirmed Action

Bit #i of the field set to “0” indicates that ABS confirms the termination of the existing CLC class with CLC ID = i, or the CLC class with CLC ID = i does not exist

Bit #i of the field set to “1” indicates that ABS confirms the activation of the CLC class with CLC ID = i. For existing CLC ID, the ABS confirms the existing configuration, the reconfiguration or the replacement if it is requested by the MS.

CLC Start Time parameters

This field is included for each Type II or III CLC class that ABS configures its starting time differently from what is recommended by AMS, and may be present for more than once. Its parameters are listed in Table 22. If the number of configured CLC classes is n, the size of this field is calculated as n x 8.

CLC ID

An integer number (0~7) to uniquely identify a CLC class

Start Time information

Start Superframe Number

The 3 LSB of the superframe number of CLC start time

Start Frame Index

The frame index of CLC start time

Page 21, Line 49~54:

15.2.5.7 CLC_INFO CLC-INFO

The CLC_INFO CLC-INFO parameters are applicable to REG-REQ, REG-RSP, RNG-REQ, RNG-RSP, CLC-REQ, SBC-REQ, SBC-RSP messages. The following sets of parameters will may be included into the CLC_INFO CLC-INFO parameters encodings.

Page 22, Table 18:

Table 18—Parameters to be applied into parameters encoding for CLC-INFO parameters information fields

| Parameters Name | Note Size (bit) | Scope |
| --- | --- | --- |
| Information Type | 8 |  |
| CLC Limit | If (Information Type = 0) variable | REG-RSP |
| CLC Request | If (Information Type = 1) variable | RNG-REQ, SBC-REQ, CLC-REQ |
| CLC Response | If (Information Type = 2) variable | RNG-RSP, SBC-RSP |
| CLC Report | If (Information Type = 3) variable | CLC-REQ |

Page 22, Line 12:

Parameters shall be as follows:

Information Type

This field indicates which of the following fields are included.

CLC Limit

This field is present if (Information Type == 0), and may be used in REG-RSP message. Table 19 lists all the parameters that are applied.

CLC Request

This field is present if (Information Type == 1), and may be used in RNG-REQ, SBC-REQ, or CLC-REQ messages. Table 20 lists all the parameters that are applied. It may be present more than once if AMS wants to request for multiple CLC classes.

CLC Response

This field is present if (Information Type == 2), and may be used in RNG-RSP or SBC-RSP messages. Table 21 lists all the parameters that are applied.

CLC Report

This field is present if (Information Type == 3),, and may be used in CLC-REQ message. Table 23 lists all the parameters that are applied.

Page 22, Table 19:

Table 19—Parameters to be applied into CLC Limit field

| Name | Size (bit) |
| --- | --- |
| the bit field set to 1 indicates the CLC limits are provided for Type I CLC class Type I Indicator | 1 |
| the bit field set to 1 indicates the CLC limits are provided for Type II CLC class Type II Indicator | 1 |
| maximum number of active CLC classes of the type Active Class Limit | 3 |
| maximum CLC active ratio Active Ratio Limit | 6 |
| maximum CLC active interval in unit of subframes Active Interval Limit | 5 |

Page 22, Line 30:

Parameters in the CLC Limit field shall be as follows:

Type I Indicator

The bit field set to 0 indicates the CLC limits are provided for Type I CLC class

Type II Indicator

The bit field set to 1 indicates the CLC limits are provided for Type II CLC class

Active Class Limit

The field indicates the maximum number of active CLC classes of the type

Active Ratio Limit

The field indicates the maximum CLC active ratio. If the Active Radio Limit is x, the maximum CLC active radio shall be x/100.

Active Interval Limit

The field indicates the maximum CLC active interval. If the Active Interval Limit is x, and a frame consists of m subframes, the maximum CLC active interval shall be xm subframes.

Page 22, Table 22:

Table 20 – Parameters to be applied into CLC Request field

| Parameter Name | Note Size (bit) |  |
| --- | --- | --- |
| Parameter set for each requested Type I, II, or III CLC class |  |  |
| CLC ID CLC Start Time parameters | 8 |  |
| Scheduling Impact | 2 |  |
| Start Superframe Number |  |  |
| Start Frame Index |  |  |
| Flag | 2 |  |
| Reserved | 4 |  |
| CLC active interval of Type I CLC class | If (Flag = 0b00) 8 |  |
| CLC active cycle of Type I CLC class | If (Flag = 0b00)21 |  |
| Start Subframe Index | If (Flag = 0b00)3 |  |
| CLC Active Interval of Type II CLC class with subtype 1 | If (Flag = 0b01) 8 |  |
| CLC Active Cycle of Type II CLC class with subtype 1 | If (Flag = 0b01) 8 |  |
| Extended Bitmap Indicator | If (Flag = 0b10) 1 |  |
| CLC Active Bitmap | If (Extended Bitmap Indicator == 0) 7 |  |
| Length of Extended CLC Active Bitmap (k) CLC Active Cycle of Type II CLC class with subtype 3 | If (Extended Bitmap Indicator == 1) 2 |  |
| Extended CLC Active Bitmap | m: number of subrames per framevariable |  |
| CLC Active Interval of Type III CLC class } | If (Flag = 0b11) 8 |  |

Page 23, Line 13~65:

Parameters in the CLC Request field shall be as follows:

CLC Start Time parameters

Its parameters is listed in Table 22

Scheduling Impact

0b00 (default) = both DL and UL allocations are prohibited in CLC Active Interval

0b01 = only DL allocations are prohibited in CLC Active Interval

0b10 = only UL allocations are prohibited in CLC Active Interval

0b11 = reserved

Flag

0b00: Type I CLC class;

0b01: Type II CLC class subtype 1;

0b10: Type II CLC class subtype 2 or 3;

0b11: Type III CLC class

Scheduling Impact

0b00 (default) = both DL and UL allocations are prohibited in CLC Active Interval

0b01 = only DL allocations are prohibited in CLC Active Interval

0b10 = only UL allocations are prohibited in CLC Active Interval

0b11 = reserved

CLC active interval of Type I CLC class

This field is present if (Flag == 0b00), and indicates The the number of subframes of the CLC Active Interval for Type I CLC class

CLC active cycle of Type I CLC class

This field is present if (Flag == 0b00), and indicates The the number of microseconds of the CLC Active Cycle for Type I CLC class

Start Subframe Index

This field is present if (Flag == 0b00), and indicates the subframe index of CLC start time

CLC active interval of Type II CLC class with subtype 1

This field is present if (Flag == 0b01), and indicates The the number of subframes of the CLC Active Interval for Type II CLC class

CLC active cycle of Type II CLC class with subtype 1

This field is present if (Flag == 0b01), and indicates The the number of frames of the CLC Active Cycle for Type II CLC class

Extended CLC Active Bitmap Indicator

This field is present if (Flag == 0b10), and indicates Indicate whether the Extended CLC Active Bitmap field is used

CLC Active Bitmap

This field is present if (Flag == 0b10) and (Extended Bitmap Indicator == 0). Setting a bit of the field to “1” indicates the corresponding subframe in each frame is in CLC active interval

CLC Active Cycle of Type II CLC class with subtype 3

This field is present if (Flag == 0b10) and (Extended Bitmap Indicator == 1), and indicates the length of the CLC active cycle in the unit of frame. If the value of this field is x, the length of Extended CLC Active Bitmap is calculated as x+2. This field shall be set to one of the following values: 0, 1, and 2.

Extended CLC Active Bitmap

This field is present if (Flag == 0b10) and (Extended Bitmap Indicator == 1). Setting a bit of the field to “1” indicates the corresponding subframe in each CLC active cycle is in CLC active interval. If the CLC active cycle is x frames, and a frame consists of m subframes, the length of this field shall be xm.

CLC Active Interval of Type III CLC class

This field is present if (Flag == 0b11), and indicates the The number of superframes of the CLC Active Interval for Type III CLC class

Page 24, Table 21:

Table 21—Parameters to be applied into CLC Response field

| Parameters Name | Size (bit) |
| --- | --- |
| CLC ID Confirmed Action | 8 |
| Start Superframe Number CLC Start Time parameters | 8n |
| Start Frame Index |  |

Parameters shall be as follows:

Confirmed Action

Bit #i of the field set to “0” indicates that ABS confirms the termination of the existing CLC class with CLC ID = i, or the CLC class with CLC ID = i does not exist.

Bit #i of the field set to “1” indicates that ABS confirms the activation of the CLC class with CLC ID = i. For existing CLC ID, the ABS confirms the existing configuration, the reconfiguration or the replacement if it is requested by the MS.

CLC Start Time parameters

This field is included for each Type II or III CLC class that ABS configures its starting time differently from what is recommended by AMS, and its parameters is listed in Table 22. If the number of configured CLC classes is n, the size of this field is calculated as n x 8.

Page 24, Table 22:

Table 22— Parameters to be applied into of CLC Report

Parameters

CLC Report Information Elements

Table 22 CLC Start Time parameters

| Name | Size (bit) |
| --- | --- |
| CLC ID | 3 |
| Start Superframe Number | 3 |
| Start Frame Index | 2 |

Parameters shall be as follows:

CLC ID

An integer number (0~7) to uniquely identify a CLC class

Start Superframe Number

The 3 LSB of the superframe number of CLC start time

Start Frame Index

The frame index of CLC start time

Page 24, Table 23:

Table 23 – Parameters to be applied into CLC Report field Information Element

| Parameters Name | Note Size (bit) |
| --- | --- |
| CLC Report Type | Table 24—8 |
| CLC Report Parameter Interference Level | Table 24—8 |

Page 24, Table 24:

Table 24: Parameters to be applied into CLC Report Information

CLC Report Parameter

Interference Level

Page 24, Line 60-65:

Parameters shall be as follows:

Report Type

This field indicates which of the following fields are included.

Interference Level

This field is present if (Report Type == 0), and indicates the average power level over one OFDMA symbol in unit of 1 dBm with the range from -127dBm to 127dBm received at the AMS from its co-located non 802.16 radios when they are active

-----------------------------------------------------------End of the Text---------------------------------------------------------