| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes/Refinements to the Sections 11.3 and 11.4 of IEEE 802.16m SDD |  |
| Date Submitted | 2009-07-03 |  |
| Source(s) | Jaeweon Cho Samsung Electronics Sassan Ahmadi and Roshni M Srinivasan Intel Corporation Jin Sam Kwak LG Electronics IK Fu MediaTek Michiharu Nakamura Keiichi Nakatsugawa Yanling Lu Zhou Hua Luciano Sarperi Rajni Agrawal Kevin Power Jie Zhang Luciano Sarperi Masato Okuda Fujitsu Limited Tetsu Ikeda Andreas Maeder Yoshikazu Watanabe Linghang Fan NEC Corporation Kenji Saito UQ Communications Mitsuo Nohara Satoshi Imata KDDI R&D Laboratories Hajime Kanzaki Hitachi Ltd | jaeweon.cho@samsung.com sassan.ahmadi@intel.com samji@lge.com ik.fu@mediatek.com michi@labs.fujitsu.com t-ikeda@ap.jp.nec.com kenji@uqc.jp mi-nohara@kddilabs.jp hajime.kanzaki.ad@hitachi.com |
| Re: | Change Request on 802.16m SDD, IEEE 802.16m-08/003r9a |  |
| Abstract | The contribution proposes text changes to Sections 11.3 and 11.4 of IEEE 802.16m SDD |  |
| Purpose | To be discussed and adopted by TGm for the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Introduction

This contribution proposes text changes to Sections 11.3 and 11.4 of the IEEE 802.16m SDD [1] where the system description requires additional detail. The proposed text is consistent with the 802.16m Amendment Working Document [2]. The refined SDD document will be a necessary supplement in the submission of IEEE 802.16m technology description template to ITU-R/WP 5D later this year.

For technical changes, AWD [2] text that has been added into the SDD has been specifically identified by section number. Proposed text has been underlined in blue and deleted text has been struck through in red. Existing SDD text is shown in black.

- References

[1] P802.16m System Description Document, 802.16m-08/003r9a, June 2009.

[2] P802.16m Amendment Working document, 802.16m-09/0010r2, June 2009.

# Physical Layer

## Duplex Mmodes

IEEE 802.16m supports TDD and FDD duplex modes, including H-FDD AMS operation, in accordance with the IEEE 802.16m Ssystem Rrequirements Ddocument [8]. Unless otherwise specified, the frame structure attributes and baseband processing are common for all duplex modes.

## Downlink and Uplink Multiple Access Schemes

IEEE 802.16m employs uses OFDMA as the multiple access scheme in the downlink and uplink.

## OFDMA Parameters

The OFDMA parameters for the IEEE 802.16m are specified as follows: in Table 4.

| Nominal cChannel bBandwidth (MHz) |  |  | 5 | 7 | 8.75 | 10 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Over-Ssampling fFactor |  |  | 28/25 | 8/7 | 8/7 | 28/25 | 28/25 |
| Sampling fFrequency (MHz) |  |  | 5.6 | 8 | 10 | 11.2 | 22.4 |
| FFT sSize |  |  | 512 | 1024 | 1024 | 1024 | 2048 |
| Sub-cCarrier sSpacing (kHz) |  |  | 10.937500 | 7.812500 | 9.765625 | 10.937500 | 10.937500 |
| Useful sSymbol tTime Tu (µs) |  |  | 91.429 | 128 | 102.4 | 91.429 | 91.429 |
| Cyclic Prefix (CP) Tg=1/8 Tu | Symbol tTime Ts (µs) |  | 102.857 | 144 | 115.2 | 102.857 | 102.857 |
|  | FDD | Number of OFDM symbols per 5ms fFrame | 48 | 34 | 43 | 48 | 48 |
|  |  | Idle time (µs) | 62.857 | 104 | 46.40 | 62.857 | 62.857 |
|  | TDD | Number of OFDM symbols per 5ms fFrame | 47 | 33 | 42 | 47 | 47 |
|  |  | TTG + RTG (µs) | 165.714 | 248 | 161.6 | 165.714 | 165.714 |
| Cyclic Prefix (CP) Tg=1/16 Tu | Symbol tTime Ts (µs) |  | 97.143 | 136 | 108.8 | 97.143 | 97.143 |
|  | FDD | Number of OFDM symbols per 5ms fFrame | 51 | 36 | 45 | 51 | 51 |
|  |  | Idle time (µs) | 45.71 | 104 | 104 | 45.71 | 45.71 |
|  | TDD | Number of OFDM symbols per 5ms fFrame | 50 | 35 | 44 | 50 | 50 |
|  |  | TTG + RTG (µs) | 142.853 | 240 | 212.8 | 142.853 | 142.853 |
| Cyclic Prefix (CP) Tg=1/4 Tu | Symbol tTime Ts (µs) |  | 114.286 | 160 | 128 | 114.286 | 114.286 |
|  | FDD | Number of OFDM symbols per 5ms fFrame | 43 | 31 | 39 | 43 | 43 |
|  |  | Idle time (µs) | 85.694 | 40 | 8 | 85.694 | 85.694 |
|  | TDD | Number of OFDM symbols per 5ms fFrame | 42 | 30 | 38 | 42 | 42 |
|  |  | TTG + RTG (µs) | 199.98 | 200 | 136 | 199.98 | 199.98 |

Table 1 OFDMA parameters for IEEE 802.16m

CP size of ¼ is used in channels with long delay spread. Tone dropping at both edges of the frequency band based on 10 and 20 MHz systems can be used to support other various bandwidths.

## Frame structure

### Basic Frame structure

The IEEE 802.16m basic frame structure is illustrated in Figure 1. Each 20 ms superframe is divided into four equally-sized 5 ms radio frames and begins with the superframe header (SFH). When using the same OFDMA parameters as in Table 1 with the channel bandwidth of 5 MHz, 10 MHz, or 20 MHz, each 5 ms radio frame further consists of eight subframes for CP sizes of 1/8 and 1/16. With the channel bandwidth of 8.75 and 7 MHz, each 5 ms radio frame further consists of seven and six subframes, respectively for CP sizes of 1/8 and 1/16.

In the case of G = 1/4, the number of subframes per frame is one less than that of other CP lengths for each bandwidth case. A subframe is assigned for either DL or UL transmission. There are three four types of subframes: 1) the type-1 subframe which consists of six OFDMA symbols, 2) the type-2 subframe that which consists of seven OFDMA symbols, and 3) the type-3 subframe which consists of five OFDMA symbols, and 4) the type-4 subframe which consists of nine OFDMA symbols. This type is applied only to UL subframe for the 8.75MHz channel bandwidth when supporting the IEEE Std 802.16-2009 frames.The basic frame structure is applied to FDD and TDD duplexing schemes, including H-FDD MS operation. The number of switching points in each radio frame in TDD systems is two, where a switching point is defined as a change of directionality, i.e., from DL to UL or from UL to DL.

When H-FDD mobile stations MS are included in an FDD system, the frame structure from the point of view of the H-FDD mobile stations MS is similar to the TDD frame structure; however, the DL and UL transmissions occur in two separate frequency bands. The transmission gaps between DL and UL (and vice versa) are required to allow switching the TX and RX circuitry.

A data burst occupies either one subframe (i.e. the default TTI transmission) or contiguous multiple subframes (i.e. the long TTI transmission). The long TTI in FDD is equal to 4 subframes for both DL and UL. The long TTI in TDD is equal to the whole DL (UL) subframes for DL (UL) in a frame.

Figure 1 Basic frame structure for 5, 10, and 20 MHz channel bandwidths

#### Frame Structure for CP=1/8 Tu

For nominal channel bandwidths of 5, 10, and 20 MHz, an IEEE 802.16m frame for a CP of 1/8 Tu has eight type-1 subframes for FDD, and seven type-1 subframes and one type-2 subframes for TDD.

Figure 30 illustrates an example of TDD and FDD frame structure for 5, 10, and 20 MHz channel bandwidth with a CP of 1/8 Tu. With OFDM symbol duration of 102.857 µs and a CP length of 1/8 Tu, the length of type-1 and type-3 subframes are 0.617 ms and 0.514 ms, respectively. TTG and RTG are 105.714 µs and 60 µs, respectively. Other numerologies may result in different number of subframes per frame and symbols within the subframes.

In FDD, the structure of frame (number of subframes, their types etc.) has to be identical for the DL and UL for each specific frame.

Figure 30 illustrates an example TDD frame structure with DL to UL ratio of 5:3. Assuming OFDMA symbol duration of 102.857µs and a CP length of 1/8 Tu, the lengths of type-1 subframe and type-3 subframe are 0.617 ms and 0.514 ms, respectively. In Figure 30, the last DL subframe, i.e., DL SF4, is a type-3 subframe. TTG and RTG are 105.714 µs and 60µs, respectively. Other numerologies may result in different number of subframes per frame and symbols within the subframes. Figure 31 shows an example of a frame structure in FDD mode.

Figure 2 Frame structure in TDD duplex mode (CP=1/8 Tu)

S0

S1

F3

F2

F1

F0

S2

S3

S4

S5

SF0

SF1

SF2

UL/DL

SF3

SF4

SF5

SF6

SF7

UL/DL PHY Frame: 5 ms (8 subframes + Idle Time)

UL/DL PHY Subframe: 0.617 ms

One OFDM symbol

Superframe: 20 ms (4 frames, 32 subframes)

Idle Time = 62.86 µs

CP=1/8 Tu

Figure 3 Frame structure with type-1 subframe in FDD duplex mode

(CP=1/8 Tu)

Figure 30 TDD and FDD Frame Structure with a CP of 1/8 Tu (DL to UL ratio of 5:3)

#### Frame Structure for CP=1/16 Tu

For nominal channel bandwidths of 5, 10, and 20 MHz, an IEEE 802.16m frame for a CP of 1/16 Tu has five type-1 subframes and three type-2 subframes for FDD, and six type-1 subframes and two type-2 subframes for TDD. The subframe preceding a DL to UL switching point is a type-1 subframe.

Figure 4 illustrates an example of TDD and FDD frame structure for 5, 10, and 20 MHz channel bandwidth with a CP of 1/16 Tu. With Assuming OFDM symbol duration of 97.143 µs and a CP length of 1/16 Tu, the length of type-1 and type-2 subframes are 0.583 ms and 0.680 ms, respectively. TTG and RTG are 82.853 µs and 60 µs, respectively. Other numerologies may result in different number of subframes per frame and symbols within the subframes.

Figure 4 TDD and FDD Frame Structure with a CP of 1/16 Tu (DL to UL ratio of 5:3)

In case of FDD, the structure of a frame (number of subframes, their types etc.) has to be identical for the DL and UL for each specific frame.

#### Superframe Header

As shown in Figure 1, each superframe begins with a DL subframe that contains a superframe header.

#### Transmission Time Interval

The transmission time interval (TTI) is the duration of the transmission of the physical layer encoded packet over the radio air interface and is equal to an integer number of subframes. The default TTI is 1 subframe.

### Frame Structure Supporting Legacy Frames

The legacy and IEEE 802.16m frames are offset by an integer number of subframes to accommodate new features such as the IEEE 802.16m Advanced Preamble (preamble), Superframe Header (system configuration information), and control channels, as shown in Figure 5. The FRAME_OFFSET shown in Figure 5 is for illustration. It is an offset between the start of the legacy frame and the start of the IEEE 802.16m frame defined in a unit of subframes.

For UL transmissions both TDM and FDM approaches are supported for multiplexing of YMS and AMS.

UL

DL

UL

DL

DL

DL

UL

UL

UL

DL

DL

DL

DL

DL

UL

UL

UL

DL

DL

DL

DL

FRAME OFFSET (Toffset)

Legacy Frame Duration (Tf)

Legacy Radio Frame from the point of view of legacy BS/MS

IEEE 802.16m Duration (Tf)

New Frame Duration (Tf)

New Radio Frame from the point of view of new BS/MS

Figure 5 Relative position of the IEEE 802.16m and IEEE Std 802.16-2009 802.16e radio frames (example TDD duplex mode)

#### The Concept of Time Zones

The time zone is defined as an integer number (greater than 0) of consecutive subframes. The concept of time zones is equally applied to TDD and FDD systems. The MZones and LZones are time-multiplexed (TDM) across time domain for the downlink. For UL transmissions both TDM and FDM approaches are supported for multiplexing of YMSs and AMSs. Note that DL/UL traffic for the AMS can be scheduled in both zones in either zone according to the mode (IEEE 802.16m or legacy IEEE Std 802.16-2009) with which AMS is connected to the ABS, but not in both zones at the same time, whereas the DL/UL traffic for the MS can only be scheduled in the LZones.

In the absence of any IEEE Std 802.16-2009 802.16e system, the LZones will disappear and the entire frame will be allocated to the MZones and thereby new systems.

##### Time Zones in TDD

In a mixed deployment of YMSs and new AMSs, the allocation of time zones in the TDD mode is as shown in Figure 6. The duration of the zones may vary. Every frame starts with a preamble and the MAP followed by IEEE Std 802.16-2009 802.16e DL zone since MSs/relays expect LZones in this region. Similarly, in a mixed deployment of MSs and new AMSs, the UL portion starts with IEEE Std 802.16-2009 802.16e UL zone since YBS /YMS/RS expect IEEE Std 802.16-2009 802.16e UL control information be sent in this region. Here the coexistence is defined as a deployment where YBSs and ABSs co-exist on the same frequency band and in the same or neighboring geographical areas. In a green-field deployment where no YMS exists, the LZones can be removed.

The DL to UL and UL to DL switching points should be synchronized across network to reduce inter-cell interference.

The switching points would require use of idle symbols to accommodate the gaps. In case of TDD operation with the generic frame structure, the last symbol in the slot immediately preceding a downlink-to-uplink/uplink-to-downlink switching point may be reserved for guard time and consequently not transmitted.

SF#7

UL

SF#6

UL

SF#5

UL

SF#4

DL

SF#3

DL

SF#2

DL

SF#1

DL

SF#0

DL

SF#7

UL

SF#6

UL

SF#5

UL

SF#4

DL

SF#3

DL

SF#2

DL

SF#1

DL

SF#0

DL

SF#7

UL

SF#6

UL

SF#5

UL

SF#4

DL

SF#3

DL

SF#2

DL

SF#1

DL

SF#0

DL

5 ms Radio Frame

DL LZone

DL MZone

UL LZone

UL MZone

Switching Point

Figure 6 Example of Time zones in TDD mode

##### Time Zones in FDD

In a mixed deployment of legacy terminals and new AMSs, an example of the allocation of time zones in the FDD mode is shown in Figure 7.

Figure 7 Example of Time zones in FDD mode

### Relay Support in Frame Structure

An ABS that supports ARSs communicates with the ARS in the MZone. The ABS multiplexes the LZone and the MZone using TDM in the DL. In the UL, the ABS can support TDM as well as FDM for multiplexing LZone and the MZone. The IEEE 802.16m specification shall not alter the LZone operation. The access link and the relay link communications in the LZone is multiplexed in accordance with the IEEE 802.16j specifications.

An RS radio frame may also define points where the RS switches from receive mode to transmit mode or from transmit mode to receive mode, where the receiveing and transmittingoperations are both performed on either DL or UL data. An ARS communicates with the YMS in the LZone.

The start of the LZone and MZone of the ABS and all the subordinate RSs/ARSs associated with the ABS are time aligned. The duration of the LZone of the ABS and the RS may be different.

- IEEE Std 802.16-2009Access Zone

  - where ABS, a RS or a ARS communicates with a IEEE Std 802.16-2009 MS.

- IEEE 802.16j Relay Zone

  - where ABS communicates with a RS.

The Relay frame structure is illustrated in Figure 8.

Figure 8 Relay Frame structure

Definitions shown in Figure 8

- IEEE 802.16m DL Access Zone: An integer multiple of subframes located in the MZone of the ABS frame, where an ABS can transmit to the AMSs.

- IEEE 802.16m UL Access Zone: An integer multiple of subframes located in the MZone of the ABS frame, where an ABS can receive from the AMSs.

- DL Access Zone: An integer multiple of subframes located in the Mzone of the DL of the ABS frame or ARS frame, where a ABS or an ARS can transmit to the AMSs. A-PREAMBLE and SFH as well as unicast transmissions may be performed in this zone.

- UL Access Zone: An integer multiple of subframes located in the Mzone of the UL of the ABS frame, where an ABS can receive from the AMSs.

- DL Transmit Zone: An integer multiple of subframes located in the MZone of the DL of the ABS frame or ARS frame, where an ABS or ARS can transmit to subordinate ARSs and the AMSs.

- DL Receive Zone: An integer multiple of subframes located in the MZone of the DL of the ARS frame, where a ARS can receive from its superordinate station.

- UL Transmit Zone: An integer multiple of subframes located in the MZone of the UL of the ARS frame, where a ARS can transmit to its superordinate station.

- UL Receive Zone: An integer multiple of subframes located in the MZone of the UL of the ABS frame or ARS frame, where an ABS or ARS can receive from its subordinate ARSs and the AMSs.

- Network Coding Transmit Zone: An integer multiple of subframes located in the DL of the frame of the Odd Hop ARS which is directly attached to the ABS, where an Odd Hop ARS can transmit network coded transmissions to the ABS and Even Hop ARS. The tTransmissions to the AMS in this zone are being FFS studied.

- Network Coding Receive Zone: An integer multiple of subframes located in the DL of the ABS or Even Hop ARS frame, where an ABS or Even Hop ARS can receive network coded transmissions from the ARS directly attached to the ABS.

If the ABS supports network coding, the presence of the aforementioned zones is determined by the ABS depending on the number of hops and the ARS capabilities. The Network Coding Transmit Zone may be present in an ARS frame if the ARS supports network coding. If the Network Coding Transmit Zone is present, it appears only in the frame of an ARS which is directly attached to the ABS. The Network Coding Receive Zone may be present only in the frames of the ABS and the even hop ARS that is two hops away from the ABS, if the ARS and the ABS support network coding.

### Coexistence Support in Frame Structure

IEEE 802.16m downlink radio frame is time aligned with reference timing signal as defined in section Error: Reference source not found and should support symbol puncturing to minimize the inter-system interference.

#### Adjacent Channel Coexistence with E-UTRA (LTE-TDD)

Coexistence between IEEE 802.16m and E-UTRA in TDD mode may be facilitated by inserting either idle symbols within the IEEE 802.16m frame or idle subframes, for certain E-UTRA TDD configurations. An operator configurable delay or offset between the beginning of an IEEE 802.16m frame and an E-UTRA TDD frame can be applied in some configurations to minimize the time allocated to idle symbols or idle subframes. Figure 9 shows two examples using frame offset to support coexistence with E-UTRA TDD in order to support minimization of the number of punctured symbols within the IEEE 802.16m frame.

Figure 9 Alignment of IEEE 802.16m frame and E-UTRA frame in TDD mode

#### Adjacent Channel Coexistence with UTRA LCR-TDD (TD-SCDMA)

Coexistence between IEEE 802.16m and UTRA LCR-TDD may be facilitated by inserting either idle symbols within the IEEE 802.16m frame or idle subframes. An operator configurable delay or offset between the beginning of an IEEE 802.16m frame and an UTRA LCR-TDD frame can be applied in some configurations to minimize the time allocated to idle symbols or idle subframes. Figure 10 demonstrates how coexistence between IEEE 802.16m and UTRA LCR-TDD can be achieved to minimize the inter-system interference.

Figure 10 Alignment of IEEE 802.16m frame with UTRA LCR-TDD frame in TDD mode