| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Signaling Overhead reduction and Text Refinements to Sections 15.3.8 (Uplink Physical Structure) of IEEE 802.16m AWD -- AWD-IMDG |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Debdeep Chatterjee, Jong-Kae Fwu, Hujun Yin Intel Corporation HanGyu Cho LG Electronics Taeyoung Kim, Jeongho Park Samsung Electronics Yanfeng Guan, Huiying Fang ZTE Corporation Chia-Lung Tsai, Yu-Tao Hsieh, Chang-Lan Tsai, Zheng Yan-Xiu ITRI Gye-Tae Gil KT Corporation | E-mail: jong-kae.fwu@intel.com E-mail: hgcho@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD comments / Area: Chapter 15.3.8 (UL-PHY) “Comments on AWD 15.3.8 UL-PHY” 802.16m amendment working document |  |
| Abstract | Proposed text for the UL Physical Structure in the draft P802.16m amendment |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Signaling Overhead reduction and Text Refinements to Sections 15.3.8 (UL PHY Structure) of IEEE 802.16m AWD -- AWD-IMDG

Debdeep Chatterjee, Jong-Kae Fwu, Hujun Yin

Intel

HanGyu Cho

LG Electronics

Taeyoung Kim, Jeongho Park

Samsung Electronics

Yanfeng Guan, Huiying Fang

ZTE Corporation

Chia-Lung Tsai, Yu-Tao Hsieh, Chang-Lan Tsai, Zheng Yan-Xiu

ITRI

Gye-Tae Gil

KT Corporation

Introduction

As a decision from the IM DG, a seperate subteam is formed to address the PHY and FFR jointly related issues with the goal to produce a clean-up version of AWD PHY Structure proposal. The proposal will refine the current AWD PHY structure related text and include decisions made by the group related to the PHY signaling, overhead reduction, text clean up, etc.

Plenty of discussions in the IM DG related to the PHY Structure related signaling by both emails and conference calls. Those signalings are mainly related to different frequency partitions, CRU/DRU partition, signaling overhead reduction, etc.  The final decisions are captured in this proposal.

This contribution provides updated amendment text for the UL physical structure in the Advanced Air Interface to be included in Section 15 of the IEEE 802.16m Amendment Working Document [1]. The proposed update is developed so that it can be readily combined with IEEE 802.16m-09/0010r2 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

References

[1] IEEE 802.16m-09/0010r2, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” Jun. 2009.

[2] IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements Document,” Jan. 2009.

[3] IEEE 802.16m-08/003r9a, “IEEE 802.16m System Description Document,” May 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

Proposed Amendment Text

The following text (including tables and figures) is proposed to replace the current contents of Chapter 15.3.8, until the beginning of Sub-clause 15.3.8.3.3, of the IEEE 802.16m Amendment Working Document [1].

[  ------------------- Proposed Text Starts --------------------]

### 15.3.8 Uplink physical structure

Each uplink subframe is divided into 4 or fewer frequency partitions; each partition consists of a set of physical resource units across the total number of OFDMA symbols available in the subframe. Each frequency partition can include contiguous (localized) and/or non-contiguous (distributed) physical resource units. Each frequency partition can be used for different purposes such as fractional frequency reuse (FFR). Figure 484 illustrates the uplink physical structure in the example of two frequency partitions with frequency partition 2 including both contiguous and distributed resource allocations, where Sc stands for Subcarrier.

### 15.3.8.1 Physical and logical resource unit

A physical resource unit (PRU) is the basic physical unit for resource allocation that comprises Psc consecutive subcarriers by Nsym consecutive OFDMA symbols. Psc is 18 and Nsym is 6, 7, 5, and 9 OFDMA symbols for type-1, type-2, type-3, and type-4 subframes, respectively. A logical resource unit (LRU) is the basic logical unit for distributed and localized resource allocations. An LRU has Psc x Nsym subcarriers.

The LRU size for control channel transmission should be same as for data transmission. Multiple users are allowed to share one control LRU. The effective number of data subcarriers in an LRU depends on the number of allocated pilots and control channel presence.

### 15.3.8.1.1 Distributed logical resource unit

The distributed logical resource unit (DLRU) contains a group of subcarriers that are spread across the distributed resource allocations within a frequency partition. The size of the DLRU equals the size of a PRU, i.e., Psc subcarriers by Nsym OFDMA symbols. The minimum unit for forming the DRU is a tile. The uplink tile size is 6 x Nsym, where the value of Nsym depends on the subframe type.

### 15.3.8.1.2 Contiguous logical resource unit

The localized logical resource unit, also known as contiguous logical resource unit (CLRU) contains a group of subcarriers that are contiguous across the resource allocations. The size of the CLRU equals the size of a PRU, i.e., Psc subcarriers by Nsym OFDMA symbols.

### 15.3.8.2 Multi-cell resource mapping

The UL multi-cell resource mapping consists of subband partitioning, miniband permutation and frequency partitioning and is defined in the following sub-clauses.

### 15.3.8.2.1 Subband Partitioning

The PRUs are first divided into subbands and minibands; a subband comprises of N1 adjacent PRUs and a miniband comprises of N2 adjacent PRUs where N1=4 and N2=1. Subbands are suitable for frequency selective allocations as they provide a continuous allocation of PRUs in frequency. Minibands are suitable for frequency diverse allocation and are permuted in frequency.

The number of subbands is denoted by KSB. The number of PRUs allocated to subbands is LSB = N1· KSB. A 5, 4 or 3-bit field called Uplink Subband Allocation Count (USAC) determines the value of KSB depending on system bandwidth. The USAC is transmitted in the SFH. The remaining PRUs are allocated to minibands. The number of minibands in an allocation is denoted by KMB. The number of PRUs allocated to minibands is LMB = N2· KMB. The total number of PRUs is NPRU = LSB + LMB. The maximum number of subbands that can be formed is denoted as Nsub where Nsub=NPRUN1. Mappings between USAC and KSB are shown in Tables 714 through 716 for system bandwidths of 20MHz, 10MHz, and 5MHz, respectively.

Table 714—Mapping between USAC and KSB for 20MHz

| USAC | # of subbands allocated (KSB) | USAC | # of subbands allocated (KSB) |
| --- | --- | --- | --- |
| 0 | 0 | 16 | 16 |
| 1 | 1 | 17 | 17 |
| 2 | 2 | 18 | 18 |
| 3 | 3 | 19 | 19 |
| 4 | 4 | 20 | 20 |
| 5 | 5 | 21 | 21 |
| 6 | 6 | 22 | N. A. |
| 7 | 7 | 23 | N. A. |
| 8 | 8 | 24 | N. A. |
| 9 | 9 | 25 | N. A. |
| 10 | 10 | 26 | N. A. |
| 11 | 11 | 27 | N. A. |
| 12 | 12 | 28 | N. A. |
| 13 | 13 | 29 | N. A. |
| 14 | 14 | 30 | N. A. |
| 15 | 15 | 31 | N. A. |
|  | # of subbands allocated (KSB) |  |  |

Table 715—Mapping between USAC and KSB for 10MHz

| USAC | # of subbands allocated (KSB) | USAC | # of subbands allocated (KSB) |
| --- | --- | --- | --- |
| 0 | 0 | 8 | 8 |
| 1 | 1 | 9 | 9 |
| 2 | 2 | 10 | N. A. |
| 3 | 3 | 11 | N. A. |
| 4 | 4 | 12 | N. A. |
| 5 | 5 | 13 | N. A. |
| 6 | 6 | 14 | N. A. |
| 7 | 7 | 15 | N. A. |
|  | # of subbands allocated (KSB) |  | # of subbands allocated (KSB) |

Table 716—Mapping between USAC and KSB for 5MHz

| USAC | # of subbands allocated (KSB) | USAC | # of subbands allocated (KSB) |
| --- | --- | --- | --- |
| 0 | 0 | 4 | N. A. |
| 1 | 1 | 5 | N. A. |
| 2 | 2 | 6 | N. A. |
| 3 | 3 | 7 | N. A. |

The PRUs are partitioned and reordered into two groups: subband PRUs (PRUSB), and miniband PRUs (PRUMB). The set of PRUSB is numbered from 0 to (LSB-1) and the set of PRUMB from 0 to (LMB-1).

Equation (223) defines the mapping of PRUs into PRUSBs. Equation (225) defines the mapping of PRUs to PRUMBs. Figure 485 illustrates the PRU to PRUSB and PRUMB mapping for a 10 MHz bandwidth with KSB equal to 7.

(223)

where

i=N1⋅NsubKSB⋅jN1+jN1⋅GCD(Nsub,NsubKSB)NsubmodNsub+jmodN1   (224)

(225)

where

i=N1⋅NsubKSB⋅k+LSBN1+k+LSBN1⋅GCD(Nsub,NsubKSB)NsubmodNsub+k+LSBmodN1KSB>0kKSB=0											    (226)

Figure 485 - PRU to PRUSB  and PRUMB mapping for BW=10 MHz, KSB=7

### 15.3.8.2.2 Miniband permutation

The miniband permutation maps the PRUMBs to permuted-PRUMBs (PPRUMBs) to insure allocation of frequency diverse PRUs to each frequency partition. Equation (227) describes the mapping from PRUMBs to PPRUMBs.

(227)

where:

Figure 486 depicts the mapping from PRUs to PRUSB and PPRUMB.

Figure 486 - Mapping from PRUs to PRUSB  and PPRUMB  for BW=10 MHz, KSB=7

### 15.3.8.2.3 Frequency partitioning

The PRUSBs and PPRUMBs are allocated to one or more frequency partitions. By default, only one partition is present. The maximum number of frequency partitions is 4. The frequency partition configuration is transmitted in the SFH in a 4, 3-bit composite field called the Uplink Frequency Partition Configuration (UFPC), depending on system bandwidth. The Frequency Partition Count (FPCT) defines the number of frequency partitions. The Frequency Partition Size (FPSi) defines the number of PRUs allocated to FPi. FPCT and FPSi are determined from UFPC as shown in Tables 717 through 719.

A 3, 2, or 1-bit called the Uplink Frequency Partition Subband Count (UFPSC) defines the number of subbands allocated to FPi for i>0.

Table 717—Mapping between UFPC and frequency partitioning for 20MHz

| UFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 3 | 3 : 1 : 1 : 1 | 4 | NPRU * 1/2 | NPRU * 1/6 |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU * 5/8 | NPRU * 1/8 |
| 5 | 9 : 1 : 1 : 1 | 4 | NPRU * 9/12 | NPRU * 1/12 |
| 6 | 9 : 5 : 5 : 5 | 4 | NPRU * 3/8 | NPRU * 5/24 |
| 7-15 | Reserved |  |  |  |

Table 718—Mapping between UFPC and frequency partitioning for 10MHz

| UFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 3 | 3 : 1 : 1 : 1 | 4 | NPRU * 1/2 | NPRU * 1/6 |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU * 5/8 | NPRU * 1/8 |
| 5 | 9 : 5 : 5 : 5 | 4 | NPRU * 3/8 | NPRU * 5/24 |
| 6-7 | Reserved |  |  |  |

Table 719—Mapping between UFPC and frequency partitioning for 5MHz

| UFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 3 | 3 : 1 : 1 : 1 | 4 | NPRU * 1/2 | NPRU * 1/6 |
| 4 | 9 : 5 : 5 : 5 | 4 | NPRU*3/8 | NPRU*5/24 |
| 5-7 | Reserved |  |  |  |

The number of subbands in the ith frequency partition is denoted by KSB,FPi, as shown in equation (228) for the cases when FPCT = 1 or FPCT = 4,

(228)

When UFPC = 1 and FPCT = 3, the number of subbands in FPi (for i>0) is given by KSB,FPi = UFPSC.  The number of minibands in the ith frequency partition is denoted by KMB,FPi, as shown in equation (229),

(229)

The numbers of subband PRUs and miniband PRUs in each frequency partition are given by LSB,FPi = N1·KSB,FPi  and LMB,FPi = N2·KMB,FPi respectively.

The mapping of subband PRUs and miniband PRUs to the frequency partition i is given by the following equations:

(230)

Where  and .

Figure 487 depicts the frequency partitioning for BW of 10 MHz, KSB = 7, FPCT = 4, FPS0 = FPSi = 12, and UFPSC = 2.

Figure 487 - Frequency partition for BW=10 MHz, KSB=7, FPCT=4, FPS0= FPSi=12, UFPSC=2

### 15.3.8.3 Cell-specific resource mapping

PRUFPis are mapped to LRUs. All further PRUs and tile permutations are constrained to the PRUs within a frequency partition.

### 15.3.8.3.1 CRU/DRU allocation

The partition between CRUs and DRUs is done on a sector specific basis. Let LSB,FPi and LMB,FPi denote the numbe of allocated subband CRUs and miniband CRUs for FPi (i  0). The number of total allocated subband-based CRUs, in units of a subband (i.e. N1 PRUs), for FPi (for i ≥ 0) is given by uplink CRU allocation size, UCASi. The numbers of subband-based and miniband-based CRUs in FP0 are given by UCASSB,0 and UCASMB,0, in units of a subband and miniband, respectively.

For FP0, the value of UCASSB,0 is explicitly signaled in the SFH as a 5, 4 or 3-bit field. A 5, 4, or 3-bit uplink miniband-based CRU allocation size (UCASMB) is sent in the SFH only for partition FP0, depending on system bandwidth. The number of subband-based CRUs for FP0 is given by the equation (XXX).

LSB,FP0=N1⋅UCASSB,0		(XXX)

The mapping between UCASMB,0 and the number of miniband-based CRUs for FP0 is shown in the Tables 720 through 722 for system bandwidths of 20MHz, 10MHz, and 5MHz, respectively.

Table 720 - Mapping between UCASMB,0 and number of miniband-based CRU for FP0 for 20MHz

| UCASMB | # of miniband-based CRU for FP0 (LMB,FP0) | UCASMB | # of miniband-based CRU for FP0 (LMB,FP0) |
| --- | --- | --- | --- |
| 0 | 0 | 16 | 16 |
| 1 | 1 | 17 | 17 |
| 2 | 2 | 18 | 18 |
| 3 | 3 | 19 | 19 |
| 4 | 4 | 20 | 20 |
| 5 | 5 | 21 | 21 |
| 6 | 6 | 22 | 22 |
| 7 | 7 | 23 | 23 |
| 8 | 8 | 24 | 24 |
| 9 | 9 | 25 | 28 |
| 10 | 10 | 26 | 32 |
| 11 | 11 | 27 | 36 |
| 12 | 12 | 28 | 40 |
| 13 | 13 | 29 | 44 |
| 14 | 14 | 30 | 48 |
| 15 | 15 | 31 | 52 |

Table 721 - Mapping between UCASMB,0 and number of miniband-based CRUs for FP0 for 10MHz

| UCASMB | # of miniband-based CRU for FP0 (LMB,FP0) | UCASMB | # of miniband-based CRU for FP0 (LMB,FP0) |
| --- | --- | --- | --- |
| 0 | 0 | 8 | 8 |
| 1 | 1 | 9 | 9 |
| 2 | 2 | 10 | 10 |
| 3 | 3 | 11 | 11 |
| 4 | 4 | 12 | 12 |
| 5 | 5 | 13 | 16 |
| 6 | 6 | 14 | 20 |
| 7 | 7 | 15 | 24 |

Table 722 - Mapping between UCASMB,0 and number of miniband-based CRUs for FP0 for 5MHz

| UCASMB | # of miniband-based CRU for FP0 (LMB,FP0) | UCASMB | # of miniband-based CRU for FP0 (LMB,FP0) |
| --- | --- | --- | --- |
| 0 | 0 | 4 | 4 |
| 1 | 1 | 5 | 6 |
| 2 | 2 | 6 | 8 |
| 3 | 3 | 7 | 10 |

For FPi (i > 0), only one value for UCASi is explicitly signaled for all i > 0, in the SFH as a 3, 2 or 1-bit field to signal the same numbers of allocated CRUs for FPi (i>0). For FPi (i > 0), the number of subband CRUs (LSB,FPi)and miniband CRUs (LMB,FPi) are derived using equations XYZ and YYZ respectively.

LSB,FPi=min						  	  (XYZ)

LMB,FPi=0,UCASi≤KSB,FPi(UCASi−KSB,FPi)⋅N1UCASi>KSB,FPi				  (YYZ)

The total number of CRUs in frequency partition FPi for 0 i <FPCT, is denoted by LCRU,FPi calculated as shown in Equation (231)

LCRU,FPi=LSB,FPi+LMB,FPi

(231)

The number of DRUs in each frequency partition is denoted by LDRU,FPi, calculated as shown in Equation (232)

(232)

The mapping of PRUFPi to CRUFPi (for 0 ≤ i < FPCT) is given by Equation (233):

CRUFPi[j]=PRUFPi[j],0≤j<LSB,FPiPRUFPi[k+UCASSB,i⋅N1],LSB,FPi≤j<LCRU,FPi		    (233)

where k=s[j−LSB,FPi]. s[] is the CRU/DRU-allocation sequence defined below and 0≤s[j]<FPSi−LSB,FPi:

s[j]=PermSeq(j)+ULPermBasemod(FPSi−LSB,FPi),

where PermSeq() is the permutation sequence of length (FPSi-LSB,FPi) and is determined by SEED={IDcell*343} mod 210. The permutation sequence is generated by the random sequence generation algorithm specified in 15.3.5.3.3. UL_PermBase is an interger ranging from 0 to 31(TBD), which is set to preamble IDcell.

The mapping of PRUFPi to DRUFPi is given by Equation (234):

(234)

where k = s[j + LCRU,FPi – LSB,FPi].

Figure xxx presents an example to illustrate the various steps of subband partitioning, miniband permutation, frequency partitioning, and cell-specific resource mapping (CRU/DRU allocation) for the case of 10MHz system bandwidth. For this example, KSB = USAC = 7, FPCT = 4, FPSi = 12 (for i≥0), UFPSC = 2, UCASSB,0 = 1, UCASMB,0=2, and UCASi=2.

Figure xxx - CRU/DRU allocation for BW=10 MHz, KSB = USAC = 7, FPCT = 4, FPSi = 12 (for i≥0), UFPSC = 2, UCASSB,0 = 1, UCASMB,0=2, and UCASi=2.

15.3.8.3.2 Tile permutation

Each of the DRUs of an UL frequency partition is divided into 3 tiles of 6 adjacent subcarriers over Nsym symbols. The tiles within a frequency partition are collectively tile-permuted to obtain frequency-diversity across the allocated resources.

The tile permutation that allocates physical tiles of DRUs to logical tiles of subchannels is performed in the following manner:

(235)

where:

Tiles(s,n,t) is the tile index of the nth tile in the sth distributed LRU of the tth subframe,

n is the tile index, 0 to 2, in a distributed LRU,

t is the subframe index with respect to the frame,

s is the distributed LRU index, 0 to LDRU,FPi-1,

PermSeq() is the permutation sequence of length LDRU,FPi and is determined by SEED={IDcell*343} mod 210. The permutation sequence is generated by the random sequence generation algorithm specified in Section 15.3.5.3.3,

and g(PermSeq(),s,n,t) = {PermSeq[(n + 107 * s + t) mod LDRU,FPi]+UL_PermBase} mod LDRU,FPi.

[  ------------------- Proposed Text Ends --------------------]