| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Logical combinations of trigger conditions |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Stavros Tzavidas Motorola | stavros.tzavidas@motorola.com |
| Re: | 802.16m amendment working document |  |
| Abstract | This contribution proposes a method for allowing the serving ABS to define any type of logical combination between trigger conditions (e.g. AND or OR combinations). |  |
| Purpose | For review and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Logical Combinations of trigger conditions

Stavros Tzavidas

Motorola

# Introduction

Section 15.2.6.2 describes how the Serving ABS (S-ABS) can define trigger conditions and the associated actions. When one or more of triggers are satisfied for a particular ABS the AMS is required to perform the action associated with the trigger.

The current text does not describe how the AMS is expected to behave when multiple triggers are satisfied for the same ABS. This is not purely an AMS implementation issue. If the method of combining multiple triggers is left to AMS implementation, ambiguous situations can arise, as the following example shows:

Suppose that the S-ABS defines two trigger conditions both with action code 0x2 (“Respond to trigger with AAI_HO-REQ”, i.e. initiate a handover to this particular ABS).

- Some AMS implementations may interpret this as a logical OR and initiate a handover to a particular neighbor ABS if any of the two conditions is satisfied.

- Other AMS implementations may interpret this as a logical AND and only initiate a handover to a neighbor ABS if both conditions are satisfied.

Clearly this situation leads to unpredictable AMS behavior since the S-ABS has no way of knowing how each AMS will behave.

# Proposed remedy

To resolve this ambiguity we propose to allow the S-ABS to explicitly define how multiple triggers should be defined. This can be accomplished by simply describing the format that the S-ABS can use to define a logical AND between triggers. If this format is used then the triggers are logically AND’d together. If not then the triggers are logically OR’d.

Note that we do not propose that multiple triggers should always be combined with a logical AND or always combined with a logical OR. We instead propose that the S-ABS be given the ability to define any logical combination between triggers (i.e. both AND and OR logical combinations).

# Examples of logical OR and AND operations between triggers

Some examples in which the operator (i.e. the S-ABS) may want to define different logical combinations of triggers are:

- The operator wants to use a trigger to ensure that sufficient improvement in CINR is available at the neighbor BS but only if the serving BS's CINR is low enough. In this case, the following HO triggers would be AND'd together

  - HO Trigger 1: Action = send MSHO-REQ; function = neighbor BS > serving BS by relative value; type = CINR, value = 3 dB

  - HO Trigger 2: Action = send MSHO-REQ; function = serving BS < by threshold; type = CINR, value = 15 dB

- The operator wants to set different relative trigger for weak CINR conditions compared to strong CINR. For example, for low CINR (say less than 20 dB), a 3 dB relative improvement in CINR would be required. However, at stronger absolute CINR values (say>= 20 dB), a lower relative value of 0 dB is allowed. In this case, 4 total triggers are required using two sets:

  - Set 1 is AND'd:

    - HO Trigger 1: Action = send MSHO-REQ; function = neighbor BS > serving BS by relative value; type = CINR, value = 3 dB

    - HO Trigger 2: Action = send MSHO-REQ; function = serving BS < by threshold; type = CINR, value = 20 dB

  - Set 2 is AND'd:

    - HO Trigger 1: Action = send MSHO-REQ; function = neighbor BS > serving BS by relative value; type = CINR, value = 0 dB

    - HO Trigger 2: Action = send MSHO-REQ; function = serving BS > by threshold; type = CINR, value = 20 dB

  - but set 1 is OR'd with set 2.

# Text Proposal

In paragraph 15.2.6.2 “Trigger Condition Definitions” modify the text immediately following table 26 as follows:

----------------------------------------------Start of the text proposal ------------------------------------------------------

### 15.2.6.2 Trigger condition definitions

…

When performing measurements, the AMS shall use the averaging parameters defined in DCD with TLV “Default HO RSSI and CINR averaging parameters”, Type 121 as defined in section 11.4.1 and Table 574.

The ABS may define multiple complex trigger conditions by including multiple Trigger TLV encodings in the same compound TLV. In this case, all included triggers shall have the same Action code (as defined in Table xx2 26). The AMS shall perform a logical AND of all the conditions in a complex trigger condition and invoke the action of the trigger only when all trigger conditions are met.

-------------------------------------------------- End of the text proposal --------------------------------------------------