| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed IEEE 802.16m Text on Scanning Initiation and Unauthorized AMS Control for CSG femtocell BSs |  |
| Date Submitted | 2009-07-13 |  |
| Source(s) | Sung-ill Kim Byung Moo Lee Young Chul Oh Jong-Sik Lee KT | E-mail: semperor@kt.com |
| Re: | IEEE 80216m-09_0028r1 Call for contributions on Project 802.16m- “Support for Femtocell BS” in 802.16m amendment working document , Support for femtocell BS (Chapter 15) of IEEE 802.16m-08/003r9a |  |
| Abstract | This contribution proposes IEEE 802.16m on femtocell scanning and Unauthorized AMS Control |  |
| Purpose | Proposed to be discussed and adopted by TGm for IEEE 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to activate, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed IEEE 802.16m Text on Scanning Initiation and Unauthorized AMS Control for CSG femtocell BSs

Sung-ill Kim, Byung Moo Lee, Young Chul Oh, Jong-Sik Lee

KT

# Introduction

This contribution proposes,

- Scanning Initiation Procedures for CSG femtocell BSs (chapter II)

- Methods of unauthorized BS control for CSG-Closed Femtocell BSs (chapter III)

# Scanning Initiation Procedures for CSG femtocell BSs

In other to get the CSG femtocell BS information, an AMS should perform a full scanning or get information from network to know CSG femtocell BS’s existence. This document proposes the scanning initiation scheme for CSG femtocell BSs to reduce AMS’s scanning overhead.

o Condition : An AMS always knows its own CGS femtocell BSs.

o Procedure

Step1) When a CSG femtocell BS is powered on, it gets the information of neighbor ABSs including overlay ABS.

Step2) When an Authorized AMS performs network entry or is handed over to the CSG femtocell BS, the AMS receives the neighbor Advertisement message from the CSG femtocell BS.

Step3) The AMS obtains and saves one or more good-conditioned neighbor and/or overlay ABS information.

Step4) The AMS moves out to the other ABS.

Step5) Only if the AMS is serviced by the saved one or more good-conditioned neighbor and/or overlay ABSs, it performs scanning for CSG femtocell BSs.

This scheme can help an AMS to reduce the scanning overhead.

# Methods of unauthorized AMS control for CSG-Closed Femtocells

Since an unauthorized AMS can try to get access to CSG femtocell BSs, this document propose how to deal with this problem. In case of an unauthorized AMS trial, CSG-Closed Femtocell BSs should reject access of it at initial ranging stage, which can help to avoid wasting ABS’s resource and reduce AMS’s detached time from network. And then CSG-Closed Femtocell BSs report the connection fail reason to the unauthorized AMS and let AMS get access to other ABS. (ex, Downlink frequency override)

# Proposed Text

15.6.x Scanning for CSG femtocell BS

When an Authorized AMS performs network entry or is handed over to the CSG femtocell BS, the AMS receives the neighbor Advertisement message from the CSG femtocell BS. The AMS obtains and saves one or more good-conditioned neighbor and/or overlay ABS information. And then AMS moves out to the other ABS Only if the AMS is serviced by the saved one or more good-conditioned neighbor and/or overlay ABSs, it can performs extra scanning for CSG femtocell BSs

15.2 Types of Femtocell Base stations

CSG-Closed Femtocell BS: A CSG-Closed Femtocell BS is accessible only to the MSs, which are in its CSG, except for emergency services. MSs which are not the members of the CSG, should not try to access CSGClosed Femtocell BSs.

CSG-Closed Femtocell BS: A CSG-Closed Femtocell BS is accessible only to the AMSs, which are in its CSG, except for emergency services. Unauthorized AMSs should not try to access CSG Closed Femtocell BSs. Also, CSG-Closed Femtocell BSs should reject access of unauthorized AMSs at initial ranging stage and report the connection fail reason to the unauthorized AMS.

# References

- “IEEE 802.16m Evaluation Methodology Document(EMD),” IEEE 802.16m-08/004r1