| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AWD Clean up Text on Channel coding and HARQ (AWD-15.3.12) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Sung-Eun Park, Chiwoo Lim, Jaeweon Cho, Jaehee Cho, Heewon Kang, Hokyu Choi Samsung Electronics Co., Ltd. Changlong Xu, Harel Tom, Huaning Niu, Jong-kae Fwu, Yang-Seok Choi, Hujun Yin Intel Corporation Lei Huang, Isamu Yoshii Panasonic Corporation Sun Bo, Xu Jin ZTE Corporation Seunghyun Kang, Jinsam Kwak LG Electronics Yu-Tao Hsieh, Chien-Yu Kao, Yu-Chuan Fang, Zheng Yan-Xiu, Pang-An Ting ITRI Pei-Kai Liao, Yu-Hao Chang, Paul Cheng MediaTek Inc. Frederick Vook, Bill Hillery, Vip Desai Motorola Inc. | se.park@samsung.com changlong.xu@intel.com Lei.Huang@sg.panasonic.com sun.bo1@zte.com.cn sh_kang@lge.com ythsieh@itri.org.tw pk.liao@mediatek.com Fred.Vook@motorola.com |
| Re: | “802.16m amendment working document”: IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content”. Target topic: 15.3.12 Channel coding and HARQ |  |
| Abstract | The contribution proposes the clean up text to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AWD Clean up Text on Channel coding and HARQ (AWD-15.3.12)

Sung-Eun Park, Chiwoo Lim, Jaeweon Cho, Jaehee Cho, Heewon Kang, Hokyu Choi
Samsung Electronics Co., Ltd.

Changlong Xu, Harel Tom, Huaning Niu, Jong-kae Fwu, Yang-Seok Choi, Hujun Yin
Intel Corporation

Lei Huang, Isamu Yoshii
Panasonic Corporation

Sun Bo, Xu Jin
ZTE Corporation

Seunghyun Kang, Jinsam Kwak
LG Electronics

Yu-Tao Hsieh, Chien-Yu Kao, Yu-Chuan Fang, Zheng Yan-Xiu, Pang-An Ting
ITRI

Pei-Kai Liao, Yu-Hao Chang, Paul Cheng
MediaTek Inc.

Frederick Vook, Bill Hillery, Vip Desai
Motorola Inc.

- Introduction

The contribution proposes the clean up text on channel coding and HARQ to be included in the 802.16m amendment. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D8 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

There are three parts related channel coding in current AWD [5]. There are CTC and HARQ for data channel [6], TBCC for Downlink control channel [7], TBCC for uplink control channel [8]. In the contribution, the three parts are consolidated together. It is easy for readers to understand.

Comments have been inserted to identify the type of change (Editorial/Technical). Comments are visible with MS Word markup. Proposed text has been underlined in blue and deleted text has been struck through in red. Existing AWD text is shown in black.

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements Document”

[3] IEEE 802.16m-08/003r9a, “IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r2, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”

[6] IEEE C802.16m-09/868r2, “Proposed Text of Channel Coding and HARQ for the IEEE 802.16m Amendment”

[7] IEEE C802.16m-09/957, “Proposed TBCC code for DL control channels in the IEEE 802.16m AWD”

[8] IEEE C802.16m-09/874r2, “Proposed Amendment Text of Tail-biting Convolutional Codes for Secondary Fast Feedback Control Channel”

- Text proposal for inclusion in the 802.16m amendment

---------------------------------------------------  Text Start  ---------------------------------------------------

<Insert the following abbreviation>

- 4. Abbreviations and acronyms

CRV	Constellation rearrangement version

TBCC     Tail-biting convolutional codes

---------------------------------------------------  Text end  ---------------------------------------------------

---------------------------------------------------  Text Start  ---------------------------------------------------

< Modify page 187, line 62 in the subsection 15.3.6.3.1.1Primary Superframe Header as follows >

- The resulting sequence of bits shall be encoded by the TBCC described in 15.3.6.3.3.15.3.12.2 with parameter M=Kbufsize=4L  where L is the number of information bits.

---------------------------------------------------  Text end  ---------------------------------------------------

---------------------------------------------------  Text Start  ---------------------------------------------------

< Modify page 188, line 32 in the subsection 15.3.6.3.1.2 Secondary Superframe Header as follows >

- The resulting sequence of bits shall be encoded by the TBCC described in 15.3.6.3.3.15.3.12.2 with parameter M=Kbufsize=4L  where L is the number of information bits.

---------------------------------------------------  Text end  ---------------------------------------------------

---------------------------------------------------  Text Start  ---------------------------------------------------

< Modify page 189, line 27 in the subsection 15.3.6.3.2.2 Assignment A-MAP as follows >

- The resulting sequence of bits shall be encoded by the TBCC described in 15.3.6.3.3. 15.3.12.2. with parameter M=Kbufsize=2L for 1/2 code rate and M=Kbufsize=4L for 1/4 code rate or M=8L,Kbufsize=4L for 1/8 code rate where L is the number of information bits.

---------------------------------------------------  Text end  ---------------------------------------------------

---------------------------------------------------  Text Start  ---------------------------------------------------

<Remove whole the subsection 15.3.6.3.3 Tail-biting convolutional code for DL control channels, from page 190, line 35 to page 192, line 50>

- 15.3.6.3.3 Tail-biting convolutional code for DL control channels

---------------------------------------------------  Text end  ---------------------------------------------------

---------------------------------------------------  Text Start  ---------------------------------------------------

<Modify subsection 15.3.9.2.1.2 as follows:>

- 15.3.9.2.1.2 Secondary fast feedback control channel

The SFBCH is comprised of 3 distributed FMTs with 2 pilots allocated in each FMT. Pilot sequence is [1 1 1 1 1 1].

The SFBCH symbol generation procedure is as follows. First, the SFBCH payload information bits a0a1a2⋯al−1 are encoded to M bits b0b1b2⋯bM−1 using the TBCC encoder described in <<Section 15.3.9.2.1.2.1>>15.3.12.2.

The values of parameters L and M are set to l and 60, respectively. The value of Kbufsize should be set as Equation (XX)

Kbufsize=30 l=7,8,95l l=10,1160 12≤ l≤24									(XX)

<Remove page 291, line 35~47>

The coded sequence b0b1b2⋯bNb0b1b2⋯bM−1 is then modulated to N/2M/2 symbols c0c1c2⋯cN2−1c0c1c2⋯cM2−1 using QPSK. The value of N is TBD. The modulated symbols c0c1c2⋯cN2−1c0c1c2⋯cM2−1  and pilot sequence (TBD) p0p1p2⋯p5 are combined to form sequence d0d1d2⋯d35 according to Equation (YY) and are mapped to the data subcarriers of the SFBCH FMTs as shown in Figure 506.

di=pi'8for  mod(i',8)=0ci−i'8−1for mod(i',8)≠0  								(YY)

wherei'=iif  0≤i≤18i+5if 19≤i≤35  , 0≤i≤35

<Remove Figure 505 in page 292, line 5>

Figure 506─SFBCH comprising of three distributed 2x6 UL FMTs.

<Remove page 292, line 42~48>

<Remove Table 723 in page 292, line 52>

---------------------------------------------------  Text end  ---------------------------------------------------

---------------------------------------------------  Text Start  ---------------------------------------------------

<Modify subsection 15.3.12.1 as follows:>

- 15.3.12.1 Channel coding for data channel

Channel coding procedures for downlink and uplink data channel are shown in Figure 515.

Figure 515─Channel coding procedure for data channel

- 15.3.12.1.3 Burst partition

Only the burst sizes NDB listed in Table 741 are supported in the PHY layer. These sizes include the addition of CRC (per burst and per FEC block) when applicable. Other sizes require padding to the next burst size. When the burst size including 16 burst CRC bits exceeds the maximum FEC block size, NFB_MAX, the burst is partitioned into KFB FEC blocks. The modulation order, the nominal code rate, the number of resource elements allocated for data transmission, the spatial multiplexing order, and the maximum FEC block size are denoted by Nmod, RFEC, NRE, NSM, NFB_MAX, respectively.

Nmod 	is the modulation order of the burst transmission, which is specified by MCS index

RFEC 	is the nominal code rate of the burst transmission, which is specified by MCS index

NRE 		is the number of resource elements allocated for burst transmission. The calculation of NRE shall exclude the resource elements used by pilot channels

NSM		is the spatial multiplexing order of the resource elements allocated for burst transmission. The value of NSM depends on the MIMO transmission mode and is defined in section x.x.x.x.

NFB_MAX	is the maximum FEC block size, which equals to 4800 bits.

- Table 741─Burst size

| idx | NDB (byte) | KFB | idx | NDB (byte) | KFB | idx | NDB (bytes) | KFB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 6 | 1 | 23 | 90 | 1 | 45 | 1200 | 2 |
| 2 | 8 | 1 | 24 | 100 | 1 | 46 | 1416 | 3 |
| 3 | 9 | 1 | 25 | 114 | 1 | 47 | 1584 | 3 |
| 4 | 10 | 1 | 26 | 128 | 1 | 48 | 1800 | 3 |
| 5 | 11 | 1 | 27 | 145 | 1 | 49 | 1888 | 4 |
| 6 | 12 | 1 | 28 | 164 | 1 | 50 | 2112 | 4 |
| 7 | 13 | 1 | 29 | 181 | 1 | 51 | 2400 | 4 |
| 8 | 15 | 1 | 30 | 205 | 1 | 52 | 2640 | 5 |
| 9 | 17 | 1 | 31 | 233 | 1 | 53 | 3000 | 5 |
| 10 | 19 | 1 | 32 | 262 | 1 | 54 | 3600 | 6 |
| 11 | 22 | 1 | 33 | 291 | 1 | 55 | 4200 | 7 |
| 12 | 25 | 1 | 34 | 328 | 1 | 56 | 4800 | 8 |
| 13 | 27 | 1 | 35 | 368 | 1 | 57 | 5400 | 9 |
| 14 | 31 | 1 | 36 | 416 | 1 | 58 | 6000 | 10 |
| 15 | 36 | 1 | 37 | 472 | 1 | 59 | 6600 | 11 |
| 16 | 40 | 1 | 38 | 528 | 1 | 60 | 7200 | 12 |
| 17 | 44 | 1 | 39 | 600 | 1 | 61 | 7800 | 13 |
| 18 | 50 | 1 | 40 | 656 | 2 | 62 | 8400 | 14 |
| 19 | 57 | 1 | 41 | 736 | 2 | 63 | 9600 | 16 |
| 20 | 64 | 1 | 42 | 832 | 2 | 64 | 10800 | 18 |
| 21 | 71 | 1 | 43 | 944 | 2 | 65 | 12000 | 20 |
| 22 | 80 | 1 | 44 | 1056 | 2 | 66 | 14400 | 24 |

The burst size index is calculated as , where  is a 5 bits index in A-MAP IE, and  is calculated based on Table 742 and the allocation size, as shown in Table 742. The allocation size is defined as the number of LRUs multiplied by the MIMO rank that are allocated for the burst. In the case of long TTI, the number of LRUs is calculated as the number of subframes in TTI multiplied by the number of LRUs per subframe. For allocaton size 185~192, ISizeOffset = 31 is invalid.

<Replace Table 742 by following table>

- Table 742─Minimal size index as a function of the number of allocated LRUs

| - Allocation size | - IMinimalSize | - Allocation size | - IMinimalSize | - Allocation size | - IMinimalSize |
| --- | --- | --- | --- | --- | --- |
| - 1 ~ 3 | - 1 | - 16 ~ 18 | - 15 | - 58 ~ 64 | - 26 |
| - 4 | - 2 | - 19 ~ 20 | - 16 | - 65 ~ 72 | - 27 |
| - 5 | - 4 | - 21 ~ 22 | - 17 | - 73 ~ 82 | - 28 |
| - 6 | - 6 | - 23 ~ 25 | - 18 | - 83 ~ 90 | - 29 |
| - 7 | - 8 | - 26 ~ 28 | - 19 | - 91 ~ 102 | - 30 |
| - 8 | - 9 | - 29 ~ 32 | - 20 | - 103 ~ 116 | - 31 |
| - 9 | - 10 | - 33 ~ 35 | - 21 | - 117 ~ 131 | - 32 |
| - 10 ~ 11 | - 11 | - 36 ~ 40 | - 22 | - 132 ~ 145 | - 33 |
| - 12 | - 12 | - 41 ~ 45 | - 23 | - 146 ~ 164 | - 34 |
| - 13 | - 13 | - 46 ~ 50 | - 24 | - 165 ~ 184 | - 35 |
| - 14 ~ 15 | - 14 | - 51 ~ 57 | - 25 | - 185 ~ 192 | - 36 |

The modulation order Nmod (2 for QPSK, 4 for 16-QAM and 6 for 64-QAM) depends on the parameter  according to Table 743. Allocation size of 1 or 2 LRUs are special cases (separated columns in the table). For allocation of at least 3 LRUs the modulation order depends only on . The allocation size and the value of ISizeOffset are set by the ABS scheduler, which takes into account the resulting modulation order and effective code rate, and according to the link adaptation.

<Replace Table 743 by following table>

Table 743─Rule for modulation order

|  | Nmod (allocation size ≥ 3) | Nmod (allocation size = 2) | Nmod (allocation size = 1) |
| --- | --- | --- | --- |
| 0 ~ 9 | 2 | 2 | 2 |
| 10 ~ 15 | 2 | 2 | 4 |
| 16 ~18 | 2 | 4 | 6 |
| 19 ~ 21 | 4 | 4 | 6 |
| 22 ~ 23 | 4 | 6 | 6 |
| 24 ~ 31 | 6 | 6 | 6 |

- The nominal MCS used rank-1 CQI feedback shall be selected from Table 744

Table 744─ MCS table for rank-1 CQI

| MCS index | Modulation | Code rate |
| --- | --- | --- |
| ‘0000’ | QPSK | 31/256 |
| ‘0001’ | QPSK | 48/256 |
| ‘0010’ | QPSK | 71/256 |
| ‘0011’ | QPSK | 101/256 |
| ‘0100’ | QPSK | 135/256 |
| ‘0101’ | QPSK | 171/256 |
| ‘0110’ | 16QAM | 102/256 |
| ‘0111’ | 16QAM | 128/256 |
| ‘1000’ | 16QAM | 155/256 |
| ‘1001’ | 16QAM | 185/256 |
| ‘1010’ | 64QAM | 135/256 |
| ‘1011’ | 64QAM | 157/256 |
| ‘1100’ | 64QAM | 181/256 |
| ‘1101’ | 64QAM | 205/256 |
| ‘1110’ | 64QAM | 225/256 |
| ‘1111’ | 64QAM | 237/256 |

If a burst is partitioned into more than one FEC block, each partitioned FEC block has same size. The size of the kth FEC blockencoder input is denoted by NFB, k, k = 0, 1, …, KFB-1. The set of supported FEC blockencoder input sizes including FEC block CRC when applicable is shown in Table 745. the subset of the burst size table, i.e., NDB of idx from 1 to 39 in Table 741.

<Remove Table 745 >

The burst size NDB including burst CRC and FEC block CRC is defined by Equation (253):

NDB=k=0KFB−1NFB,  k.

NDB = KFB × NFB 										(253)

The payload size excluding burst CRC and FEC block CRC is defined by Equation (254):

NPL=NDB−NDB−CRC−IMFB⋅KFB⋅NFB−CRC							(254)

where,

IMFB		equals 0 when KFB =1, 1 when KFB >1,
NFB-CRC	equals 16, which is the size of FEC block CRC,
NDB-CRC	equals 16, which is the size of burst CRC.

The burst partition block generates KFB FEC blocks, with each FEC block processed by the FEC block CRC encoding block as described in 15.3.12.1.4.

- 15.3.12.1.4 FEC block CRC encoding

The burst partition procedure generates KFB FEC blocks for each burst. If KFB > 1, the FEC block CRC generator appends a 16-bit FEC block CRC for each FEC block. The cyclic generator for FEC block CRC encoding is shown in Equation (255):

gFB−CRC(D)=D16+D15+D2+1								(255)

Denote the bits of the k-th input FEC blockencoder input by d1, d2 ,d3, ⋯, dNFB,  k−16d1, d2 ,d3, ⋯, dNFBwith d1d1 being the MSB and NFB, k being the size of the k-th input FEC blockencoder input, including the 16-bit FEC block CRC. Denote the parity bits produced by the burstFEC block CRC generator by p1, p2 ,p3, ⋯, p16. The burstFEC block CRC encoding is performed in a systematic form, which means that in GF(2), the polynomial in Equation (256):

d1DNFB, k−1+ d2DNFB, k−2+ ⋯ +dNFB, k−16D16+p1D15+p2D14+⋯+p15D1+p16

d1DNFB−1+ d2DNFB−2+ ⋯ +dNFB−16D16+p1D15+p2D14+⋯+p15D1+p16			(256)

yields a remainder equal to 0 when divided by gFB-CRC(D).

- 15.3.12.1.5 FEC encoding

Each FEC block shall be encoded using the convolutional turbo codes specified in section 15.3.12.1.5.1.

- 15.3.12.1.5.1 Convolutional turbo codes

CTC encoder

The CTC encoder, including its constituent encoder, is depicted in Figure 517.

It uses a double binary CRSC (Circular Recursive Systematic Convolutional) code.

The bits of the data to be encoded are alternatively fed to A and B, starting with the MSB of the first byte being fed to A, followed by the next bit being fed to B. The encoder is fed by blocks of NEPNFB bits or N couples (NFB=2N bits).

The polynomials defining the connections are described in octal and symbol notations as follows:

- For the feedback branch: 13, equivalently 1+D+D3 (in octal notation)
- For the Y parity bit: 15, equivalently 1+D2+D3
- For the W parity bit: 11, equivalently 1+D3

Figure 517─CTC encoder

First, the encoder (after initialization by the circulation state SC1, see <<15.13.1.6.1.38.4.9.2.3.3>>Error: Reference source not found) is fed the sequence in the natural order (switch 1 in Figure 517) with incremental address i=1,2,⋯,N. This first encoding is called C1 encoding.

Then the encoder (after initialization by the circulation state SC2, see <<15.13.1.6.1.38.4.9.2.3.3>>) is fed by the interleaved sequence (switch 2 in Figure 517) with incremental address i=1,2,⋯,N. This second encoding is called C2 encoding.

The order in which the encoded bits shall be fed into the bit separation block <<(15.x.1.6.1.4)>>(<<8.4.9.2.3.4>>) is:

CTC interleaver

The CTC interleaver requires the parameters P0, P1, P2, and P3 shown in Table 746 of which NEP set corresponds to that in Table 745.

The detailed interleaver structures except table for interleaver parameters correspond to <<8.4.9.2.3.2>>.

<Replace Table 746 by following table>

- Table 746─Interleaver Parameters

| Index | NFB | P0 | P1 | P2 | P3 | Index | NFB | P0 | P1 | P2 | P3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 48 | 5 | 0 | 0 | 0 | 21 | 568 | 19 | 102 | 140 | 226 |
| 2 | 64 | 11 | 12 | 0 | 12 | 22 | 640 | 23 | 84 | 296 | 236 |
| 3 | 72 | 11 | 18 | 0 | 18 | 23 | 720 | 23 | 130 | 156 | 238 |
| 4 | 80 | 7 | 4 | 32 | 36 | 24 | 800 | 23 | 150 | 216 | 150 |
| 5 | 88 | 13 | 36 | 36 | 32 | 25 | 912 | 29 | 14 | 264 | 94 |
| 6 | 96 | 13 | 24 | 0 | 24 | 26 | 1024 | 29 | 320 | 236 | 324 |
| 7 | 104 | 7 | 4 | 8 | 48 | 27 | 1160 | 31 | 314 | 348 | 222 |
| 8 | 120 | 11 | 30 | 0 | 34 | 28 | 1312 | 31 | 214 | 160 | 506 |
| 9 | 136 | 13 | 58 | 4 | 58 | 29 | 1448 | 33 | 254 | 372 | 158 |
| 10 | 152 | 11 | 38 | 12 | 74 | 30 | 1640 | 33 | 164 | 432 | 748 |
| 11 | 176 | 17 | 52 | 68 | 32 | 31 | 1864 | 33 | 504 | 444 | 664 |
| 12 | 200 | 11 | 76 | 0 | 24 | 32 | 2096 | 39 | 400 | 688 | 68 |
| 13 | 216 | 11 | 54 | 56 | 2 | 33 | 2328 | 41 | 254 | 884 | 1054 |
| 14 | 248 | 13 | 6 | 84 | 46 | 34 | 2624 | 47 | 378 | 1092 | 1250 |
| 15 | 288 | 17 | 74 | 72 | 2 | 35 | 2944 | 41 | 338 | 660 | 646 |
| 16 | 320 | 17 | 84 | 108 | 132 | 36 | 3328 | 37 | 258 | 28 | 1522 |
| 17 | 352 | 17 | 106 | 56 | 50 | 37 | 3776 | 53 | 772 | 256 | 408 |
| 18 | 400 | 19 | 142 | 0 | 142 | 38 | 4224 | 59 | 14 | 668 | 1474 |
| 19 | 456 | 17 | 184 | 0 | 48 | 39 | 4800 | 53 | 66 | 24 | 2 |
| 20 | 512 | 19 | 64 | 52 | 124 |  |  |  |  |  |  |

Determination of CTC circulation states

Correspond to <<8.4.9.2.3.3>>.

Bit separation

Correspond to <<8.4.9.2.3.4.1>>.

Subblock interleaving

The subblock interleaver requires the parameters m and J shown in Table 747 of which NEP set corresponds to that in Table 745.

The detailed subblock interleaver structures except table for subblock interleaver parameters correspond to <<8.4.9.2.3.4.2>>.

<Replace Table 747 by following table>

- Table 747─Parameters for the subblock interleavers

| Index | NFB | m | J | Index | NFB | m | J | Index | NFB | m | J | Index | NFB | m | J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 48 | 3 | 3 | 11 | 176 | 5 | 3 | 21 | 568 | 7 | 3 | 31 | 1864 | 9 | 2 |
| 2 | 64 | 4 | 2 | 12 | 200 | 5 | 4 | 22 | 640 | 7 | 3 | 32 | 2096 | 9 | 3 |
| 3 | 72 | 4 | 3 | 13 | 216 | 5 | 4 | 23 | 720 | 7 | 3 | 33 | 2328 | 9 | 3 |
| 4 | 80 | 4 | 3 | 14 | 248 | 6 | 2 | 24 | 800 | 7 | 4 | 34 | 2624 | 9 | 3 |
| 5 | 88 | 4 | 3 | 15 | 288 | 6 | 3 | 25 | 912 | 8 | 2 | 35 | 2944 | 9 | 3 |
| 6 | 96 | 4 | 3 | 16 | 320 | 6 | 3 | 26 | 1024 | 8 | 2 | 36 | 3328 | 9 | 4 |
| 7 | 104 | 4 | 4 | 17 | 352 | 6 | 3 | 27 | 1160 | 8 | 3 | 37 | 3776 | 10 | 2 |
| 8 | 120 | 5 | 2 | 18 | 400 | 6 | 4 | 28 | 1312 | 8 | 3 | 38 | 4224 | 10 | 3 |
| 9 | 136 | 5 | 3 | 19 | 456 | 7 | 2 | 29 | 1448 | 8 | 3 | 39 | 4800 | 10 | 3 |
| 10 | 152 | 5 | 3 | 20 | 512 | 7 | 2 | 30 | 1640 | 8 | 4 |  |  |  |  |

Bit grouping

The interleaved subblocks shall be multiplexed into four blocks; those four blocks consist of an interleaved A subblock, an interleaved B subblock, a bit-by-bit multiplexed sequence of the interleaved Y1 and Y2 subblock sequences, which is referred to Y, and a bit-by-bit multiplexed sequence of the interleaved W2 and W1 subblock sequences, which is referred to W. Information subblocks, A and B, are by-passed while parity subblocks are multiplexed bit by bit. The bit-by-bit multiplexed sequence of interleaved Y1 and Y2 subblock sequences shall consist of the first output bit from the Y1 subblock interleaver, the first output bit from the Y2 subblock interleaver, the second output bit from the Y1 subblock interleaver, the second output bit from the Y2 subblock interleaver, etc. The bit-by-bit multiplexed sequence of interleaved W2 and W1 subblock sequences shall consist of the first output bit from the W2 subblock interleaver, the first output bit from the W1 subblock interleaver, the second output bit from the W2 subblock interleaver, the second output bit from the W1 subblock interleaver, etc.

After multiplexing subblocks into four blocks, Subblock B and Subblock W are circularly left-shifted by k bits. Subblock Y is circularly left-shifted by 1 bit. When the FEC block size Nep NFB is equal to multiple of the modulation order, k is set as 1. Otherwise, let k be 0. Figure 518 shows interleaving scheme as explained above.

<Replace Figure 518 by following figure>

Figure 518─Block diagram of interleaving scheme

Resource segmentation

If KFB > 1, the NRE data resource elements allocated for the subpacket are segmented into KFB blocks, one for each FEC block. The number of data resource elements for the kth FEC block is defined by Equation (257).

NRE, k=2×NRE2+(KFB−k−1)KFB,0≤k<KFB					(257)

Bit selection and repetition

Bit selection and repetition are performed to generate the subpacket.

Let NCTC, k be the number of coded bits that shall be transmitted for the kth FEC block. The value of NCTC, k is calculated by the Equation (258):

(258)

where NSM is equal to the product of MIMO rank that are allocated for the burst and the number of subframes when long TTI is used for the burst.

The index in the HARQ buffer for the jth bit transmitted for the kth FEC block, uk, j, i, shall be:

for k = 0, 1, …, KFB – 1, and j = 0, 1, …, NCTC, k – 1, where i is the subpacket ID of the subpacket (SPID = i), Pi, k is the starting position for subpacket i of the kth FEC block as specified in 15.3.12.2.1, and  is the buffer size for the kth FEC block.

Bit collection

The selected bits from each FEC block are collected in the order of FEC block for the HARQ transmission.

Modulation

Correspond to <<8.4.9.4.2>>.

---------------------------------------------------  Text End  ---------------------------------------------------

---------------------------------------------------  Text Start  ---------------------------------------------------

<Insert the new subsection 15.3.12.2 as follows:>

- 15.3.12.2 Channel coding for control channel

Tail-biting convolutional codes (TBCC) are used for most of uplink and downlink control channels. The structure of TBCC encoder is depicted in Figure 519. The output encoded bits of rate-1/5 TBCC encoder are separated to five subblocks and each subblock goes through its subblock interleaver. In bit selection block, rate-matching mechanism is implemented by puncture or repetition operation.

Figure 519─Block diagram of TBCC structure

- 15.3.12.2.1 TBCC encoder

The binary TBCC shall have mother code rate of 1/5, a constraint length equal to K=7, and shall use the following generator polynomials to derive its five code bits:

G1=171OCT FOR A

G2=133OCT FOR B

G3=165OCT FOR C

G4=117OCT FOR D

G5=127OCT FOR E

The TBCC encoder using above generator polynomials is depicted in Figure 520.

Figure 520─TBCC encoder of rate 1/5

- 15.3.12.2.2 Bit separation

All of the encoded bits shall be demultiplexed into five subblocks denoted A, B, C, D, and E. Suppose L information bits are input to the encoder. The encoded bits u[i]with i=0,⋯,5L−1shall be distributed into five subblocks withyAk=u5k,yBk=u5k+1, yCk=u5k+2, yDk=u5k+3, and yEk=u5k+4 where k=0,⋯,L−1.

- 15.3.12.2.3 Subblock interleaver

The five subblocks shall be interleaved separately. The interleaving is performed by the unit of bits. The output of the subblock interleaver zjk with j=A,B,C,D,E and k=0,⋯,L−1 is related to the input sequenceyjk. It can be expressed as zj[k]=yjΠk where Πk is the interleaver index. It can be calculated by the following four steps

a) Initialize x and k to 0.

b) Form a tentative output address Tx according to the following formula:

Tx=15x+1+32x+12 mod128

c) If Tx is less than L, Πk=Tx and increment x and k by 1. Otherwise, discard Tx and increment x only.

d) Repeat steps b) and c) until all L interleaver output addresses are obtained.

The maximum value of L must be not greater than 128.

- 15.3.12.2.4 Bit grouping

The output sequence of subblock interleaver shall consist of the interleaved A, B, C, D, and Esubblock sequences. Let the bit grouping output denoted by w[i] with i=0,⋯,5L−1. It can be expressed as wk=zAk,wk+L=zBk,wk+2L=zCk,wk+3L=zDk, and wk+4L=zEk with k=0,⋯,L−1.

- 15.3.12.2.5 Bit selection

Suppose the desired number of the output bits is M. The output sequence cn can be expressed as cn=wnmodKbufsize, n=0,1,⋯,M−1

where Kbufsize≤5L is the size of buffer used for repetition.

---------------------------------------------------  Text End  ---------------------------------------------------

---------------------------------------------------  Text Start  ---------------------------------------------------

<Modify subsection 15.3.12.2 as follows:>

- 15.3.12.23 HARQ

- 15.3.12.23.1 IR HARQ

HARQ IR is performed by changing the starting position, Pi, k, of the bit selection for HARQ retransmissions.

For downlink HARQ, the starting point for the bit selection algorithm as described in section 15.3.12.1.5.1 is determined as a function of SPID using Table 748.

Table 748─Starting position determination for downlink HARQ

| SPID | Starting position Pi,k |
| --- | --- |
| 0 | 0 |
| 1 | −NCTC,kmod NFBBuffer,k |
| 2 | NFBBuffer,k2−NCTC,k2mod NFBBuffer,k |
| 3 | NFBBuffer,k−NCTC,k2mod NFBBuffer,k |

For uplink HARQ, the starting position for the bit selection algorithm as described in section 15.3.12.1.5.1 is determined as a function of SPID for Equation (259).

.								(259)

For uplink HARQ, subpackets shall be transmitted in sequential order. In other words, for the tth transmission, the subpacket ID shall be set to SPID = t mod 4.

- 15.3.12.23.1.12 Constellation rearrangement

Two constellation re-arrangement versions shall be supported. Constellation rearrangement only applies to 16QAM and 64QAM. In case of QPSK, it is transparent.

<<table>>Table 749 and Table 750 describes the operations that produce different CoRe versions for 16QAM and 64QAM respectively, so that the four bits in a 16QAM symbol (the six bits in a 64 QAM) are of the same resilience. In other words, the two bits of higher quality at CoRe-version 0 are of lower quality at CoRe-version 1 while the two bits of lower quality at CoRe-version 0 are of higher quality at CoRe-version 1.

Table 749─Constellation rearrangement version (rank =1)

| Constellation | Ncbps | CRV | Mapping rule |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 QAM | 4 | 0 | b0 | b1 | b2 | b3 | - | - |
| 16 QAM | 4 | 1 | b3 | b2 | b1 | b0 | - | - |
| 64 QAM | 6 | 0 | b0 | b1 | b2 | b3 | b4 | b5 |
| 64 QAM | 6 | 1 | b5 | b4 | b3 | b2 | b1 | b0 |

Table 750─Constellation rearrangement version (rank =≥ 2)

| Constellation | Ncbps | CRV | Mapping rule |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | First symbol |  |  |  |  |  | Second symbol |  |  |  |  |  |
| 16 QAM | 4 | 0 | b0 | b1 | b2 | b3 | - | - | b4 | b5 | b6 | b7 | - | - |
| 16 QAM | 4 | 1 | b1 | b4 | b3 | b6 | - | - | b5 | b0 | b7 | b2 | - | - |
| 64 QAM | 6 | 0 | b0 | b1 | b2 | b3 | b4 | b5 | b6 | b7 | b8 | b9 | b10 | b11 |
| 64 QAM | 6 | 1 | b2 | b7 | b0 | b5 | b10 | b3 | b8 | b1 | b6 | b11 | b4 | b9 |

In DL HARQ operation, CRV is signaled for each subpacket to an AMS as a starting value for CRV. In UL HARQ operation, the information on CRVs is implicitly known at AMS and ABS. The CRV shall be changed to the next version of CoRe in DL and UL HARQ operation whenever the transmitted bits wraparound at the end of circular buffer.

---------------------------------------------------  Text End  ---------------------------------------------------

---------------------------------------------------  Text Start  ---------------------------------------------------

<Modify subsection 15.3.13.1 as follows:>

15.3.13.1 DL Link Adaptation

The serving ABS may adapt the modulation and coding scheme (MCS) level based on the channel quality indicator (CQI) and/or HARQ ACK/NACK reported by the AMS.

The serving ABS shall adapt the MIMO mode, according to CQI reports from the AMS and considering system parameters, such as: traffic load, number of users, ACK/NACK, CQI variation, preferred MFM etc.

AMS shall measure the DL channel quality and report back to ABS. The exact measurement method used to derive the CQI feedback is implementation specific.

- The nominal MCS for rank-1 CQI feedback shall be selected from Table 744

Table 744─MCS table for rank-1 CQI

| MCS index | Modulation | Code rate |
| --- | --- | --- |
| ‘0000’ | QPSK | 31/256 |
| ‘0001’ | QPSK | 48/256 |
| ‘0010’ | QPSK | 71/256 |
| ‘0011’ | QPSK | 101/256 |
| ‘0100’ | QPSK | 135/256 |
| ‘0101’ | QPSK | 171/256 |
| ‘0110’ | 16QAM | 102/256 |
| ‘0111’ | 16QAM | 128/256 |
| ‘1000’ | 16QAM | 155/256 |
| ‘1001’ | 16QAM | 184/256 |
| ‘1010’ | 64QAM | 135/256 |
| ‘1011’ | 64QAM | 157/256 |
| ‘1100’ | 64QAM | 181/256 |
| ‘1101’ | 64QAM | 205/256 |
| ‘1110’ | 64QAM | 225/256 |
| ‘1111’ | 64QAM | 237/256 |

---------------------------------------------------  Text End  ---------------------------------------------------