| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Signaling Header Type I and Bandwidth Request Header Format |  |
| Date Submitted | 2009-07-13 |  |
| Source(s) | Joey Chou, Shantidev Mohanty, Muthaiah Venkatachalam, Xiangying Yang, Jose Puthenkulam, Sassan Ahmadi, Jie Hui, Intel YihShen Chen, MediaTek Frank Ren, ITRI | E-mail: joey.chou@intel.com E-mail: Jie.hui@intel.com E-mail: yihshen.chen@mediatek.com E-mail: frank_ren@ITRI.ORG.TW |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text for MAC signaling header and BW-REQ header. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Signaling Header Type I and Bandwidth Request Header Format

Joey Chou, Muthaiah Venkatachalam, Sassan Ahmadi, Xiangying Yang, Jose Puthenkulam, Jie Hui, Shantidev Mohanty, Intel

YihShen Chen, MediaTek

Frank Ren, ITRI

- Introduction

Table 1 summarizes the MAC signaling header type I/II, and UL/DL extended header messages being defined in IEEE 802.16-2009.

|  | Signaling Messages | Sizes |  | Signaling Messages | Sizes |
| --- | --- | --- | --- | --- | --- |
| MAC Signaling Type I | BR incremental | 19 bits | MAC Signaling Type II | Combined DL average CINR | 5 bits |
|  | BR aggregate | 19 bits |  | MIMO mode condition feedback | 32 bits |
|  | PHY channel report | 16 bits |  | CINR feedback | 16 bits |
|  | BR with UL Tx power report | 19 bits |  | Close-loop MIMO feedback | 26 bits |
|  | BR and CINR report | 19 bits | DL EH | SDU SN | 8 bits |
|  | BR with UL sleep control | 19 bits |  | DL sleep control | 24 bits |
|  | SN Report | 11 bits |  | Feedback request | 24 bits |
|  | CQICH allocation request | 7 bits |  | SN request | 8 bits |
| MAC Signaling Type II | CQI & MIMO feedback | 9 bits |  | PDU SN (short) | 8 bits |
|  | DL average CINR | 5 bits |  | PDU SN (long) | 16 bits |
|  | MIMO Coeff feedback | 9 bits | UL EH | MIMO mode feedback | 8 bits |
|  | Preferred DL Channel | 8 bits |  | UL Tx Power report | 8 bits |
|  | UL Tx Power | 8 bits |  | Mini feedback | 16 bits |
|  | PHY Channel feedback | 18 bits |  | PDU SN (short) | 8 bits |
|  | AMC band indication bitmap | 32 bits |  | PDU SN (long) | 16 bits |
|  | Short-term precoding feedback | 32 bits |  | Persistent allocation error event | 8 bits |
|  | Multiple types of feedback | 32 bits |  | ertPS reassumption bitmap | 8 bits |
|  | Long-term precoding feedback | 14 bits |  |  |  |

Table 1: IEEE 802.16-2009 Signaling Messages

The signaling messages range from 5 bits to 32 bits in size, and are not piggybacked on the data payload. Therefore, the 11 bit length field in GMH becomes huge overhead if GMH is used to send signaling messages.

This contribution proposes type I and type II MAC signaling headers to send signaling messages.

Type I signaling header

Figure 1 shows the Type I MAC signaling header to be used to support UD/DL signaling messages that are up to 16 bits long and are not piggybacked on the data payload. It can be identified by the dedicated flow ID. It contains fixed size signaling data (i.e. 24 bits); therefore length field can be eliminated for better efficiency.

Flow ID = xxxx (4)

Signaling Data (8)

Signaling Data (8)

Signaling Type (4)

1

2

Signaling Data (8)

3

Figure 1: Type I Signaling Header

- Flow ID (4): dedicated flow, e.g. 0011

- Signaling type (4): support up to 16 signaling types

- Signaling data (24): signaling body, 24 bits

- Proposed Text

15.2.3.2.4  Signaling Header

Signaling header shall be used to send signaling messages, and consists of type I and type II signaling headers.

15.2.3.2.4.1 Type I signaling header

Figure 28 shows the Type I signaling header to be used to support UD/DL signaling messages that are up to 16 bits long and are not piggybacked on the data payload. It can be identified by the dedicated flow ID). It contains fixed size signaling data (i.e. 24 bits); therefore length field can be eliminated for better efficiency.

Table 28: Type I Signaling Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Type I Signaling Header () { | - |  |
| Flow ID | 4 | Flow identifier |
| Signaling Type | 4 | Signaling indicator |
| Signaling Data | 24 | Signaling data payload |
| } | - |  |

Table 29: Type I Signaling Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| Flow ID | 4 | This field indicates the service flow that has been addressed |
| Signaling Type | 4 | Indicates the signaling type |
| Signaling Data | 24 | Signaling data payload |

15.2.3.2.4.2 Bandwidth Request Signaling Headers

Table 30 defines the bandwidth request signaling header types.

Table 30: Bandwidth Request Signaling Type Field Encoding

| Sigaling Type Field (4bits) | MAC Signaling Header Type |
| --- | --- |
| 0000 | BR with STID |
| 0001 | BR without STID |

15.2.3.2.4.2.1 BR with STID

To send the signaling header through the UL resource assigned anonymously (such as in bandwidth request procedure step 3), STID shall be included in the BR header. Detailed contents for BR Header format are illustrated in Table 31.

Table 31: BR with STID Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| BR with STID () { | - |  |
| STID | 12 | Station identifier of AMS which requests UL bandwidth. |
| BR Flow ID | 4 | Flow ID of the connection requesting for bandwidth |
| Bandwidth Request Size | 8 | Aggregated bandwidth request |
| } | - |  |

15.2.3.2.4.2.2 BR without STID

BR Header without STID is sent through UL resource specifically allocated to the AMS. Without STID, BR type shall be included to indicate whether BR type is increment or aggregate.

Table 32: BR with STID header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| BR with STID () { | - |  |
| BR Type | 1 | 0 : aggregate BR; 1 : incremental BR |
| BR Flow ID | 4 | Flow ID of the connection requesting for bandwidth |
| Bandwidth Request Size | 11 | Aggregated bandwidth request |
| GPI change indicator | 1 | 1: If (scheduling type == aGPS) indicating the request to switch between primary and secondary service flow QoS parameter set, and Running GPI field is not applicable. 0: If (scheduling type == aGPS or BE) indicating the Running GPI field is valid. |
| If GPI change indicator = 1 { | - |  |
| Running Grant Polling Interval (GPI) | 6 | When GPI change indicator = 0 If (scheduling type == aGPS) Indicating new GPI (frames) to use for future allocation If (scheduling type == BE) Indicating minimum delay of the requested grant |
| }else { | - |  |
| Reserved | 6 |  |
| } | - |  |
| Reserved | 1 |  |
| } | - |  |