| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | CR on SDD Chapter 15: Light-Weighted Femtocell BS Discovery |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Feng Xie Zhaohua Lu Yang Liu Hongyun Qu Kaiying Lv ZTE Corporation | Xie.feng2@zte.com.cn Lu.zhaohua@zte.com.cn Liu.yang8@zte.com.cn |
| Re: | Call for reply comment on SDD IEEE 802.16-08/003r9a |  |
| Abstract | This contribution provides text on Femtocell BS Discovery. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Light-Weighted Femtocell BS Discovery

- Feng Xie, Zhaohua Lu, Yang Liu, Hongyun Qu, Kaiying Lv

- ZTE Corporation

- Introduction

A Femtocell BS is a BS with low transmit power, typically installed by a subscriber in home or SOHO to provide the access to closed or open group of users as configured by the subscriber and/or the access provider. A Femtocell BS is connected to the service provider’s network via broadband (such as DSL, or cable).

In current SRD 80216m-07_002r8.pdf, “The air interface shall allow dense deployment of large number of femto cells by an operator.” Moreover, femtocell BS can be deployed with or without overlay macro BS. Considering both of the cases, a general light-Weighted approach is necessary to facilitate MS’s femtocell BS discovery.

- Light-Weighted Femtocell BS Discovery

During network entry or re-entry, an MS detects neighboring femtocell BSs based on scanning mechanism with assistance of other BSs. Macro BS and femtocell BSs provide the following information to facilitate MS scanning for femtocell BS.

- Femtocell Shared Information Provisioning

Macro BS and femtocell BS may broadcast indications of one or more carrier frequencies used for dedicated femtocell BS deployment.

Macro BS and femtocell BS may broadcast indications of CELL ID ranges used for dedicated femtocell BS deployment. Macro BS and femtocell BS may broadcast indications of CELL ID ranges used for dedicated CSG-Closed/CSG-Open/Open femtocell BS deployment.

- Femtocell Specific Information Provisioning

Femtocell BS broadcast a neighbor femtocell list containing CSG-Closed, CSG-Open, and Open femtocell BS. A femtocell BS broadcasts system information of neighboring CSG-Open and Open femtocell BS. A femtocell YBS may broadcast system information of neighboring CSG-Closed femtocell BS.

Macro and femtocell BS may support femtocell specific information query by MS. The femtocell specific information in response may be optimized based on available information of the MS in query (e.g. the location of the MS, the MS access control list). The response message to an MS shall include a list of the accessible femtocell BS. The response message may include the system information of femtocell BS.

- Add the following text in the beginning of 15.5.2 (IEEE 802.16m-08/003r9a):

--------------------------------------------- Proposed text -----------------------------------------------------------------

15.5.2 Femtocell BS Detection

An MS can perform initial scanning for femtocell BS(s) with possible frequencies to identify one or more surrounding femtocell BS(s). The MS can report detected femtocell BS list to the serving macro BS to obtain the Femtocell BS list in the neighborhood of the MS. Based on the MS’s report, the macro BS provides the MS with an optimized femtocell BS(s) neighbor list which contains information of accessible femtocell BS(s) in the vicinity of MS. The MS can also obtain the neighbor femtocell BS list from the NBR_ADV message of the detected femtocell BS. Then the MS performs the second step scanning with the received neighbor list for access or handover decision.

--------------------------------------------- End of Proposed text ----------------------------------------------------------