| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for PA release in IEEE802.16m Amendment |  |
| Date Submitted | 2009-07-07 |  |
| Source(s) | Isamu Yoshii Takaaki Kishigami Panasonic Corporation | Voice: + 81-46-840-5123 E-mail: yoshii.isamu@jp.panasonic.com kishigami.takaaki@jp.panasonic.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6. DL Control Structure Comments on AWD 15.3.6.5.2.6 |  |
| Abstract | To add PA release detail description. |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for PA release in IEEE802.16m Amendment

Isamu YOSHII, Takaaki KISHIGAMI

Panasonic Corporation

# Introduction

15.3.6.5.2.6 for DL PA A-MAP IE shows deallocation indication. However, there is not clear description for PA release. We would like to add the explanation of PA release in Table 676.

# Proposed Change Text in 15.3.6.5.2.6

============================== Start of Proposed Text =================================

Table 676—DL Individual Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Persistent A-MAP_IE() { | - | - |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| If (Allocation Period==0b00){ |  |  |
| Resource Allocation | Variable | Variable number of bits - depends on system bandwidth. Information may include: • Type of resource unit (DRU/CRU) • Location (start/end) • Allocation size When allocation size is “0”, it means PA is release or deactivate. |
| HFA | [4] | TBD HARQ Feedback Allocation |
| } else if (Allocation Period != 0b00){ |  |  |
| MCS | 4 | Depends on supported modes, 16 modes assumed as baseline |

============================ End of Proposed Text ===================================