| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of the DL A-MAP IE in MIMO modes in IEEE802.16m Amendment |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Takaaki KISHIGAMI, Isamu YOSHII Panasonic Corporation | Voice: + 81-46-840-5123 E-mail: kishigami.takaaki@jp.panasonic.com yoshii.isamu@jp.panasonic.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 “Comments on AWD 15.3.6.5.2.2 DL basic assignment A-MAP IE” |  |
| Abstract | We would like to clarify the definitions of PSI and Mt in the DL Basic A-A-MAP IE. |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of the DL A-MAP IE in MIMO modes in IEEE802.16m Amendment

Takaaki KISHIGAMI, Isamu YOSHII

Panasonic Corporation

# Introduction

The DL assignment A-MAP information contents are defined in Section 15.3.6.5.2.2 of AWD [1].

However, we consider the definitions of PSI and Mt still remain ambiguous.

In this contribution, we would like to clarify the definitions of PSI and Mt in the DL Basic A-A-MAP IE.

# References

[1]IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document (AWD)”

# Proposed Text

We propose to add the following text for the IEEE802.16m Amendment Working Document in Section 15.3.6.5.2.2.

-------------------------------  Start text proposal  ---------------------------------------------------

- 15.3.6.5.2.2 DL basic assignment A-MAP IE

Section 15.3.6.5.2.2 DL basic assignment A-MAP IE, page 200, line 58

- modify the following text

“PSI: Allocated pilot stream index for horizontal encoding.”

to

“PSI: Allocated pilot stream index for horizontal encoding. The range of pilot stream index depends on the number of Mt. The pilot stream index is less than or equal to Mt, or 1<= PSI <= Mt .”

Section 15.3.6.5.2.2 DL basic assignment A-MAP IE, page 200, line 60

- modify the following text

“Mt: Number of streams in transmission. The DL pilot pattern with Mt streams shall be used in the allocated resource.”

to

“Mt: Number of streams in transmission. The DL pilot pattern with Mt streams shall be used in the allocated resource. In MU-MIMO mode, the number of streams is, Mt <= min(Nt, Min_Nr), where Min_Nr is the minimum value of receive antennas of the co-scheduled AMSs in MU-MIMO mode.”

-------------------------------  End text proposal  ---------------------------------------------------