| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for DL A-MAP IE for MU-MIMO mode in IEEE802.16m Amendment |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Takaaki KISHIGAMI, Isamu YOSHII Panasonic Corporation | Voice: + 81-46-840-5123 E-mail: kishigami.takaaki@jp.panasonic.com yoshii.isamu@jp.panasonic.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 “Comments on AWD 15.3.6.5.2.2 DL basic assignment A-MAP IE” |  |
| Abstract | We propose to add the modulation information of the other co-scheduled user’s stream in MU-MIMO mode to the DL Basic A-A-MAP IE in order to enable to apply a MLD receiver. |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for DL A-MAP IE for MU-MIMO mode 
in IEEE802.16m Amendment

Takaaki KISHIGAMI, Isamu YOSHII

Panasonic Corporation

# Introduction

The DL Basic assignment A-MAP IE does not include the modulation information of the other co-scheduled user’s stream in MU-MIMO modes in current AWD [1]. Therefore, a MS can not apply a MLD receiver which would be better performance than a MMSE receiver.

In the previous session, the “modulation aware” horizontal encoding (MU-MIMO) mode [2] was proposed as the additional MEF mode to enable to applay a MLD receiver. In “modulation aware” horizontal encoding mode, each user is informed about the modulation corresponding to the other co-scheduled AMSs (streams), which was defined as a MDF(Modulation Field) expressed by 2 bits for Mt = 2, 4 bits for Mt = 3 and 5 bits for Mt = 4.

In this contribution, we support to add the “modulation aware” horizontal encoding mode to DL A-A-MAP IE and propose to modify the bit expression of the MDF to reduce the overhead of the MDF.

# Proposed method

We propose to reduce the overhead of MDF by giving the relation to PSI and modulation information. To reduce the overhead of the MDF, PSIs are assigned in order with small modulation level of Tx streams. This causes the reduction of the information for the pairing of modulation information and PSI. This PSI assignment method doesn't influence the receiving performance, since the pilot density per Tx stream is equal in 16m pilot structure.

Table 1 shows an example of applying the PSI assignment rule mentioned above for the four user MU-MIMO (Mt = 4) . And Table 2 shows the bit expression of the MDF for Mt=4. For example, AMS #2 assigned to #1 stream is informed by ‘0b1001’ as the bit expression of MDF based on Table 2.

Table 3 shows the comparison results of the required bits for the MDF between the proposed and conventional method[2]. We found that the proposed method can reduce the MDF bits compared with the conventional.

Table 1 example of PSI assignment for the four user MU-MIMO case ( Mt = 4 )

|  | The modulation of Tx stream for AMS#n | Assigned PSI |
| --- | --- | --- |
| AMS #1 | 16QAM | #2 stream |
| AMS #2 | QPSK | #1 stream |
| AMS #3 | 64QAM | #4 stream |
| AMS #4 | 16QAM | #3 stream |

Table 2 The bit expression of the MDF ( Mt = 4 )

MDF: Modulation information of (#j stream, #k stream, #m stream). Here, 0 < j < k < m < Mt and the number of j, k and m are not equal to the number of the stream assigned by the PSI.

0b0000: (QPSK, QPSK, QPSK) 0b0010: (QPSK, QPSK, 16QAM) 0b0011: (QPSK, QPSK, 64QAM) 0b0100: (QPSK, 16QAM,16QAM) 0b0101: (QPSK, 16QAM, 64QAM) 0b0111: (QPSK, 64QAM, 64QAM) 0b1000: (16QAM, 16QAM,16QAM) 0b1001: (16QAM, 16QAM,64QAM) 0b1011: (16QAM, 64QAM,64QAM) 0b1111: (64QAM, 64QAM,64QAM)

Table 3 Comparison of the required bit for the MDF

| The number of Tx streams | Mt = 2 | Mt = 3 | Mt = 4 |
| --- | --- | --- | --- |
| Proposed MDF | 2 bits (3 patterns) | 3 bits (6 patterns) | 4 bits (10 patterns) |
| Conventional MDF | 2 bits (3 patterns) | 4 bits (9 patterns) | 5 bits (27 patterns) |

# References

[1]IEEE 802.16m-09/0010r2, “IEEE802.16m Amendment Working Document (AWD)”

[2]IEEE 802.16m-09/1017, “Text proposal on DL MAP”

# Proposed Text

We propose the following text for the IEEE802.16m Amendment Working Document in Section 15.3.6.5.2.2.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and  existing AWD text is shown in black.

-------------------------------  Start text proposal  ---------------------------------------------------

- 15.3.6.5.2.2 DL basic assignment A-MAP IE

Table 668 describes the fields in a DL Basic Assignment A-MAP IE used for resource assignment in the DL.

Definitions of the fields in the DL Basic Assignment A-MAP IE are listed following Table 668.

Table 668 –DL basic assignment A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL-MAP() { |  |  |
| A-MAP IE Type | 4 | DL Basic Assignment A-MAP IE |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: Horizontal encoding with the other co-scheduled AMSs modulation information |
| If (MEF==0b01){ |  | Parameters for vertical encoding |
| if(Nt == 2){ |  |  |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| }else if(Nt == 4){ |  |  |
| Mt | 2 | Number of streams in transmission for Nt = 4 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| }else if(Nt == 8){ |  |  |
| Mt | 3 | Number of streams in transmission for Nt = 8 (Mt <= Nt ) 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| } |  |  |
| }else if(MEF == 0b10){ |  | Parameters for horizontal encoding |
| If(Nt == 2){ |  |  |
| PSI | 1 | Allocated pilot stream index for Nt = 2 0b0: #1 stream 0b1: #2 stream |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| } else{ |  |  |
| PSI | 2 | Allocated pilot stream index for Nt = 4 or 8 0b00: #1 stream 0b01: #2 stream 0b10: #3 stream 0b11: #4 stream |
| Mt | 2 | Number of streams in transmission for Nt = 4 or 8 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| } |  |  |
| }else if(MEF == 0b11){ |  | Parameters for Horizontal encoding with the other co-scheduled AMSs modulation information |
| If(Nt == 2){ |  |  |
| PSI | 1 | Allocated pilot stream index for Nt = 2 0b0: #1 stream 0b1: #2 stream |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| If (Mt == 0b1){ |  |  |
| MDF | 2 | 0b00: QPSK 0b01: 16 QAM 0b10: 64 QAM 0b11: n/a |
| } |  |  |
| } else |  |  |
| PSI | 2 | Allocated pilot stream index for Nt = 4 or 8 0b00: #1 stream 0b01: #2 stream 0b10: #3 stream 0b11: #4 stream |
| Mt | 2 | Number of streams in transmission for Nt = 4 or 8 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| If (Mt ==0b01){ |  |  |
| MDF | 2 | 0b00: QPSK 0b01: 16 QAM 0b10: 64 QAM 0b11: n/a |
| }else if (Mt ==0b10){ |  |  |
| MDF | 3 | MDF: Modulation information of (#j stream, #k stream). Here, 0 < j < k < Mt and the number of j and k are not equal to the number of the stream assigned by the PSI. 0b000: QPSK, QPSK 0b010: QPSK, 16QAM 0b011: QPSK, 64QAM 0b100:16QAM, 16QAM 0b101:16QAM, 64QAM 0b111: 64QAM, 64QAM other possibilities n/a |
| }else if (Mt ==0b11){ |  |  |
| MDF | 4 | MDF: Modulation information of (#j stream, #k stream, #m stream). Here, 0 < j < k < m < Mt and the number of j, k and m are not equal to the number of the stream assigned by the PSI. 0b0000: (QPSK, QPSK, QPSK) 0b0010: (QPSK, QPSK, 16QAM) 0b0011: (QPSK, QPSK, 64QAM) 0b0100: (QPSK, 16QAM,16QAM) 0b0101: (QPSK, 16QAM, 64QAM) 0b0111: (QPSK, 64QAM, 64QAM) 0b1000: (16QAM, 16QAM,16QAM) 0b1001: (16QAM, 16QAM,64QAM) 0b1011: (16QAM, 64QAM,64QAM) 0b1111: (64QAM, 64QAM,64QAM) other possibilities n/a |
| } |  |  |
| } |  |  |
| Resource Allocation | Variable | Variable number of bits - depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) - Allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b0: 4 DL subframes for FDD or all DL subframes for TDD |
| if (Nsubframe, A-MAP == 2){ |  |  |
| Allocation Relevance | TBD | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| } |  |  |
| AHFA | [4] | TBD HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| SPID/CoRe Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| Reserved | TBD | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |
| } |  |  |

A-MAP IE Type: Defines the structure of the A-MAP IE for the bits in the A-MAP IE following the A-MAP IE type field. A-MAP IE Type distinguishes between UL/DL, basic/extended IE. Additional IE types are reserved for future use.

MEF: MIMO Encoder Format.

PSI: Allocated pilot stream index for horizontal encoding.
PSI shall be allocated in order with small modulation level of AMS’s streams.

Mt: Number of streams in transmission. The DL pilot pattern with Mt streams shall be used in the allocated resource.

RA: Resource Allocation information is used to signal the type of resource unit allocated (DRU/CRU), the location (start/end) and allocation size.

Long TTI Indication: Indicator to signal allocations span multiple subframes in time.

Allocation Relevance: Subframe index corresponding to the A-MAP region in which the resource is allocated.

HFA: TBD allocation for HARQ feedback.

SPID/CoRe Version: Signaling for HARQ IR including HARQ subpacket identifier for IR and Constellation

Rearrangement version.

MCRC: 16 bit CRC masked by Station ID/Flow ID.

MDF: Modulation information of the other co-scheduled AMS’s streams in MU-MIMO mode.

-------------------------------  End text proposal  ---------------------------------------------------