| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | CR on SDD Chapter 15.2: CSG femtocell BS type change |  |
| Date Submitted | 2009-07-03 |  |
| Source(s) | Yang Liu Zhaohua Lu Feng Xie Hongyun Qu Kaiying Lv ZTE Corporation | Liu.yang8@zte.com.cn Lu.zhaohua@zte.com.cn Xie.feng2@zte.com.cn |
| Re: | Call for reply comment on SDD IEEE 802.16-08/003r9a |  |
| Abstract | This contribution provides text on CSG femtocell type change for load balance between overlay Macro BS and the femtocell BS. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

CSG femtocell BS type change

Yang Liu, Zhaohua Lu, Feng Xie, Hongyun Qu, Kaiying Lv

ZTE Corporation

Introduction

In current SDD text, the CSG Femtocell BS refers to either CSG-Open Femtocell BS or CSG-Closed Femtocell BS. When the overlay BS is over loaded, it is expected that the nearby CSG femtocell BS could change its type to CSG-Open femtocell. Accordingly, some non-CSG users can handover from the overlay BS to the CSG femtocell.

In addition, when the overlay Macro BS’s recovers from a heavy load, the CSG femto cell could change to CSG-closed after directing all the non-CSG users to handover to the overlay Macro BS.

The type change mentioned above should be directed by the macro BS or the network.

Add the following text in the end of 15.2 (IEEE 802.16m-08/003r9a):

--------------------------------------------- Proposed text -----------------------------------------------------------------

15.2 Types of Base stations

When the overlay BS is over loaded, the CSG-Closed femtocell BS may change its type to CSG-Open femtocell BS. Accordingly, some non-CSG users could perform handover from the overlay macro BS to the CSG femtocell BS. This change should be directed by the macro BS or the network.

--------------------------------------------- End of Proposed text ----------------------------------------------------------