| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification to DL non-adaptive precoding in OL MIMO region |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Kiran Kuchi, J Klutto Milleth, Vinod R, Divagar , Padmanabhan M S, Bhaskar R ¦ CEWiT, India | E-mail: kkuchi@cewit.org.in (+91 9962045186) klutto@cewit.org.in (+91 9444107970) |
| Re: | Call for Comments on IEEE 802.16m-09/0010r2 IEEE 802.16m Amendment text/Area: Chapter 15.3.7.2.3.2 (DL-MIMO) |  |
| Abstract | Amendment text for Non-adaptive precoding in OL MIMO region |  |
| Purpose | For inclusion in AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification to DL non-adaptive precoding in OL MIMO region

Kiran Kuchi, J Klutto Milleth, Vinod R, Divagar , Padmanabhan M, Bhaskar R

CEWiT, India

# Introduction:

In OL MIMO region,

- N1 cycling should be used for CRUs based on logical sub band index

- N2 cycling should be used for DRUs and mini band CRUs based on logical PRU index

The current AWD proposes code word cycling (CWC) based on physical subcarrier indices.

In the presence of multiple frequency partitions, the CWC based on logical PRU indices will perform better than that based on physical subcarrier indices.

When a user is allocated in a frequency partition with logically contiguous RUs (subbands or PRUs), there is higher possibility that the RUs may use the same codeword. The purpose of CWC to give diversity over flat fading channels is defeated in such a scenario. If the CWC is based on logical miniband and/or subband index, this issue can be solved.

## ……………………..Start of Text, page 251, line 51……………………………………

Section 15.3.7.1.2.1

Replace the current text which is defined in “Non-adaptive pre-coding inside the OL region” with the following text:

Non-adaptive pre-coding inside the OL region: In a RU allocated in the MaxMt-streams OL Region, with MEF = 0b00, 0b01 or 0b10 and non-adaptive precoding, the matrix W changes every N PRUs within a frequency partition.

For sub band CRU permutation N is N1.In this case, the Nt×MaxMt  precoding matrix W applied in in CRUFP,i[j] is selected as the codeword of index q in the open-loop codebook subset of rank MaxMt, where q is given by

q=jN1modNW0≤j≤CASSB,iN1−1

where CRUFP,i[j] is the jth CRU in ith frequency partition,CASSB,pis the number of sub bands in ith frequency partition.

For DRU and mini band CRU permutation N is N2.In the case of mini band permutation, the Nt× MaxMt precoding matrix W applied in CRUFP,i[j] and in subframe number t in ith frequency partition is selected as the codeword of index q in the open-loop codebook subset of rank MaxMt, where q is given by .

q=(jN2+t)modNWCASSB,iN1≤j≤LCRU,FPp

where CRUFP,i[j] is the jth CRU in pth frequency partition,CASSB,iis the number of sub bands and LCRU,FPiis the number of CRUs in ith frequency partition

In the case of DRU permutatations, the Nt× MaxMt precoding matrix W applied in DRUFP,p[j] and in subframe number t in ith frequency partition is selected as the codeword of index q in the open-loop codebook subset of rank MaxMt, where q is given by

q=(jN2+t)modNW01≤j≤LDRU,FPi

where DRUFP,i[j] is the jth DRU and LCRU,FPiis the number of DRUs in ith frequency partition respectively

q=(s+t)modNWs=0,1,,..KPRUi−1 whereKPRUiis the number of  PRUs  in ith frequency partition

## ……………………..End of Text……………………………………………………….