| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AWD Text for AMS Privacy in IEEE 802.16m |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Chengyan Feng ZTE Corporation | E-mail: feng.chengyan@zte.com.cn |
| Re: | IEEE 802.16m-09/0020. ”Call for Comments and Contributions on Project 802.16m Amendment Working Document” Category:AWD/Area: Chapter 15.2.3 (Security) Target topic: AMS Privacy |  |
| Abstract | This contribution proposes the texts for AMS privacy section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AMS Privacy

Chengyan Feng

ZTE Corporation

# 1. Introduction

In this contribution, we propose the detailed AMS Privacy texts to Amendment Working Document(AWD). The text proposal is based on the current 802.16m SDD.

# 2. Solution

The AMS privacy support is the process of protecting both the identity of AMS so that AMS MAC address is not revealed via air interface, and the mapping of AMS MAC address and Station ID so that an intruder cannot obtain the mapping between AMS MAC address and Station ID, based on which perform specific attacks to that specific user by monitoring the ranging procedure.

The mapping between the STID and the AMS MAC Address have been defined in SDD, but the AMS MAC Address transmission is still a problem. In this contribution, we propose a solution that transmitting the hash value of the AMS MAC Address to ABS to derive all the keys. This can avoid the real AMS MAC Address exposure in the air interface.

# 3. Text Proposal

======================== Start of Proposed Text =====================

15.2.3.6 AMS Privacy

AMS Privacy is achieved by protecting the AMS MAC Address transmission and the mapping between the STID and the AMS MAC Address.

In order to avoid exposure in the air interface, the AMS MAC Address is hashed before transmission. The hash result AMS MAC Address* is derived as follows:

AMS MAC Address*=Dot16KDF(AMS MAC Address, ABSID|Random, 48)

-ABSID is used to ensure different permutation per BS.

-Random is an random number of 48-bit generated by AMS before sending RNG-REQ message. If the AMS doesn’t receive a successful RNG-RSP from the ABS, the AMS should re-generate a Random and update the AMS MAC Address*, and then send another RNG-REQ with it to the ABS.

The AMS sends AMS MAC Address* to ABS in RNG-REQ message. After the successful authentication/authorization procedure, the AMS and ABS derive AK/CMAC KEY/TEK based on AMS MAC Address*. And the real AMS MAC Address can be defered to be sent to the ABS in REG-REQ message in encryption manner. Afterwards, the real AMS MAC Address is used to derive all the keys when re-authentication, HO, network re-entry, etc.

In order to protect the mapping between the STID and the AMS MAC Address, two types of STIDs are defined to an AMS during network entry - temporary STID (TSTID) and (normal) STID. A TSTID is assigned in RNG-RSP message, and is used until the STID is allocated. And the STID is assigned in REG-RSP message after the successful authentication/ authorization. The STID need to be encrypted during transmission. The TSTID is released after STID is assigned. And the STID is used for all the remaining transactions.

Figure x shows the overall network entry procedure to Support AMS Privacy in IEEE 802.16m.

Figure x Network Entry Procedure to Support AMS Privacy in IEEE 802.16m

============================== End of Proposed Text ===============

# 4. References

[1] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[3] IEEE 802.16m-08/003r9a, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010R2, “IEEE 802.16m Amendment Working Document”