| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MIMO Midamble Pattern Design |  |
| Date Submitted | 2009-07-14 |  |
| Source(s) | Ting Liang, Dengkui Zhu, Zirong Li, Dihao Ning, Bo Sun ZTE Corporation | {liang.ting12, zhu.dengkui, lizirong, sun.bo1}@zte.com.cn |
| Re: | Category: AWD comments / Area: Chapter 15.3.5.4.2 (MIMO Midamble) “Comments on AWD 15.3.5.4.2 MIMO midamble” |  |
| Abstract | The contribution proposes the text of MIMO midamble pattern design be included in the 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MIMO Midamble Pattern Design

Ting Liang, Dengkui Zhu, Zirong Li, Dihao Ning, Bo Sun

ZTE Corporation

# Introduction

MIMO midamble is proposed to be used for PMI selection in closed loop MIMO and also for CQI calculation in OL MIMO. The midamble signal occupies one OFDMA symbol in a DL sub-frame. For the type-1 subframe case, the remaining 5 consecutive symbols form a type-3 subframe. For the type-2 subframe case, the remaining 6 consecutive symbols form a type-1 subframe. But the place of the midamble symbol and pattern of the midamble are not defined in current AWD description.

This contribution proposes the MIMO midamble structure for channel estimation of different number of TX antennas. And the performances of different midamble structures are evaluated as well.

# The structure of MIMO midamble

The midamble has been defined to occupy one OFDMA symbol in a DL sub-frame in current AWD. In order to use this symbol to estimate channel correctly, this contribution proposes a physical structures of the MIMO midamble of 2Tx/4Tx/8Tx.

- In SDD description, the MS processing delay is 1.23ms. In order to get tradeoff between the midamble feedback performance and the processing delay, the midamble symbol placed in the last symbol of the second last DL sub-frame. And MS can achieve the feedback information from the latest midamble which has been processed. The latest midamble can be the one transmitted in current frame if MS can finish the processing before the first UL subframe comes. Or it can be the one transmitted in last frame for enough processing time. In this way, MS which the processing capability is limited may has enough time to process the midamble. And meanwhile, the MS which can finish midamble processing fastly can obtain more exact channel estimation.

- The midamble of different Tx antenna are evenly spaced across all the useful subcarriers except the guard band subcarriers and the DC subcarrier of this symbol. Eliminating the DC subcarrier can avoid the periodicity of the midamble in time domain by which the midamble will not conflict with PA-preamble.

- The decimation of midamble in 2Tx is 6, and the decimation of midamble in 4Tx is 12. The midamble of 2Tx and 4Tx in neighbor sectors are placed in the way of FDM in the same symbol. The decimation of midamble in 8Tx is 8. Midambles of 8Tx in different sectors are placed in the way of CDM.

Following is the pattern of midamble in detail.

Figure 1 Proposed midamble patterns for 2Tx

Figure 2 Proposed midamble patterns for 4Tx

Figure 3 Proposed midamble patterns for 8Tx

Figure1 show the proposed midamble patterns for 2Tx antennas. The midamble of different antenna placed in different subcarriers. The decimation of the midamble in each Tx antenna is 6. Midambles in neighbor sectors can be placed in the way of FDM in the same symbol thus the interference between them is avoid perfectly.

Figure2 show the proposed midamble patterns for 4Tx antennas. The midamble of different antenna placed in different subcarriers. The decimation of the midamble in each Tx antenna is 12. Midambles in neighbor sectors can be placed in the way of FDM in the same symbol thus the interference between them is avoid perfectly.

Figure 3 shows the proposed midamble patterns for 8Tx antennas. The midamble of different antenna placed in different subcarriers. The decimation of the midamble in each Tx antenna is 8. Midambles in different sectors can be placed in the way of CDM to get more exact channel estimation.

# The Performance of Proposal Midamble

The performance of different midamble structure for different number of Tx antennas was evaluated in noise limited scenario. MS estimates the channel of different antennas with MIMO midamble and calculates the values of PMI.

Figure 4  8Tx midamble performance in QPSK1/2 (VA10km/h)

Figure 5  8Tx midamble performance in QPSK1/2 (VA10km/h)

Figure 6  2Tx and 4Tx midamble performance in QPSK1/2 (VA10km/h)

Figure 7 Tb/2 per antenna Auto-Correlation FFT=512 and FFT=1024, Reuse 3

Figure 8 Tb/2 per antenna Auto-Correlation FFT=2048,Reuse3 and Tx =8, Reuse1

The simulation results show that the midamble placed in the last symbol of the second last DL sub-frame has better PER and throughput performances than the midamble placed in any other sub-frame. (For DL:UL = 5:3, it is the forth sub-frame)

The performance of midamble pattern of decimation 2 in CDM is almost the same as the midamble pattern of decimation 6 in FDM in interference limited scenario. While the estimation of midamble pattern in FDM is simpler than the estimation of midamble pattern in CDM. That’s because the estimation of midamble pattern in FDM can take in frequency domain only using the method that received signal divide midamble send by transmitter.

The evaluation results of Midamble Tb/2 auto-correlation show that eliminating the DC subcarrier can avoid the periodicity of the midamble sequence in time domain. In this way, the midamble will not conflict with PA-preamble.

So we recommend the midamble placed in the last symbol of the second last DL sub-frame. And the midamble of different Tx antenna are evenly spaced across all the useful subcarriers except the guard band subcarriers and the DC subcarrier. The decimation of midamble in 2Tx is 6, and the decimation of midamble in 4Tx is 12. The midamble of 2Tx and 4Tx in neighbor sectors are placed in the way of FDM in the same symbol. The decimation of midamble in 8Tx is 8, and the midamble 8Tx in neighbor sectors are placed in the way of CDM in the same symbol.

# Appendix

Simulation Parameters List

| Parameter | Values |
| --- | --- |
| Channel Bandwidth | 10MHz |
| Over-sampling Factor | 28/25 |
| FFT Size | 1024 |
| Cyclic Prefix (CP) ratio | 1/8 |
| DL:UL | 5:3 |
| Resource Unit | CRU (4x18x6) |
| DL:UL | 5:3 |
| Channel condition | mPED_B 3km/h |
| The number of antennas | 2Tx×2Rx，4Tx×2Rx |
| Channel estimation for midamble | 2D-MMSE |
| Channel estimation for demodulation | 2D-MMSE |
| Receiver | MMSE receiver |
| Report period | 5ms |
| CL-SU-MIMO schemes | SM |
| Rank | 1 |
| Codebook | 16e 3bit，16e 6bit, DFT |
| MidambleSubframeIndex | 2、3、4 |
| MidambleDecimation | 2/6(2Tx), 4/12(4Tx), 8(8Tx) |

# References

[1] IEEE 802.16m Amendment Working Document (AWD), IEEE 802.16m-09/0010r2

# Text proposal for inclusion in the 802.16m AWD

-------------------------------  Text Start  ------------------------------------------------------------------------------------

15.3.5.4.2 MIMO midamble

MIMO midamble is used for PMI selection in closed loop MIMO. For OL MIMO, midamble can be used to calculate CQI. The midamble signal occupies the one OFDMA symbol in a DL sub-frame. For the type-1 subframe case, the remaining 5 consecutive symbols form a type-3 subframe. For the type-2 subframe case, the remaining 6 consecutive symbols form a type-1 subframe.

[Insert the following text into the last paragraph of 15.3.5.4.2]

The midamble shall be placed in the last symbol of the second last DL sub-frame. And MS achieves the feedback information from the latest midamble which has been processed. The midamble of different Tx antenna are evenly spaced across all the useful subcarriers except the guard band subcarriers and the DC subcarrier of this symbol. The decimation of midamble in 2Tx is 6, and the decimation of midamble in 4Tx is 12. The midamble of 2Tx and 4Tx in neighbor sectors are placed in the way of FDM in the same symbol. The decimation of midamble in 8Tx is 8. Midambles of 8Tx in different sectors are placed in the way of CDM.

Following is the illustration of midamble patterns with 2Tx, 4Tx and 8Tx.

Figure xxx Proposed midamble patterns for 2Tx

Figure xxx Proposed midamble patterns for 4Tx

Figure xxx Proposed midamble patterns for 8Tx

-------------------------------  Text End  ------------------------------------------------------------------------------------