| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AWD Text for Idle mode procedure in Femtocell |  |
| Date Submitted | 2009-07-15 |  |
| Source(s) | Jung-Mao Lin, Chie Ming Chou, Chun-Yen Wang, Frank Ren, Yung-Han Chen, Chun-Yuan Chiu ITRI | E-mail: moumou3@itri.org.tw |
| Re: | Category: AWD comments / Area: Femtocell |  |
| Abstract | The contribution proposes the text of idle mode procedure in the Femtocell section of the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AWD Text for Idle mode procedure in Femtocell

Jung-Mao Lin, Chie Ming Chou, Chun-Yen Wang, Frank Ren, Yung-Han Chen, Chun-Yuan Chiu

ITRI

- Introduction

Based on current SDD, femtocell BS shall support AMS idle mode procedure and the CSG Femtocell BSs may broadcast the paging messages that are related to only the AMSs of this CSG. The current SDD also describes that the overlay macrocell ABS and the CSG Femtocell BSs may share the same paging group ID. Furthermore, a femtocell BS is allowable to negotiate with a macro ABS over the air. In this contribution we propose the paging procedure and related messages for idle mode support in CSG Femtocell BS.

- Discussion and proposal

If the overlay macrocell ABS and the CSG Femtocell BSs share the same paging group ID, an AMS won’t perform location update procedure when it moves between overlay macrocell ABS and the CSG femtocell BS. Under this design, CSG femtocell BS has to broadcast the paging message same as the overlay macrocell BS and this paging message may contain an AMS which may not be allowed to access the CSG femtocell BS. It may raise large paging overhead for the CSG Femtocell BSs. To solve this paging overhead for CSG Femtocell BSs, current SDD describes that the CSG Femtocell BSs may broadcast the paging messages that are related to only the AMSs of this CSG. Based on this description, we further propose that the CSG femtocell BS broadcasts the paging messages that are related to only the MSs which initiates idle mode at this CSG femtocell BS to further reduce the paging overhead of CSG femtocell BS.

Fig. 1, an example of paging process

As an example shown in Figure 1, overlay macrocell ABS1 and CSG femtocell BS1 share the same paging group 1 and both AMS1 and AMS2 are accessible for CSG femtocell BS1. AMS1 and AMS2 request to go into idle mode in overlay macrocell ABS1 and CSG femtocell BS1 respectively. Based on the example, we consider two cases. Case 1: AMS1 goes into idle mode at macrocell ABS1. In this case, if paging controller needs to page AMS1, it will request all macrocell BSs which belong the PGID1 to page AMS1. The paging controller won’t request any CSG femtocell BSs in the PGID1 to page AMS1 in this time. This can reduce the paging overhead of CSG femtocell BSs. When AMS1 moves into the coverage of CSG femtocell BS1, AMS1 may perform cell reselection to CSG femtocell BS1 and DL synchronize with CSG femtocell BS1 to listen to the paging message. Once macrocell ABS1 broadcast the paging message that is related to AMS1, AMS1 won’t make any response after macrocell ABS1 paging because AMS1 can’t listen to the paging from macrocell ABS1.

As AMS1 doesn’t make any response after macrocell ABS1 paging, paging controller will start 2nd time paging. There will have two options to perform 2nd time paging. In option1, paging controller may request all macrocell ABSs belong the PGID1 and the CSG femtocell BSs which belong the PGID1 and are accessible for AMS1 to page AMS1. In this option, paging controller needs to records the white list of each CSG femtocell BSs. This is a large overhead for paging controller. In order to reduce the overhead for paging controller, we propose option2 to perform 2nd time paging. In option2, paging controller requests the macrocell BSs belong PGID1 to page AMS1, and then those macrocell BSs will request the overlapped CSG femtocell BSs which are accessible for AMS1 to page AMS by sending PAG-CMD over the air. The PAG-CMD is described in table1. Therefore paging controller doesn’t need to record the white list for each femtocell BSs.

AMS2

CSG femtocell BS1

DREG-REQ

DREG-CMD

Paging message

Fig 2, AMS requests idle mode operation at CSG femtocell BS

Case 2: AMS2 requests to go into idle mode at CSG femtocell BS1. As shown in Figure2, when AMS2 wants to go into idle mode operation, it will send DREG-REQ to CSG femtocell BS1 and waits DREG-CMD from CSG femtocell BS1. Based on the action code on DREG-CMD, it determines whether goes into idle mode or not. If AMS2 is allowed to go enter idle mode, it will listen the paging message from CSG femtocell BS1. The initiation process between CSG femtocell BS and AMS is the same as in 15.2.17.1. Once a paging controller needs to page AMS2, it will make a paging request to CSG femtocell BS1 only. And then CSG femtocell BS1 will update the corresponding action code regarding AMS2 in its paging message. The overlay macrocell ABS1 doesn’t broadcast the AMS2 in its paging message. This can reduce the paging overhead for macrocell BSs.

Once AMS2 doesn’t make paging response to CSG femtocell BS1 after paging, paging controller will start 2nd time paging for AMS2. There are still two options to perform the 2nd time paging. In the option1, paging controller will request the macrocell BSs belong the PGID1 and the CSG femtocell BSs which belong the PGID1 and are accessible for AMS2 to page AMS2. In this option, paging controller need to record the white list of all CSG femtocell BSs. This is cause large overhead for paging controller. In order to reduce the overhead, we propose option 2 to perform the 2nd time paging for AMS2. In option2, paging will request macrocell BSs belong the PGID1 to page AMS2 and those macrocell BSs then request overlapped CSG femtocell BSs which are accessible for AMS2 to page AMS2 together by sending PAG-CMD over the air. In option2, paging controller doesn’t record the white list of all CSG femtocell BSs.

Table 1, Format of PAG-CMD

| Field | Size (bits) | Notes |
| --- | --- | --- |
| Format of PAG-CMD (){ |  |  |
| Number_AMS | 4 | This value represents that the number of AMS that doesn’t make response after paging |
| For(i=0;i< Number of AMS;i++) { |  |  |
| Action code | 2 |  |
| TLV encoded parameters | TBD |  |
| } |  |  |

With this proposal, some advantages can be expected:

- A CSG femtocell BS only needs to provide paging message for those AMS goes into idle mode at its covered and this will reduce paging overhead for a CSG femtocell BS.

- When a CSG femtocell BS goes into low-duty mode, it doesn’t consider the idle mode support for those AMSs out of its coverage. This can simplify the low-duty cycle design.

- When an AMS migrates between overlay macro BS and CSG Femtocell BS, there is no location update process.

- Proposed AWD Text

 Proposed Text for 15.7 

---------------------------------------------------- Start of Proposed Text----------------------------------------------------

15.7.	Idle Mode

Femtocell BS shall support idle mode. Dependent on topology design to support both Femtocell BS and macro BS, one or more paging groups may be assigned to a Femtocell or a macro BS. The overlay macro BS and the CSG Femtocell BS may share the same paging group ID.

The OSG Femtocell BSs operate like macro BSs when paging an MS. The CSG Femtocell BSs may broadcast the paging messages that are related to only the MSs of this CSG.

When paging controller requests to paging an MS, any of CSG Femtocell BSs shall not be involved in the first paging if the MS didn’t initial idle mode entry or perform location update at a CSG Femtocell BS. In this case, macro BSs which share the same paging group ID with the MS are only requested to page the MS in the first paging. If the first paging is failed, not only macro BSs which share the same paging group ID with the MS, but all CSG Femtocell BSs which allow the MS for access and share the same paging group ID with the MS shall be requested to page the MS in the following paging actions.

If the paged MS initialed idle mode entry or performed location update at a CSG Femtocell BS, only the CSG Femtocell BS shall be requested to page the MS in the first paging. If the first paging is failed, all CSG Femtocell BSs which allow the MS for access and share the same paging group ID with the MS and all macro BSs which share the same paging group ID with the MS shall be requested to page the MS in the following paging actions.

---------------------------------------------------- End of Proposed Text ------------------------------------------------