| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Length Field usage in MAC Headers (AWD - 15.2.3.1.1) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son Samsung Electronics | anilag@samsung.com, yb.chang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | “802.16m amendment working document”: IEEE 802.16m-09/00028r1, “Call for comments on Project 802.16m Amendment Working Document (AWD) IEEE 802.16m-09/00010r2” |  |
| Abstract | This contribution proposes text clarifications for the usage of length fields in various MAC headers, to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Length Field usage in MAC Headers

Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

The length field in Generic MAC header should represent the length of the entire MAC PDU including extended header, which is consistent with 16 legacy system (802.16-2009). This facilitates to identify the Multiple MAC PDUs in a data burst quickly without the need to parse the extended headers of each MPDU.

In the current AWD text length field in generic MAC header represents the length of the payload. This force the implementations to perform sequential processing of multiple MAC PDUs in a data burst. This can cause decoding delay of management connection PDU when transport PDU and management PDU are concatenated in PHY burst.

The change of ‘length’ definition does not cause any additional overhead in current AWD headers.

# Text Proposal for inclusion in the 802.16m amendment working document

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Change 1 - Modify the table 1 and 2 in subclause 15.2.3.1.1, page: 11, as follows]

Table 1 - GMH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Generic MAC Header () { | - |  |
| FlowID | 4 | Flow Identifier |
| EH | 1 | Extended header presence indicator |
| Length | 11 | Length of the payload in bytes of MAC PDU |
| } | - |  |

Table 2 - GMH fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| FlowID | 4 | This field indicates the service flow that is addressed. |
| EH | 1 | When set to ‘1’, this field indicates that an Extended Header is present following this GMH. |
| Length | 11 | This field indicates the length of the payload in bytes of MAC PDU including the GMH and extended header if present. |

[Change 2 - Modify the table 3 and 4 in subclause 15.2.3.1.2, page: 11 & 12, as follows]

Table 3 – CMH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Compact MAC Header () { | - |  |
| EH | 1 | Extended header presence indicator |
| Length | 7 | Length of the payload in bytes of MAC PDU. |
| } | - |  |

Table 4 - GCMH fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| EH | 1 | When set to ‘1’, this field indicates that an Extended Header is present following this CMH. |
| Length | 7 | This field indicates the length of the payload in bytes of MAC PDU including the CMH and extended header if present. |

[Change 3 - Modify the table 8 in subclause 15.2.3.2.1, page: 13, as follows]

Table 8 FPEH Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| SN | 10 | Payload Sequence Number |
| FC | 2 | Fragmentation Control bits (encoding shown in Table x.9) |
| End | 1 | Indication of more information 0 = Indicating another “Length” and “End” fields are followed 1 = Indicating no more “Length” and “End” fields are followed |
| Length | 11 | This field indicates the length of SDU or SDU fragment. If a payload consists of ‘N’ SDU/SDU fragments, N-1 ‘Length’ fields are present in FPEH. These represent the length of first ‘N-1’ SDU/SDU fragments in payload. The length of the first last SDU or SDU fragment in the payload is = MAC PDU payload length – sum of ‘N-1’ length fields in FPEH. indicated by the ‘Length’ field in GMH. |
| Reserved | Variable | Reserved bits are added at the end of FPEH for byte alignment |

[Change 4 - Modify the table 12, 13, 14 in subclause 15.2.3.2.3, page: 15, 16, as follows]

Table 12 Multiplexing Extended Header Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| Last | 1 | Last Extended Header Indicator (always set to 1) |
| Type | TBD | Extended Header Type |
| MEHB | variable | Multiplexing extended header block (format shown in Table 13 ) |

Table 13 MEHB format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MEHB () { | - |  |
| M | 1 | 0 = Another MEHB follows this MEHB 1 = This is the last MEHB in MEH |
| If (! First MEHB) { |  |  |
| FlowID | 4 | Flow Identifier |
| } |  |  |
| FC | 2 | Fragmentation Control |
| SN | 10 |  |
| do { | - |  |
| Length | 11 | SDU or SDU fragment length |
| End | 1 |  |
| } while (!End) | - |  |
| } | - |  |

Table 14 First Last MEHB format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| First MEHB () { | - |  |
| M | 1 | 0 = Another MEHB follows this MEHB 1 = This is the last MEHB in MEH |
| If (! First MEHB) { |  |  |
| FlowID | 4 | Flow Identifier |
| } |  |  |
| FC | 2 | Fragmentation Control |
| SN | 10 |  |
| do { | - |  |
| End | 1 |  |
| If ( End == 0) { | - |  |
| Length | 11 | SDU or SDU fragment length |
| } | - |  |
| } while (!End) | - |  |
| } | - |  |

[Change 5 - Modify the text in subclause 15.2.3.2.3, page: 15, lines 53-58 as follows]

The format of MEHB is shown in Table 13, except the first last MEHB. The format of the first last MEHB is shown in Table 14. The first MEHB doesn’t contain the Flow ID. and the length for the first SDU/SDU fragment associated with the Flow ID. The Flow ID and the Length fields in the generic MAC header represent the flow ID and the length of the first SDU/SDU fragment associated with the first MEHB. If the last flow payload consists of ‘N’ SDU/SDU fragments, N-1 ‘Length’ fields are present in last MEHB. These represent the length of first ‘N-1’ SDU/SDU fragments in last flow payload. The length of the last SDU or SDU fragment in the last flow payload is = last flow payload length – sum of ‘N-1’ length fields in MEHB. The fields of MEHB are defined in Table 15.

[Change 6 - Modify the text in subclause 15.2.3.2.3, page: 15, lines 3-8 as follows]

The format of MEH is defined in Table 11—. It is used when SDUs/SDUs fragments from different connections are included in the same MPDU. MEH shall be the first extended header, if it is present in MAC PDU. Each MEH contains multiple Multiplexing Extended Header Blocks (MEHBs).MEH together with GMH identifies the connections whose payload is present in the MAC PDU. The Flow ID in the GMH identifies the connection corresponding to the first connection payload in the MAC PDU.  The Flow IDs in MEHBs identifies the connections corresponding to the remaining connection payloads in the MAC PDU. The fragmentation and packing information corresponding to the SDU/SDU fragments packed together in a connection payload is included in the MEHB corresponding to the connection payload. The fields of MEH are defined in Table 12—. The M bit in MEHB indicates if there is more MEHB followed.

---------------------------------------------------------End of the Text----------------------------------------------------------

# Reference

[1] IEEE 802.16m-09/0010r2, “802.16m Amendment Working Document (AWD)”