| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Length Field usage in MAC Headers |  |
| Date Submitted | 2009-09-02 |  |
| Source(s) | Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son Samsung Electronics Joey Chou Intel | anilag@samsung.com, yb.chang@samsung.com joey.chou@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0044, IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | This contribution proposes text clarifications for the usage of length fields in various MAC headers, to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Length Field usage in MAC Headers

Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

The length field in Generic MAC header should represent the length of the entire MAC PDU including extended header, which is consistent with 16 legacy system (802.16-2009). This facilitates to identify the Multiple MAC PDUs in a data burst quickly without the need to parse the extended headers of each MPDU.

In the current IEEE P802.16m/D1 document [1] length field in generic MAC header represents the length of the payload. This force the implementations to perform sequential processing of multiple MAC PDUs in a data burst. This can cause decoding delay of management connection PDU when transport PDU and management PDU are concatenated in PHY burst.

The change of ‘length’ definition does not cause any additional overhead in current headers adopted in IEEE P802.16m/D1 document [1].

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Change 1 - Modify the table 652 in subclause 15.2.2.1.1, page: 8, as follows and delete table 653]

Table 652 - AGMH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Advanced Genewric MAC Header () { | - |  |
| FlowID | 4 | Flow Identifier |
| EH | 1 | Extended header presence indicator; When set to ‘1’, this field indicates that an Extended Header is present following this GMH. |
| Length | 11 | Length of payload This field indicates the length in bytes of MAC PDU including the GMH and extended header if present. |
| } | - |  |

Table 653 - GMH fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| FlowID | 4 | This field indicates the service flow that is addressed. |
| EH | 1 | When set to ‘1’, this field indicates that an Extended Header is present following this GMH. |
| Length | 11 | This field indicates the length in bytes |

[Change 2 - Modify the table 654 in subclause 15.2.2.1.2, page: 9, as follows and delete table 655]

Table 654 653 – CMH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Compact MAC Header () { | - |  |
| EH | 1 | Extended header presence indicator; When set to ‘1’, this field indicates that an Extended Header is present following this CMH. |
| Length | 7 | Length of the payload This field indicates the length in bytes of MAC PDU including the CMH and extended header if present. |
| } | - |  |

Table 655 - GMH fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| EH | 1 | When set to ‘1’, this field indicates that an Extended Header is present following this CMH. |
| Length | 11 | This field indicates the length of the payload in bytes |

---------------------------------------------------------End of the Text----------------------------------------------------------

# Reference

[1] IEEE P802.16m/D1, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”