| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Construction & Transmission of MAC PDU section for AWD (AWD-section 15.2. x) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Anil Agiwal, Youngbin Chang, Sungjin Lee, Rakesh Taori, Jungje Son Samsung Electronics | anilag@samsung.com yb.chang@samsung.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | “802.16m AWD”: IEEE 802.16m-09/0028r1, “Call for Comments on Project 802.16m Amendment Working Document (AWD) Content”. |  |
| Abstract | This contribution proposes text for construction and transmission of MAC PDU section to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Construction & Transmission of MAC PDU in 16m

Anil Agiwal, Youngbin Chang, Sungjin Lee, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

This contribution proposes the text for construction and transmission of MAC PDU based on SDD [2].

# Outline of Construction & Transmission of MAC PDU in 16m

15.2. x Construction and Transmission of MAC PDUs

##### 15.2.x.1 Multiplexing

##### 15.2.x.2 Concatenation

##### 15.2.x.3 Fragmentation

15.2.x.3.1 Transport Connections

###### 15.2.x.3.1.1 Non-ARQ Transport Connections

###### 15.2. x.3.1.2 ARQ-enabled Transport Connections

15.2.x.3.2 Management Connections

##### 15.2.x.4 Packing

###### 15.2.x.4.1 Packing for non-ARQ Connections

###### 15.2.x.4.2 Packing for ARQ Connections

##### 15.2.x.5 Encryption of MAC PDUs

##### 15.2.x.6 Padding

# Text Proposal for inclusion in the 802.16m amendment working document

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Insert the texts in subclause 15.2.x as follows:]

15.2.x Construction and Transmission of MAC PDUs

The figure xx illustrates the various functional blocks involved in the construction of MAC PDU, input and output of each functional block, and sequence in which these functional blocks are applied during the construction of MAC PDUs of various types of connections i.e. ARQ connection, non ARQ connection and management connection. The construction of a MAC PDU is illustrated in figure yy below.

Figure xx Data Path Functional Blocks involved in construction of MAC PDUs

Figure yy Construction of MAC PDU

##### 15.2.x.1 Multiplexing

Multiple connections’ payload (one or more SDU/SDU fragments) associated with same security association can be multiplexed and encrypted together in a MAC PDU. The GMH (as defined in 15.2.3.1.1) and the MEH (as defined in section 15.2.3.2.3) provides the details about connection payloads as well as the SDU/SDU fragments in each connection payload. For example multiple connections’ payloads which are encrypted using AES CCM can be multiplexed and encrypted together in a MPDU. The figure zz illustrates the multiplexing of two connection payloads which are associated with same security association (i.e. AES CCM).

Figure zz Multiplexing of connection payload associated with same SA

##### 15.2.x.2 Concatenation

Multiple MAC PDUs may be concatenated into a single transmission in either the UL or DL directions. For  AMS attached to ABS, each MPDU in UL/DL burst is uniquely identified by Flow ID.

Figure xxx illustrates the concept for an UL burst transmission. Since the MAC SDUs in MAC PDU are indentified by the Flow ID in the GMH and MEH ( in case of multiplexing), the receiving MAC entity is able to present the MAC SDU (after reassembling the MAC SDU from one or more received MAC PDUs) to the correct instance of the MAC SAP. MAC management messages, user data (from one or more connections), and BR MAC PDUs may be concatenated into the same transmission.

Figure xxx MAC PDU concatenation showing example Flow IDs

##### 15.2.x.3 Fragmentation

Fragmentation is the process by which a MAC SDU (or MAC management message) is divided into one or more MAC PDUs. This process is undertaken to allow efficient use of available bandwidth relative to the QoS requirements of a connection’s service flow. Capabilities of fragmentation and reassembly are mandatory.

15.2.x.3.1 Transport Connections

For transport connections FPEH (as defined in section 15.2.3.2.1) or MEH(as defined in section 15.2.3.2.3) in the MAC PDU provides the information about the SDU fragment. Either FPEH or MEH shall always be present in a MAC PDU for transport connections. SN in FPEH or MEH (in case of multiplexing) is used for sequencing the SDU fragments and Fragmentation control (FC) bits in FPEH or MEH, are used to tag the SDU fragments with respect to their position in the parent SDU.

###### 15.2.x.3.1.1 Non-ARQ Transport Connections

For non-ARQ transport connections, fragments are transmitted once and in sequence. The SN assigned to each connection PDU carrying SDU fragment allows the receiver to recreate the original payload and to detect the loss of any intermediate fragments. A connection may be in only one fragmentation state at any given time. Upon loss, the receiver shall discard all SDU fragments on the connection until a new first SDU fragment is detected or a non-fragmented SDU is detected.

###### 15.2.x.3.1.2 ARQ-enabled Transport Connections

For ARQ connections, fragments are transmitted in sequence. The SN assigned to each ARQ PDU carrying SDU fragment allows the receiver to recreate the original payload and to detect the loss of any intermediate fragments.

15.2.x.3.2 Management Connections

For management connections FEH (as defined in section 15.2.3.2.2) in the MAC PDU provides the information about the management message fragment. SN in FEH is used for sequencing the management message fragments and Fragmentation control (FC) bits in FEH, are used to tag the management message fragments with respect to their position in the parent management message. MAC PDU with un-fragmented management message need not contain FEH.

Only one management message can be in fragmentation state at any given time. The SN assigned to each management connection PDU carrying management message fragment allows the receiver to recreate the original payload and to detect the loss of any intermediate fragments. Upon loss, the receiver shall wait for the lost management message fragments until a new first management message fragment is detected or a new non-fragmented management message is detected.

##### 15.2.x.4 Packing

MAC may pack multiple MAC SDUs into a single MAC PDU. The transmitting side has full discretion whether to pack a group of MAC SDUs in a single MAC PDU. The capability of packing and unpacking is mandatory. The packing and fragmentation mechanisms for both the ARQ and non-ARQ connections are specified in 15.2.x.4.1 and 15.2.x.4.2 respectively.

###### 15.2.x.4.1 Packing for non-ARQ Connections

A MAC PDU containing a packed sequence of variable-length MAC SDUs is constructed as shown in Figure zzz. Note that non-fragmented MAC SDUs and MAC SDU fragments may both be present in the same MAC PDU. The MAC attaches a FPEH (defined in section 15.2.3.2.1) or a MEH ( defined in section 15.2.3.2.3, in case of multiplexing) in the MPDU.

Figure zzz Packing variable length MSDUs into a single MAC PDU

Simultaneous fragmentation and packing allows efficient use of the air link, but requires guidelines to be followed so it is clear which MAC SDU is currently in a state of fragmentation. The fragmentation control bits should be set according to the rules defined in Table 9. The packing with fragmentation is illustrated in figure xxxx.

Figure xxxx Packing with fragmentation

###### 15.2.x.4.2 Packing for ARQ Connections

The use of FPEH for ARQ-enabled connections is similar to that for non-ARQ connections as described 15.2.x.4.1. The transmitting side has full discretion whether to pack a group of MAC SDUs and/or fragments in a single MAC PDU. The SN of the FPEH or MEHB (in case of multiplexing) shall be used by the ARQ protocol to identify and retransmit ARQ blocks.

##### 15.2.x.5 Encryption of MAC PDUs

When transmitting a MAC PDU on a connection that is mapped to an SA, the sender shall perform encryption and data authentication of the MAC PDU payload as specified by that SA. When receiving a MAC PDU on a connection mapped to an SA, the receiver shall perform decryption and data authentication of the MAC PDU payload, as specified by that SA.

The generic MAC header and extended headers shall not be encrypted. The receiver determines whether the payload in the MAC PDU is encrypted or not from the Flow ID in the GMH. The encryption information needed to decrypt a payload at the receiving station is present at the beginning and at the end of the connection payload. For e.g. in case of AES CCM, PN & EKS are present at the beginning of connection payload and ICV is appended at the end of the connection payload in MAC PDU as shown in figure xxxxx.

Figure xxxx MAC PDU with Transport Connection Payload– Authenticated & Encrypted using AES CCM

If multiple connection payloads are transmitted in same burst and the connections are mapped to same SA then multiple connection payload are multiplexed (as explained in section ) before encryption and multiplexed payload is encrypted together. The receiver shall perform the decryption and data authentication on the multiplexed payload, as specified by the SA. The receiver determines whether the payloads in the MAC PDU is encrypted or not from the Flow ID in the GMH. The encryption information needed to decrypt the multiplexed payload at the receiving station is present at the beginning of the first connection payload and at the end of the last connection payload. For e.g. in case of AES CCM, PN & EKS are present at the beginning of connection payload 1 and ICV is appended at the end of the connection payload n in MAC PDU as shown in figure yyyyy.

Figure yyyyy MAC PDU with Multiple Transport Connection Payloads– Authenticated & Encrypted using AES CCM

##### 15.2.x.6 Padding

Within the data burst, the unused portion shall be initialized to a known state. If the size of the unused portion is only one byte then the unused byte is set to 0xF0. If the size of the unused portion is greater than or equal to two bytes, the unused second byte is set to 0x00.

# Reference

[1] IEEE 802.16m-09/0010r1a, “802.16m Amendment Working Document (AWD)”

[2] IEEE 802.16m-08/003r8, “802.16m System Description Document (SDD)”