| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | FPEH/MEH format modification to facilitate polling, ARQ feedback piggybacking and rearrangement (AWD-section 15.2.3.2.1) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son Samsung Electronics | anilag@samsung.com, yb.chang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | “802.16m amendment working document”: IEEE 802.16m-09/00028r1, “Call for comments on Project 802.16m Amendment Working Document (AWD) IEEE 802.16m-09/00010r2” |  |
| Abstract | This contribution proposes text changes to modify FPEH/MEH to facilitate polling, ARQ feedback piggybacking and rearrangement to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modified FPEH/MEH format to facilitate polling, ARQ feedback piggybacking and rearrangement

Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

The following functionalities are adopted in AWD text (refer sections 15.2.13.1.1, 15.2.13.1.2, and 15.2.13.3.2 of C80216m-09_0010r2):

- Piggybacking ARQ feedback with payload

- Polling using FPEH & MEH

- Rearrangement by dividing ARQ block into fixed size sub blocks

This contribution proposes modifications to FPEH & MEH format to facilitate the above functionalities. The proposed changes are outlined below:

- Rearrangement indicator (RI), Sub Sequence number (SUB SN) & last sub-block indicator (LSI) in case of re-arrangement: In case of ARQ connection, the RI bit is set to ‘1’ if the payload consists of sub blocks of an ARQ block, otherwise it is set to ‘0’. For non ARQ connection the RI bit is always set to ‘0’ by the transmitter and is ignored by the receiver. See Figure 1, Figure 2, Figure 3.

- ARQ Feedback indicator (AFI) to indicate whether ARQ feedback is piggybacked with payload or not. In case of ARQ connection, the AFI bit is set to ‘1’ by the transmitter if ARQ feedback is present in the payload, otherwise it is set to ‘0’. The ARQ feedback is present before the SDU/SDU fragments in the payload. For non ARQ connection the AFI bit is always set to ‘0’ by the transmitter and is ignored by the receiver. See Figure 7, Figure 8, Figure 9.

- Polling Indicator (PI) to request for ARQ feedback from the receiver: In case of ARQ connection, the PI bit is set to ‘1’ by the transmitter to request ARQ feedback from the receiver, otherwise it is set to ‘0’. For non ARQ connection PI bit is always set to ‘0’ by the transmitter and is ignored by the receiver. See Figure 4, Figure 5, Figure 6.

The addition of (RI, AFI & PI bits) does not increase the header overhead as the reserved bits in existing format are used to define them.

# Illustrations

This section illustrates the usage of RI, AFI and PI bits.

## Rearrangement Indicator

Figure 1 Rearrangement of ARQ block with one SDU

Figure 2 Rearrangement of ARQ block with multiple SDUs

Figure 3 Re-arrangement bit in FPEH (Non ARQ flow)

## Polling Indicator

Figure 4 FPEH with polling

Figure 5 FPEH without polling

Figure 6 Polling bit in FPEH (Non ARQ flow)

## ARQ Feedback Indicator

Figure 7 ARQ Feedback piggybacking

Figure 8 ARQ Feedback piggybacking with polling

Figure 9 AFI bit in FPEH (Non ARQ flow)

# Text Proposal for inclusion in the 802.16m amendment working document

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Change 1 - Modify the texts in subclause 15.2.3.2.1, page: 13, line numbers: all as follows]

15.2.3.2.1 Fragmentation and packing extended header (FPEH)

The FPEH shall be used when MAC PDU contains payload from single transport connection. The FPEH exists after the last extended header (i.e. the extended header with ‘Last’ = ‘1’), if ‘EH’ in the GMH is set to ‘1’ or after the GMH, if ‘EH’ in the GMH is set to ‘0’. The FPEH format is defined in table 7 and its fields are defined in table 8.

Table 7 FPEH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| FPEH () { | - |  |
| PI | 1 | Polling Indicator |
| AFI | 1 | ARQ Feedback Indicator |
| SN | 10 | Payload sequence number |
| RI | 1 | Re-arrangement Indicator |
| If (RI == 1) { |  |  |
| SUB-SN | TBD | Sequence number of first Sub Block in the payload |
| LSI | 1 | Last Sub Block Indicator |
| } |  |  |
| FC | 2 | Fragmentation control (see table 9) |
| do { | - |  |
| End | 1 |  |
| if (End==0) { | - |  |
| Length | 11 | Length of SDU or SDU fragment |
| } | - |  |
| } while (!End) | - |  |
| Reserved | variable |  |
| } | - |  |

Table 8 FPEH Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| PI | 1 | In case of ARQ connection, the PI bit is set to ‘1’ by the transmitter to request ARQ feedback from the receiver, otherwise it is set to ‘0’. For non ARQ connection PI bit is always set to ‘0’ by the transmitter and is ignored by the receiver. |
| AFI | 1 | In case of ARQ connection, the AFI bit is set to ‘1’ by the transmitter if ARQ feedback is present in the payload, otherwise it is set to ‘0’. The ARQ feedback is present before the SDU/SDU fragments in the payload. For non ARQ connection the AFI bit is always set to ‘0’ by the transmitter and is ignored by the receiver. |
| SN | 10 | Payload Sequence Number |
| RI | 1 | In case of ARQ connection, the RI bit is set to ‘1’ if the payload consists of sub blocks of an ARQ block (indicated by the SN field), otherwise it is set to ‘0’. For non ARQ connection the RI bit is always set to ‘0’ by the transmitter and is ignored by the receiver. |
| SUB-SN | TBD | Sequence number of first Sub Block in the payload. This field is present only if RI is set to ‘1’. |
| LSI | 1 | This bit is set to ‘1’ if the last sub block of the ARQ block represented by SN field is the last sub block present in the payload. This field is present only if RI is set to ‘1’. |
| FC | 2 | Fragmentation Control bits (encoding shown in Table x.9) |
| End | 1 | Indication of more information 0 = Indicating another “Length” and “End” fields are followed 1 = Indicating no more “Length” and “End” fields are followed |
| Length | 11 | This field indicates the length of SDU or SDU fragment. If a payload consists of ‘N’ SDU/SDU fragments, N-1 ‘Length’ fields are present in FPEH. The length of the first SDU or SDU fragment in the payload is indicated by the ‘Length’ field in GMH. |
| Reserved | Variable | Reserved bits are added at the end of FPEH for byte alignment |

[Change2 - Modify the texts in subclause 15.2.3.2.3, Tables 11, 13, 14, 15 on page: 15, 16, 17:  as follows]

Table 11 Multiplexing Extended Header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MEH () { | - |  |
| LAST | 1 | 0 = Another extended header follows MEH 1 = another extended header does not follow MEH |
| Type | TBD | MEH type |
| AFI | 1 | ARQ feedback indication. If a packed ARQ feedback payload presents within this MAC PDU, this field is set to ‘1’ and the ARQ feedback payload shall be transported as the first packed payload |
| do { | - |  |
| MEHB (M) | variable | Multiplexing extended header block |
| }while (!M) | - |  |
| Reserved | variable |  |
| } | - |  |

Table 13 MEHB format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MEHB (M) { | - |  |
| M | 1 | 0 = Another MEHB follows this MEHB 1 = This is the last MEHB in MEH |
| FlowID | 4 | Flow Identifier |
| PI | 1 | In case of ARQ connection, the PI bit is set to ‘1’ by the transmitter to request ARQ feedback from the receiver, otherwise it is set to ‘0’. For non ARQ connection PI bit is always set to ‘0’ by the transmitter and is ignored by the receiver. |
| SN | 10 | Payload sequence number |
| RI | 1 | In case of ARQ connection, the RI bit is set to ‘1’ if the payload consists of sub blocks of an ARQ block (indicated by the SN field), otherwise it is set to ‘0’. For non ARQ connection the RI bit is always set to ‘0’ by the transmitter and is ignored by the receiver. |
| If (RI == 1) { |  |  |
| SUB-SN | TBD | Sequence number of first Sub Block in the payload. This field is present only if RI is set to ‘1’. |
| LSI | 1 | This bit is set to ‘1’ if the last sub block of the ARQ block represented by SN field is the last sub block present in the payload. This field is present only if RI is set to ‘1’. |
| } |  |  |
| FC | 2 | Fragmentation Control |
| do { | - |  |
| Length | 11 | SDU or SDU fragment length |
| End | 1 |  |
| } while (!End) | - |  |
| } | - |  |

Table 14 First MEHB format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| First MEHB (M) { | - |  |
| M | 1 | 0 = Another MEHB follows this MEHB 1 = This is the last MEHB in MEH |
| PI | 1 | In case of ARQ connection, the PI bit is set to ‘1’ by the transmitter to request ARQ feedback from the receiver, otherwise it is set to ‘0’. For non ARQ connection PI bit is always set to ‘0’. |
| SN | 10 | Payload sequence number |
| RI | 1 | In case of ARQ connection, the RI bit is set to ‘1’ if the payload consists of sub blocks of an ARQ block (indicated by the SN field), otherwise it is set to ‘0’. For non ARQ connection the RI bit is always set to ‘0’. |
| If (RI == 1) { |  |  |
| SUB-SN | TBD | Sequence number of first Sub Block in the payload. This field is present only if RI is set to ‘1’. |
| LSI | 1 | This bit is set to ‘1’ if the last sub block of the ARQ block represented by SN field is the last sub block present in the payload. This field is present only if RI is set to ‘1’. |
| } |  |  |
| FC | 2 | Fragmentation Control |
| do { | - |  |
| End | 1 |  |
| If ( End == 0) { | - |  |
| Length | 11 | SDU or SDU fragment length |
| } | - |  |
| } while (!End) | - |  |
| } | - |  |

Table 15 MEHB Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| M | 1 | Indication of more MEHB 0 = no more MEHB follows the current MEHB 1 = one or more MEHB follows the current MEHB |
| Flow ID | 4 | Flow ID of the SDU/SDUs fragment identified in the MEHB |
| PI | 1 | In case of ARQ connection, the PI bit is set to ‘1’ by the transmitter to request ARQ feedback from the receiver, otherwise it is set to ‘0’. For non ARQ connection PI bit is always set to ‘0’. |
| SN | 10 | Sequence number for the payload identified in the MEHB |
| RI | 1 | In case of ARQ connection, the RI bit is set to ‘1’ if the payload consists of sub blocks of an ARQ block (indicated by the SN field), otherwise it is set to ‘0’. For non ARQ connection the RI bit is always set to ‘0’. |
| SUB-SN | TBD | Sequence number of first Sub Block in the payload. This field is present only if RI is set to ‘1’. |
| LSI | 1 | This bit is set to ‘1’ if the last sub block of the ARQ block represented by SN field is the last sub block present in the payload. This field is present only if RI is set to ‘1’. |
| FC | 2 | Fragmentation Control bits (encoding shown in Table 9) |
| Length | 11 | Length of the correspondent SDU/SDU fragment |
| End | 1 | Indication of more information 0 = Indicating another “Length” and “End” fields are followed 1 = Indicating no more “Length” and “End” fields are followed |

---------------------------------------------------------End of the Text----------------------------------------------------------

# Reference

[1] IEEE 802.16m-09/0010r2, “802.16m Amendment Working Document (AWD)”