| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal of the Broadcast A-MAP IE for Concurrent transmission of the Broadcast management messages (AWD-15.2.16.2.3.1) |  |
| Date Submitted | 2009-07-14 |  |
| Source(s) | Yeongmon Son, Hyunjeong Kang, Jungje Son, Rakesh Taori Samsung Electronics Co., Ltd. Anshuman Nigam Samsung India Software Operation Baowei Ji Samsung Telecommunication America Shantidev Mohanty, Muthaiah Venkatachalam Intel Corporation | ym1004.son@samsung.com +82-31-279-5845 |
| Re: | 802.16m AWD: IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content” |  |
| Abstract | The contribution proposes a scheme to transmit concurrently multiple broadcast management messages in a subframe; especially to process paging message and traffic indication message in the subframe. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal of the Broadcast A-MAP IE for Concurrent transmission of the Broadcast management messages

Yeongmoon Son, Hyunjeong Kang, Jungje Son, Rakesh Taori, Anshuman Nigam, Baowei Ji

Samsung Electronics Co., Ltd.

Shantidev Mohanty, Muthaiah Venkatachalam

Intel Corporation

- Introduction

In the current AWD, There is neither general definition nor description about requirement for transmitting a broadcast management message, except Sleep Mode and Idle Mode.

Moreover, AAI_PAG-ADV and AAI_TRF-IND are fighting for the same location. Both seem to be located after the A-MAP region

- In Sleep Mode, an AAI_TRF-IND_I message is transmitted at a pre-determined location (i.e. in the continuous NTRFIND distributed LRUs right after the A-MAP region).

- In Idle Mode, the size of AAI_PAG-ADV message is also transmitted at a pre-determined location (i.e. the burst with fixed size right after the A-MAP region). The AAI_PAG-ADV message pre-determined location follows the size information at the pre-determined location.

As you can guess from the above bullets, when both broadcast management message is transmitted at the same subframe, the AMSs in Sleep and Idle Mode cannot identify which message (i.e. AAI_TRF-IND_I or the size information of AAI_PAG-ADV) is transmitted first right after A-MAP region.

Now, we propose the unified mechanism (i.e. a broadcast A-MAP IE) with smaller overhead than unicast A-MAP IE.

- Suggested proposal

We propose:

- Assigned broadcast A-MAP group in Non-user specific A-MAP IE

It is determined by ‘Assigned broadcast A-MAP group’ how many broadcast A-MAP IEs will be included in A-MAP region.

- Assigned broadcast A-MAP group = 0 è There is no Broadcast A-MAP IE. Eventually, there is no Broadcast MAC Management Messages right after A-MAP region.

- Assigned broadcast A-MAP group > 0 è There is(are) Broadcast A-MAP IE(s) in A-MAP region. Eventually, Broadcast Management Message(s) is(are) transmitted right after the A-MAP region in the same order as Broadcast A-MAP IE(s) appear(s) in A-MAP region

Table 667—The number of assignment A-MAPs in each assignment A-MAP group

| Index | Assignment Broadcast A-MAP group. | Assignment A-MAP A-MAP group-1. | Assignment A-MAP group-2 | .... |
| --- | --- | --- | --- | --- |
| ... | The number of Broadcast A-MAP IEs | ... | ... | ... |

- Broadcast A-MAP IE to be included in A-MAP region.

Broadcast A-MAP IE is transmitted on the same MCS level (i.e. robust MCS) as Non-User Specific part

Type

Table 1—Broadcast A-MAP IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Broadcast A-MAP IE_format() { | — | — |
| Type | 4 | This indicates the type of broadcast management message 0b0000 : AAI_TRF-IND message 0b0001 : AAI_PAG-ADV message 0b0010 ~ 0b1111 : Reserved |
| Length | 12 | This indicates the length of broadcast management message |
| } | — | — |

- Broadcast management message indicated by Broadcast A-MAP IE in A-MAP region

Broadcast A-MAP IEs are transmitted right after A-MAP region in the same order as Broadcast A-MAP IEs appear in A-MAP region

Broadcast management message are transmitted on the same MCS level(i.e. robust MCS) as Non-User Specific part so that AMS at cell edge can receive them.

The following figure depicts the above mentioned approach.

We can achieve Advantages from the above approach as follows

- Reduced overhead compared with A-MAP IEs

- Selective Decoding based on “Type”

- Generic solution to support all type of broadcast management messages

- Support of concurrent transaction of broadcast management messages

- Text to change in AWD

-------------------------------  Text Start  ---------------------------------------------------

[Replace the 1st paragraph of subclause 15.3.6.5.2.1 at line 60 of page 197 as follows:]

15.3.6.5.2.1 Non-user-specific A-MAP IE

Non-user-specific A-MAP IE consists of information that is not dedicated to a specific user or a specific group of users. It includes information required to decode assignment A-MAP IE. The presence of Broadcast MAC management message is indicated in the Non-user-specific A-MAP IE using one bit flag. The number of assignment A-MAPs in each assignment A-MAP group is indicated in the Non-user-specific A-MAP IE. The detailed information included in non-user specific information is TBD. The non-user specific A-MAP IE is shown in Table 666.

[Replace the Table 667 at line 23 of page 198 in subclause 15.3.6.5.2.1 as follows:]

Table 667 [Detailed entry is TBD] shows the number of the assignment A-MAPs in each assignment A-MAP group.

Table 667—The number of assignment A-MAPs in each assignment A-MAP group

| Index | Broadcast MAC management presence indicator | Assignment A-MAP group 1 | Assignment A-MAP group 2 | … |
| --- | --- | --- | --- | --- |
| … | 1 | … | … | … |

[Add a new section 15.3.6.5.2.12 after subclause 15.3.6.5.2.11 at line 50 of page 229]

15.3.6.5.2.1.1 Transmission of Broadcast MAC Management Messages

When the Broadcast MAC management presence indicator in the Non-user-specific A-MAP IE is zero, it implies that there is no broadcast MAC management corresponding to the A-MAP allocations. When this indicator is set, a fixed-length bitmap shall be presented right after A-MAP region, where each bit corresponds to the existence of a broadcast MAC management message type, e.g., AAI_PAG-ADV, AAI_TRF-IND, AAI_NBR-ADV, etc. Following the bitmap, the corresponding message length is specified in the same order as a bit set to ‘1’ appears in the bitmap. Right after the length fields, the corresponding messages are present as shown in Figure xxx.

Figure xxx: Broadcast MAC management message transmission

[Replace section 15.2.16.2.3.1 at line 12 of page 91 as follows:]

15.2.16.2.3.1 Traffic Indication

Traffic Indication is sent for one or a group of AMS using the AAI_TRF-IND message. AAI_TRF-IND is transmitted at a pre-determined location, i.e. in the continuous NTRFIND distributed LRUs right following the A-MAP region in the 1st subframe of a frame in the listening window.

The AAI_TRF-IND message, when present, shall be transmitted at the first frame of Listening Window of each AMS. The AAI_TRF-IND message shall be transmitted as described in the section 15.3.6.5.2.1.1.

If the traffic indication is enabled for an AMS with SLPID assigned, the AMS shall wait for a traffic indication message. Upon receiving the traffic indication message, the AMS shall check whether there is positive traffic indication (e.g. by the SLPID-Group Indication bit-map and Traffic Indication bit-map or the SLPID assigned to it).

If the AMS receives a negative traffic indication, then it shall end the Listening Window and proceed with Sleep Window operation for the remainder of the Sleep Cycle. If the ABS transmits a negative indication to the AMS, the ABS shall not transmit any DL data traffic to the AMS during the remaining part of the Listening Window, unless there are UL bandwidth requests or UL MAC PDU sent from the AMS which have not been fulfilled.

If the ABS sends a positive indication to a specific AMS, the ABS shall transmit at least one DL MAC PDU to the AMS during the AMS's Listening Window.

If the traffic indication message is lost or otherwise not detected by the AMS, the AMS shall stay awake for the rest of the Listening Window. If the AMS receives any unicast data during the listening window, then it shall assume that the traffic indication was positive. If the AMS receives neither the traffic indication message nor any unicast data in the Listening Window, the AMS shall send a MAC management message (e.g. a signaling header) to ask the ABS what was the traffic indication for the AMS. The ABS shall respond to the AMS by unicasting a MAC management message containing the traffic indication for that AMS.

AAI_TRF-IND is segmented into two parts: AAI_TRF-IND_I and AAI_TRF-IND_II. AAI_TRF-IND_I is transmitted using fixed LRUs. If AAI_TRF-IND_II is transmitted, it follows the AAI_TRF-IND_I and its length will be indicated in AAI_TRF-IND_I.

-------------------------------  Text End  ---------------------------------------------------