| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The format of MAC management messages for Sleep Mode (AWD-15.2.5) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Yeongmoon Son, Jung Je Son, Rakesh Taori Samsung Electronics | E-mail: ym1004.son@samsung.com +82-31-279-5845 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 802.16m AWD: IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content” |  |
| Abstract | This contribution provides the detailed message format of MAC management messages for Sleep Mode |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The format of MAC management messages for Sleep Mode

Yeongmoon Son, Jung Je Son, Rakesh Taori

Samsung Electronics

# Introduction

This contribution provides detailed message format of AAI_SLP-REQ/RSP, AAI_TRF-IND, and AAI_TRF_IND-REQ/RSP which are compliant with AWD .

# Proposed Text #1

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

15.2 Medium access control

15.2.5 AAI MAC Management Messages

[Add the following sections after subclause 15.2.5.7]

15.2.5.X AAI_SLP-REQ

An AMS in Active Mode may use the AAI_SLP-REQ message with Request_Code = 0b01(i.e. Enter Sleep Mode) to request a permission to enter Sleep Mode. The AMS in Sleep Mode can change the sleep mode settings by transmitting AAI_SLP-REQ with Operation = 0b10 (i.e. Change Sleep Mode settings). The AMS in Sleep Mode can exit from Sleep Mode by transmitting AAI_SLP-REQ with Operation = 0b00 (i.e. Exit from Sleep Mode).

Table X—AAI_SLP-REQ message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_SLP-REQ message_format() { | — | — |
| Management Message Type = X | 8 | — |
| Operation | 2 | 0b00 : Exit from Sleep Mode 0b01 : Enter Sleep Mode 0b10 : Change Sleep Mode 0b11 : Switch Sleep Cycle setting |
| if(Operation != 0b00) { | — | — |
| SCID | 3 | Sleep Cycle ID |
| if(Operation != 0b11) { |  |  |
| TIMF | 1 | Traffic Indication Message Flag 0 : AAI_TRF-IND message is not sent for the AMS 1 : AAI_TRF-IND message is sent to the AMS during every Listening Window |
| Start Frame Number | 6 | Least Significant 6 bits of Frame Number |
| Initial Sleep Cycle | 4 | — |
| Final Sleep Cycle | 10 | — |
| Listening Window | 6 | — |
| Padding | variable | Padding bits to ensure byte aligned. |
| } |  |  |
| } | — | — |
| } | — | — |

The following parameters shall be included as follows

Operation

This indicates operation request type of AAI_SLP-REQ message.

0b00 = AAI_SLP-REQ message is transmitted to exit from sleep Mode

0b01 = AAI_SLP-REQ message is transmitted to enter sleep Mode

0b10 = AAI_SLP-REQ message is transmitted to change the sleep Cycle change

0b10 = AAI_SLP-REQ message is transmitted to switch Sleep Cycle setting which has been negotiated since the AMS entered Sleep Mode

SCID

Assigned Sleep Cycle identifier. The ID shall be unique within the AMS. This ID may used in further AAI_SLP-REQ/RSP message for changing/switching the Sleep Cycle setting

TIMF

1 = ABS is requested to transmit an AAI_TRF-IND message during the AMS’s Listening Window. When the ABS has DL pending unicast traffic for the AMS, the ABS shall inform the AMS of positive traffic indication via AAI_TRF-IND message.

0 = Traffic Indication via AAI_TRF-IND is not required

Start_Frame_Number

Start frame number for first Sleep Cycle. This represents the 6 significant bits of frame number in which AMS wants to enter the first sleep state in Sleep Mode.  .

Initial_Sleep_Cycle

This indicates an assigned duration for the Initial Sleep Cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames).

Final Sleep Cycle

This indicates assigned duration for the Final Sleep Cycle (measured in frames).

Listening_Window

Assigned duration of AMS’s default Listening Window (measured in frames). This Listening_Window may be extended as long as there is UL/DL data traffic between AMS and ABS when Listening Window Extension is enabled

The AAI_SLP-REQ shall include the following parameters at end of AAI_SLP-REQ message when Listening Window Extension is enabled:

T_AMS

This timer is required in AMS for Listening Window Extension (see 15.2.16.2.3.2)

15.2.5.(X+1)  AAI_SLP-RSP

The AAI_SLP-RSP message shall be sent from ABS to an AMS in response to an AAI_SLP-REQ message. The ABS may send the AAI_SLP-RSP message in unsolicited manner with Response_Code = 0b00 (i.e. Request by ABS in Unsolicited manner).

If the request sent by an AMS is rejected by an ABS, the AMS shall not retransmit the AAI_SLP-REQ message before the duration, indicated by the REQ_duration in AAI_SLP-RSP with Response_Code = 0b10 (i.e. Rejection of AAI_SLP-REQ), expires.

Table X+1— AAI_SLP-RSP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_SLP-RSP_message_format() { | — | — |
| Management Message Type = X+1 | 8 | — |
| Response_Code | 2 | 0b00 : Request by ABS in Unsolicited manner 0b01 : Approval of AAI_SLP-REQ 0b10 : Rejection of AAI_SLP-REQ 0b11 : Reserved |
| if(Response_Code != 0b10) { |  |  |
| Operation | 2 | 0b00 : Exit Sleep Mode 0b01 : Enter Sleep Mode 0b10 : Change Sleep Mode 0b11 : Switch Sleep Cycle setting |
| if(Operation != 0b00) { | — | — |
| SCID |  | Sleep Cycle ID |
| if(Operation != 0b11) { |  |  |
| TIMF | 1 | 0 : Traffic Indication via AAI_TRF-IND message is disabled 1 : Traffic Indication via AAI_TRF-IND message is enabled |
| Start_Frame_Number | 6 | Least Significant 6 bits of Frame Number |
| Initial Sleep Cycle | 8 | —— |
| Final Sleep Cycle | 10 | — |
| Listening Window | 6 | — |
| if(TIMF == 1) { |  |  |
| SLPID | 10 | —— |
| } | — | — |
| } |  |  |
| } |  |  |
| else { | — |  |
| REQ_duration | 8 | Least Significant 8 bits of Frame Number |
| } | — |  |
| Padding | variable | Padding bits to ensure byte aligned. |
| } | — | — |

The following parameters shall be included as follows

Response_Code

This indicates response type of AAI_SLP-RSP message.

0b00 = AAI_SLP-RSP message is transmitted in unsolicited manner

0b01 = AAI_SLP-RSP message is transmitted to approve the request sent by AMS

0b10 = AAI_SLP-RSP message is transmitted to reject the request sent by AMS

Operation

This indicates operation type of AAI_SLP-RSP message.

0b00 = Approves/Requests the exit from sleep Mode

0b01 = Approves/Requests entrance to sleep Mode

0b10 = Approves/Requests the change of an existing Sleep Cycle Setting

0b10 = Approves/Requests the switch of a Sleep Cycle setting which has been negotiated since the AMS entered Sleep Mode

SCID

Assigned Sleep Cycle Identifier. The ID shall be unique within the AMS. This ID may used in further AAI_SLP-REQ/RSP message for changing/switching the Sleep Cycle setting

TIMF

1 = ABS will transmit an AAI_TRF-IND message during an AMS’s Listening Window. When the ABS has DL pending data traffic for the AMS, the ABS shall inform the AMS of positive traffic indication via AAI_TRF-IND message.

0 = Traffic Indication via AAI_TRF-IND is disabled

Start_Frame_Number

Start frame number for first sleep window. This represents the 6 significant bits of frame number in which AMS enters the first sleep state in Sleep Mode. The frame designated by Start_Frame_Number shall be the first frame of a certain Super Frame.

Initial_Sleep_Cycle

This indicates an assigned duration for the initial sleep cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames).

Final Sleep Cycle

This indicates assigned duration for the final sleep cycle (measured in frames).

Listening_Window

Assigned Duration of AMS’s default Listening Window (measured in frames). Listening_Window may be extended as long as there is UL/DL data traffic between AMS and ABS when Listening Window Extension is enabled.

SLPID

This is an identifier assigned by the ABS when TIMF is set to 1. This ID shall be unique within an ABS. The other AMS shall not be assigned the same ID and SLP Group ID while the AMS is still in sleep mode.

REQ-duration

Waiting value for the AAI_SLP-REQ message retransmission (measured in MAC frames) when AMS’s request is rejected by ABS: the AMS may retransmit the AAI_SLP-REQ message after at least the frame designated by REQ-duration.

The AAI_SLP-RSP shall include the following parameters at end of AAI_SLP-RSP message when Listening Window Extension is enabled:

T_AMS

This timer is required in AMS for Listening Window Extension (see 15.2.16.2.3.2)

15.2.5.(X+2) AAI_TRF-IND

This message, when present, is sent from the ABS to the AMSs. The message is sent in the first frame of AMS’ Listening Window. An AMS, that has not been assigned a SLPID, shall ignore this message. The message indicates whether there is DL traffic for AMSs addressed by the AAI_TRF-IND message.

There are two formats for the AAI_TRF-IND message, indicated by the FMT field. When FMT = 0, and if the AMS does not find its own SLPID-Group Indication bit-map or Traffic Indication bit-map, the AMS shall consider it as a negative indication and may go back to sleep. When FMT = 1, and if the AMS does not find its own SLPID in the AAI_TRF-IND message, the AMS shall consider it as a negative indication and may go back to sleep .

Table X+2— AAI_TRF-IND message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_TRF-IND_message_format() { | — | — |
| Management Message Type = X+2 | 8 | — |
| FMT | 1 |  |
| if(FMT == 0) { | — | — |
| SLPID Group Indication Bitmap | 32 | N-th bit of SLPID-Group Indication Bitmap [MSB corresponds to N = 0] is allocated to SLPID Group that includes AMS with SLPID values from N*32 to N*32+31 0: There is no traffic for any of the 32 AMSs that belong to the SLPID-Group 1: There is traffic for at least one AMS in SLPID-Group. |
| Traffic Indication Bitmap | Variable | The Traffic Indication bitmap comprises multiples of 32-bit long Traffic Indication units. A Traffic Indication unit for 32 SLPIDs is added to AAI_TRF-IND message whenever its SLPID Group is set to 1 32 bits of Traffic Indication Unit (starting from MSB) are allocated to AMS in the ascending order of their SLPID values: 0: Negative indication 1: Positive indication |
| } | — | — |
| else { | — | — |
| Num_of_SLPIDs | 6 |  |
| for (i = 0; i < Num_of_SLPIDs; i++) { |  |  |
| SLPID | 10 |  |
| } |  |  |
| } |  |  |
| Padding | Variable | If needed, for alignment to byte boundary. |
| } | — | — |

The following parameters shall be included as follows

FMT

The FMT field indicates one of the SLPID bit-map based format and the SLPID based format.

SLPID-Group Indication Bitmap

SLPIDs from 0 to 1023 are divided into 32 SLPID-Groups. Therefore, the respective SLPID-Group has the range as follows: SLPID-Group#0 (MSB) corresponds to SLPID = 0…31. SLPID-Group #1 corresponds to SLPID = 32…63. … SLPID-Group#31 corresponds to SLPID = 992…1023. SLPID-Group Indication Bitmap is a 32-bit field where each bit is assigned to the respective SLPID-Group. In other words, the MSB in the field is assigned to SLPID-Group#0, and subsequent bit relates to SLPID-Group #1, etc. The n-th bit (bn), n = 0~31, of SLPID-Group Indication Bitmap shall be interpreted in the following manner: bn = 0 means that there is no traffic for all the 32 AMS belonging to SLPID-Group #n. In this case, the AMS in sleep mode belonging to SLPID-Group #n may return to sleep mode. bn = 1 means that there exists traffic for one or more AMS belonging to SLPID-Group #n. In this case, the AMS in sleep mode belonging to SLPID-Group #n shall read its own Traffic Indication bit-map in AAI_TRF-IND message.

Traffic Indication bit-map

The Traffic Indication bit map comprises the multiples of 32-bit long Traffic Indication Unit for every SLPID-Group with SLPID-Group indication bit = 1. Bits in a 32-bit Traffic Indication unit (starting from MSB) are allocated to AMS to in ascending order of SLPIDs. Each bit signals traffic information for the corresponding AMS as follows:

0: Negative indication

1: Positive indication

Num_of_SLPID

The number of SLPID with positive indication.

SLPID

The SLPID for AMS which has DL pending traffic.

The AAI_TRF-IND may include the following parameters at end of AAI_SLP-REQ message

SLPID_Update

The SLPID_Update provides a shorthand method for changing the SLPID used by the AMS in sleep mode operation. The SLPID_Update specifies a new SLPID that replaces an old SLPID. The SLPID_Update may contain multiple pairs of Old and New SLPID values for the AMSs. Those SLPID update will be applied from next Listening Window

15.2.5.(X+3) AAI_TRF_IND-REQ

If the TIMF is set to 1, the AMS shall receive AAI_TRF-IND message in the first frame during its own Listening Window. However, if the traffic indication message is lost or otherwise not detected by the AMS, the AMS shall stay awake for the rest of the Listening Window. If the AMS receives any unicast data during the listening window, then it shall assume that the traffic indication was positive. If the AMS receives neither the traffic indication message nor any unicast data in the Listening Window, the AMS shall send an AAI_TRF_IND-REQ message to the ABS in order to ask which kind of traffic indication (i.e. positive or negative traffic indication) the ABS sent to it. The ABS shall respond to the AMS by sending an AAI_TRF_IND-RSP with traffic indication for the AMS.

Table X+3—AAI_TRF_IND-REQ message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_TRF_IND-REQ_Message_format() { | — | — |
| Management Message Type = X+3 | 8 | — |
| Frame_Number | 4 | Least Significant 4 bits of Frame number |
| Padding | 4 | If needed, for alignment to byte boundary. |
| } | — | — |

The following parameters shall be included as follows

Frame_Number

This indicates the least significant 4 bits of frame number in which the AMS expected to receive the AAI_TRF-IND.

15.2.5.(X+4) AAI_TRF_IND-RSP

When the ABS receives AAI_TRF_IND-REQ message from an AMS, the ABS shall respond to the AMS by sending AAI_TRF_IND-RSP message with the traffic indication for the AMS. When the AMS receives AAI_TRF_IND-RSP from the ABS, the AMS shall update the current Sleep Cycle based on the Traffic_Indication in the AAI_TRF_IND-RSP.

Table X+4—AAI_TRF_IND-RSP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_TRF-RSP_Message_format() { | — | — |
| Management Message Type = X+4 | 8 | — |
| Frame_Number | 4 | Least Significant 4 bits of Frame number |
| Traffic_Indication | 1 | 0: Negative Traffic Indication 1: Positive Traffic Indication |
| Padding | 3 | If needed, for alignment to byte boundary. |
| } | — | — |

The following parameters shall be included as follows

Frame_Number

This indicates the least significant 4 bits of frame number in which the ABS sent the AAI_TRF-IND message to the AMS.

Traffic_Indication

This indicates the traffic indication of which the ABS informed the AMS in the specified frame number.

0 = the ABS sent the negative traffic indication to the AMS in the frame

1 = the ABS sent the positive traffic indication to the AMS in the frame

------------------------------------------------------ End of Proposed Text ------------------------------------------------------