| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for CQI definition in IEEE802.16m Amendment |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Takaaki KISHIGAMI, Isamu YOSHII Panasonic Corporation | Voice: + 81-46-840-5123 E-mail: kishigami.takaaki@jp.panasonic.com yoshii.isamu@jp.panasonic.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.9.3 (UL control information content) “Comments on AWD 15.3.9.3.1.3 Channel quality indicator(CQI) definition ” |  |
| Abstract | We propose the text for the CQI definition. |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for CQI definition in IEEE802.16m Amendment

Takaaki KISHIGAMI, Isamu YOSHII

Panasonic Corporation

# Introduction

In current AWD text[1], CQI definition is not clearly defined.

In this contribution, we propose to add the text to include the detail definitions on wideband CQI in case of FFR and differential subband CQI by TDM[2] .

# References

[1]IEEE 802.16m-09/0010r2,“802.16m Amendment Working Document (AWD)”

[2]IEEE 802.16m-09/0854,“Proposed Text for the transmission format of FBCH in IEEE802.16m Amendment”

# Proposed Text

We propose the following text for the IEEE802.16m Amendment Working Document in Section 15.3.9.3.1.3

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and  existing AWD text is shown in black.

-------------------------------  Start text proposal  ---------------------------------------------------

15.3.9.3.1.3 Channel quality indicator(CQI) definition

Jointly encoding of CQI and STC rate is TBD, which specifies the spectrum efficiency.

Wideband CQI

Wideband CQI is one average CQI over whole band corresponding to the MIMO mode.

In case of FFR , the wideband CQI TBD.is one average CQI over the band indicated by FPS.

Subband CQI

Subband CQI is represented by one base CQI (ave_CQI) over the selected M subbands (or whole bandwidth TBD) plus a differential CQI, which is used for best-M feedback.

For best-M feedback, the subband selection and Ave_CQI and Diff_CQI may be are reported at different intervals, and share the same FBCH with TDM manner. Subband selection and Ave_CQI will puncture the Diff_CQI at every [4th TBD] report if TDM is enabled.

The differential CQI is composed of the previous CQI (Pre_CQI) of same subband and the differential CQI(Diff_CQI) over the Pre_CQI. And [2 (TBD)] bits are used to indicate the differential levels between the Pre_CQI and a current CQI.

The Ave_CQI uses [4 (TBD) ] bits to indicate the average subband CQI.

MCS is set based on the Ave_CQI, or both of the Ave_CQI and the Diff_CQI.

[All subband CQIs across the whole bandwidth may be organized in hierarchical tree (TBD)]

[Reporting of transformed coefficients is TBD, where MS linearly transforms all the subband CQIs (whole band) into a series of progressively insignificant coefficients, and quantize the coefficients to few bits for feedback. The transformed values are sent over one or several reports]

Specified CQI (TBD)

Specified CQI is the CQI specified for a certain RU indicated by DLRU parameters in feedback allocation A-MAP IE.

-------------------------------  End text proposal  ---------------------------------------------------