| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for FBCH feedback content in IEEE802.16m Amendment |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Takaaki KISHIGAMI, Isamu YOSHII Panasonic Corporation | Voice: + 81-46-840-5123 E-mail: kishigami.takaaki@jp.panasonic.com yoshii.isamu@jp.panasonic.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.9.3 (UL control information content) “Comments on AWD 15.3.9.3.1 Fast feedback control channel ” |  |
| Abstract | We propose the text for the FBCH Feedback Content. |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for FBCH feedback content in IEEE802.16m Amendment

Takaaki KISHIGAMI, Isamu YOSHII

Panasonic Corporation

# Introduction

In current AWD text[1], the Primary FBCH feedback content is not clearly defined. In this contribution, we propose to add the text to support subband CQI, Best-1 subband selection, subband PMI (Mt=1).

# References

[1]IEEE 802.16m-09/0010r2,“802.16m Amendment Working Document (AWD)”

# Proposed Text

We propose the following text for the IEEE802.16m Amendment Working Document in Section 15.3.9.3.1.1.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and  existing AWD text is shown in black.

-------------------------------  Start text proposal  ---------------------------------------------------

15.3.9.3.1.1 Primary fast feedback control channel

The UL PFBCH carries 4 to 6 bits of information, providing wideband channel quality feedback and MIMO feedback. and other necessary CQI-related information. Information contents to be carried over PFBCH is described in Table 725. The PFBCH also provides for reporting subband CQI, Best-1 subband selection, PMI in PFBCH for MFM 2,3,5,6 (Mt=1) .

Different contents are distinguished by sequence in PFBCH. The mapping of 6 bits payload to these contents is described in Table xxx[TBD].

PFBCH is semi statically allocated to AMS by feedback allocation A-MAP IE to send periodic CQI and RI feedback with pre-configured reporting frequency. And only one PFBCH will be allocated for each active AMS.

Non-regular information such as EDI (event-driven indicator), BW requests indicator, FPS (Frequency partition selection) and buffer management will be sent on-demand in same PFBCH and supersedes CQI and RI reporting.

Table 725- PFBCH Feedback Content

| PFBCH Feedback Content | Related MIMO feedback mode | Description/Notes |
| --- | --- | --- |
| CQI | MFM 0, 1, 4, 7 | 1) Wideband CQI |
|  | MFM 2, 3, 5,6 | 2) Subband CQI for Best -1 subband (TBD) 3) Average CQI over resource units indicated by BS (TBD) Resource units are indicated by LRU parameter (defined in feedback allocation A-MAP IE in 15.3.6.3.2.2) |
| STC Rate Indicator | MFM 1, 2, 3 | Number of streams |
| BSI (Band Selection Info for best-1 subband) (TBD) | MFM 2, 3, 5,6 | Selected band information for best subband is specified by LRU parameter which defined in feedback allocation A-MAP IE in 15.3.6.3.2.2 |
| PMI (TBD) | MFM 0, 1, 4, 7 | 1) wideband PMI |
|  | MFM 2, 3, 5,6 | 2) subband PMI for best-1 |
| Event-driven Indicator (EDI) for CQICH de-allocation (TBD) | N/A | MS to indicate channel variation event |
| Event-driven confirmation for CQICH de-allocation (TBD) | N/A | MS to indicate channel variation event |
| Event-driven Indicator (EDI) for Preferred MIMO operation (TBD) |  | Sequences: Option 1: 1) OL distributed 2) OL localized 3) CL distributed 4) CL localized Option 2: 1) OL 2) CL 3) SU 4) MU MaxMt =2 5) MU MaxMt = 3 6) MU MaxMt = 4 Option 3: 1) two steps method |
| Event-driven Indicator (EDI) for Multi-cell MIMO Initiation (TBD) |  | Sequence based |
| Event-driven Indicator (EDI) for Bandwidth Request Indicator (TBD) | N/A | This is used to request UL bandwidth. 2 sequences (two services) |
| Event-driven Indicator (EDI) for Frequency partition selection (FPS) | N/A | Inform BS the frequency partition of the reported CQI (DL diversity mode only) |
| Event-driven Indicator (EDI) for Buffer management (TBD) | N/A | Indicates occupancy status of HARQ soft buffer |

-------------------------------  End text proposal  ---------------------------------------------------