| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Type-4 Subframe Pilot Patterns |  |
| Date Submitted | 2009-07-14 |  |
| Source(s) | Jia-Hao Wu, Yu-Tao Hsieh, Pang-An Ting ITRI | Voice : 886-3-5914854 E-mail: yutaohsieh@gmail.com E-mail: jefferywu@itri.org.tw |
| Re: | Re: IEEE 802.16m-09/0028 AWD Call for Contributions after Session #61 IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document.” |  |
| Abstract | The contribution proposes the text of frame structure section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Type-4 subframe Pilot Patterns

Jia-Hao Wu, Yu-Tao Hsieh, Pang-An Ting

ITRI

# Introduction

In the current AWD[1], type-4 subframe is used for 8.75 MHz with guard time ratio G=1/4 in uplink (UL) transmission. However, the UL pilot structure of type-4 subframe is still absent. In this contribution, we suggest pilot structures of type-4 subframe which are derived from current 6-symbol UL pilot structures.

# References

[1] IEEE 802.16m-08/0010r2, “IEEE 802.16m Amendment Working Document.”

[2] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment.”

# Text proposal for the 802.16m amendment

Insert the following text and illustrative figures in subclause 15.3.8.4 at the end of page 279.

=============================== Proposed text start ================================

The pilot patterns of type-4 subframe are derived from the type-2 subframe patterns. The first seven symbols of type-4 subframe pilot patterns are identical to the type-2 subframe patterns. The last two symbols of type-4 subframe pilot patterns are generated by appending the first two symbols of type-2 subframe pilot patterns.

Figure 491a Pilot patterns of 1-Tx stream for type-4 subframe distributed LRUs

Figure 492a Pilot patterns of 2-Tx stream for type-4 subframe distributed LRUs

Figure 493a Pilot patterns of 1-Tx stream for type-4 subframe distributed PUSC LRUs

Figure 494a Pilot patterns of 2-Tx stream for type-4 subframe distributed PUSC LRUs

Figure 495a Pilot patterns of 1-Tx stream for type-4 subframe contiguous LRUs

Figure 496a Pilot patterns of 2-Tx stream for type-4 subframe contiguous LRUs

Figure 497a Pilot patterns of 3-Tx stream for type-4 subframe contiguous LRUs

Figure 498a Pilot patterns of 4-Tx stream for type-4 subframe contiguous LRUs

============================= Proposed text end ===============================