| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text modification for Uplink MIMO transmission modes |  |
| Date Submitted | 2009-07-07 |  |
| Source(s) | Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting ITRI Hsi-Pin Ma, Yeong-Luh Ueng, Jen-Ming Wu, Yuan-Hao Huang NTHU | Email: pinghengkuo@itri.org.tw |
| Re: | AWD comments |  |
| Abstract | The contribution proposes text for the MIMO section of the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text Modification for Uplink MIMO Transmission Modes

Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting

ITRI

Hsi-Pin Ma, Yeong-Luh Ueng, Jen-Ming Wu, Yuan-Hao Huang

NTHU

1. Introduction

Uplink MIMO transmission mode 3 and mode 4 are used to support MU-MIMO, as known as collaborative spatial multiplexing (CSM). In the current amendment working document (AWD) [1], uplink MIMO parameters for all modes have been listed in Table 735 of [1]. However, the definition of Nt for CSM (mode 3 and mode 4) is vague as the transmit side of these modes involve multiple AMS. In particular, when multiple AMSs involved in CSM are equipped with different number of antennas, the definition of Nt results in ambiguity.

In fact, presuming one stream per AMS in uplink MU-MIMO, the total number of stream (Mt) depends on how many AMSs are involved in CSM. Therefore, we suggest to modify the table as shown in the proposed text to clarify this issue.

2. References

[1] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-06-01.

3. Proposed Text

------------------------------------------- Text Start -------------------------------------------

15.3.10.1.3 Uplink MIMO transmission modes

There are five MIMO transmission modes for UL MIMO transmission as listed in Table xx0.

Table xx0 – Uplink MIMO modes

| Mode Index | Description | MIMO encoding format (MEF) | MIMO Precoding |
| --- | --- | --- | --- |
| Mode 0 | OL SU-MIMO | SFBC | Non-adaptive |
| Mode 1 | OL SU-MIMO (SM) | VE | Non-adaptive |
| Mode 2 | CL SU-MIMO (SM) | VE | adaptive |
| Mode 3 | OL Collaborative spatial multiplexing (MU-MIMO) | VE | Non-adaptive |
| Mode 4 | CL Collaborative spatial multiplexing (MU-MIMO) | VE | adaptive |

The allowed values for UL MIMO modes are shown in Table xx1 and xx2.

Table xx1– DL UL MIMO parameters for Mode 0,1 and 2

|  | Number of transmit antennas | STC rate per layer | Number of streams | Number of subcarriers | Number of layers |
| --- | --- | --- | --- | --- | --- |
|  | Nt | R | Mt | NF | L |
| MIMO mode 0 | 2 | 1 | 2 | 2 | 1 |
|  | 4 | 1 | 2 | 2 | 1 |
|  | 8 | 1 | 2 | 2 | 1 |
| MIMO mode 1 and mode 2 | 2 | 1 | 1 | 1 | 1 |
|  | 2 | 2 | 2 | 1 | 1 |
|  | 4 | 1 | 1 | 1 | 1 |
|  | 4 | 2 | 2 | 1 | 1 |
|  | 4 | 3 | 3 | 1 | 1 |
|  | 4 | 4 | 4 | 1 | 1 |
|  | 8 | 1 | 1 | 1 | 1 |
|  | 8 | 2 | 2 | 1 | 1 |
|  | 8 | 3 | 3 | 1 | 1 |
|  | 8 | 4 | 4 | 1 | 1 |
|  | 8 | 5 | 5 | 1 | 1 |
|  | 8 | 6 | 6 | 1 | 1 |
|  | 8 | 7 | 7 | 1 | 1 |
|  | 8 | 8 | 8 | 1 | 1 |
| MIMO mode 3 and MIMO mode 4 | 2 | 1 | 2 | 1 | 2 |
|  | 4 | 1 | 2 | 1 | 2 |
|  | 4 | 1 | 3 | 1 | 3 |
|  | 4 | 1 | 4 | 1 | 4 |
|  | 8 | 1 | 2 | 1 | 2 |
|  | 8 | 1 | 3 | 1 | 3 |
|  | 8 | 1 | 4 | 1 | 4 |

Table xx2– UL MIMO parameters for Mode 3 and 4

|  | Number of transmit antennas | Number of AMS involved in CSM | STC rate per layer | Number of streams | Number of subcarriers | Number of layers |
| --- | --- | --- | --- | --- | --- | --- |
|  | Nt | M | R | Mt | NF | L |
| MIMO mode 3 and mode 4 (CSM) | 1, 2, 4, 8 | 2 | 1 | 2 | 1 | 2 |
|  |  | 3 | 1 | 3 | 1 | 3 |
|  |  | 4 | 1 | 4 | 1 | 4 |

Mt refers to the number of streams transmitted to one MS BS with MIMO modes 0, 1, and 2.

Mt refers to the total number of streams transmitted to from multiple MS on the same RU with MIMO modes 3 and 4.

------------------------------------------- Text End -------------------------------------------