| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Bit level Scrambler After FEC Encoding and Bit Repetition (AWD:15.3.12.1.5.1 9) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Kiran Kuchi, J Klutto Milleth, Vinod R, Divagar , Padmanabhan M S, Bhaskar R CEWiT, India | E-mail: kkuchi@cewit.org.in (+91 9962045186) klutto@cewit.org.in (+91 9444107970) |
| Re: | Call for Comments on IEEE 802.16m-09/0010r2 IEEE 802.16m Amendment text/Area: Chapter 15.3.12.1.5.1 9 (Channel Coding) |  |
| Abstract | A bit level scrambler is introduced after FEC doing and bit repetition to maximize the repetition coding gain in the presence of interference |  |
| Purpose | For inclusion in AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

###### Bit level Scrambler After FEC encoding and Bit Repetition (AWD:15.3.12.1.5.1 9)

Kiran Kuchi, J Klutto Milleth, Vinod R, Divagar , Padmanabhan M, Bhaskar R

# CEWiT, India

# Introduction:

# A bit level scrambler is introduced after FEC doing and bit repetition to maximize the repetition coding gain in the presence of interference.

The scrambler/Randomizer

The modulation symbols from each FEC block are multiplexed using a 	parallel to 	serial converter to obtain a stream of modulation data which will be 	mapped to the resources. The output bits from each FEC block are passed 	through a 	scrambler. 	The scrambler uses a cell specific scrambling sequence. 	The scrambled bits are mapped to modulation symbols. The outputs of the mod	ulators are multiplexed into a single stream of modulation symbols and are then 	mapped to the resource elements.

Randomizing the encoded bits by a PRBS gives a protection against the 	lack of interference averaging, in situations where the desired and interfering 
	symbols follow the same pattern. This bit-level scrambler works as follows:

1.The FEC encoded bits are 'XOR' ed by a PRBS at the transmitter and modulated.

2. At the receiver, the LLR values going in to the Turbo encoder are inverted, iff the 	    PRBS value at that bit-location is 1. This is the equivalent reverse operation of 	    scrambling at the transmitter.

The performance gain of a randomizer, which uses a PRBS generated using the 	primitive polynomial X3 + X + 1 (This sequence 	has a periodicity of 7) is de	picted in figure 2.

:

:

FEC block 1

FEC block 2

FEC block KFB

Parallel to Serial Converter

Scrambler

Scrambler

Scrambler

MIMO encoder

Modulation

Modulation

Modulation

Ssubcarrier mapping

Figure 1: The transsmission chain

SNR dB

Figure 2:  The performance gain by using a randomizer, in the case of  rate 48/256 CTC, MMSE Rx.

The merit of using a randomizer is evident when the desired signal and the 	interference follow exactly the same pattern. Figure 3 shows that, the randomizer 	effectively destroys the  underlying pattern and enables the interference averaging 	gain.

Figure 3: Rate 48/256 CTC, MRC Rx. PRBS is generated by X11 +  X9 + 1.