| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Facilitating Interference Avoidance and Interference Mitigation of Femtocell BSs (AWD-Femto) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Hsin-Piao Lin and Kun-Yi Lin NTUT Rong-Terng Juang ITRI | Voice: + 886 2 27712171 E-mail: t6419016@ntut.edu.tw |
| Re: | Call for Contribution on 80216m-09_0028r1 |  |
| Abstract | The contribution proposes text for the Interference Avoidance and Interference Mitigation of Femtocell BSs to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Facilitating Interference Avoidance and Interference Mitigation of Femtocell BSs (AWD-Femto)

Hsin-Piao Lin and Kun-Yi Lin

NTUT

Rong-Terng Juang

ITRI

# Abstract

This contribution provides amendment text for the Support for Femtocell BS in the Advanced Air Interface to be included in Section 15 of the IEEE 802.16m Amendment Working Document [1]. The amendment text was developed using baseline text from the Draft IEEE 802.16m System Description Document (SDD) [2], and follow the amendment text style guide [3]. Further details of the signaling procedure between Femtocell BS and AMS to support Interference Avoidance and Interference Mitigation are provided.

# Introduction

Excluding the reuse-1 frequency partition, FP0, Fig. 1 shows an example of macrocell ABS FFR configuration. To investigate the impact of co-channel femtocells on macrocells, this contribution assumes each macrocell overlays on top of N femtocells. The transmit power of each co-channel femtocell BS is set to a value that on average equals the power received from the overlaid macrocell at a target cell radius of r=10 meters. The femtocell transmit power, pf, here is calculated as

pf=1010log10pm+G(θ)−Lm(dm)+Lf(df)/10                                   (1)

where pm is the macrocell transmit power, G(θ) is the antenna gain of the macrocell at user direction θ, Lm(dm)=35.2+35log10dm+26log10(fc/2) with fc being the carrier frequency in GHz is the urban macrocell path-loss at distance dm in meters, and Lf(df)=46.4+20log10df+20log10(fc/5)+Lwallswith Lwalls being the loss due to external/internal walls denotes the femtocell path-loss at distance df in meters [17]. The antenna gain for macrocell BS is calculated as G(θ)=Gmax−min[12(θ/θ3dB)2,Gm], where −180≤θ≤180, Gmax=17dBi, θ3dB=70degrees, Gm=20dB [17].

Fig 1. Conventional configuration of fractional frequency reuse.

Femtocells in the inner region of macrocells are assigned frequency partitions PF1 , PF 2 and PF 3, while those in the outer region are assigned the identical sub-channel with their overlaid macrocell. Given an AMS connected to macrocell i, the received downlink SINR (signal-to-interference-plus-noise ratio) can be expressed as

SINR=pm,igiqm,ik=0,k≠iMpm,kgm,iqm,k+k=0Npf,kqf,k+σ2                             (2)

where M is the number of macrocells, pm,i and pf,i are respectively the transmit powers of ith macrocell and femtocell, gi is the antenna gain of ith macrocell at the AMS, qm,i  and qf,i are respectively the path gains between the AMS and ith macrocell and femtocell, and σ2 represents the variance of additive white Gaussian noise.

In the simulations, 19 macro-BS sites with a transmit power of 46 dBm and a site-to-site distance of 2R=1.5 km were deployed. Femtocells were randomly located in two different zones. One is inner zone, which defines a circle with radius R'<R/3, and the other is outer zone defining a circular ring with radius 2R/3≤R'<R. Figure 2 show the SINR statistics of random-located macrocell AMSs. Femtocells located at outer zone do not rise a significant impact when the density is increased from 50 to 200 femtocells per macro-sector. This means inter-macrocell interference dominates the performance of cell-edge users because of the lower requirement of transmit power for femtocells at edges of macrocells. With a density of 50 femtocells at inner zone per macro-sector, however, femtocells have a great impact on macrocell users. To facilitate interference mitigation, therefore, some geographic information could be provided to femtocells.

Figure 2. SINR statistics of macrocells.

- References

[1]	P802.16m Amendment Working document, 802.16m-09/0010r2, June 2009.

[2]	IEEE 802.16m System Description Document (SDD) [Draft], IEEE 802.16m-08/003r9a.

[3]	Style guide for writing the IEEE 802.16m amendment, IEEE 802.16m-08/043

# Proposed Text

------------------------------------------------------ Start of the proposed text ------------------------------------------------

15.x.x Interference Avoidance and Interference Mitigation

Femtocells in co-channel operation with the overlaid macrocells reduce the macrocell performance. Especially for femtocells in the vicinities of macrocell BSs. To facilitate interference mitigation, therefore, some geographic information can be provided to femtocells. If femtocell BSs are close to macrocell BSs and the density of femtocell BSs is high, femtocell BSs should use radio resource that is orthogonal to their overlaid macrocells. If the density of femtocell BSs is low, femtocell BSs can hop between the radio resource to mitigate the interference.

------------------------------------------------------- End of the proposed text ------------------------------------------------